| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0118 WG11 Number: m30859 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0063 |
| --- | --- |

| Title: | On Video Signal Information in VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes syntax and semantics for signalling video signal information including video_format, video_full_range_flag, colour_primaries, transfer_characteristics, matrix_coeffs regarding layers in VPS extension.  It is asserted that this is useful for session negotiation. Additionally a mechanism is proposed to share this information with SPS and to also enable update of the video signal information in the SPS.

# Introduction

In Vienna meeting video representation format signalling in VPS was adopted from JCTVC-N0092 [1]. This included signalling of spatial resolution, bit depth and colour format in VPS for session negotiation. In addition to the proposed signalling in JCTVC-N0092 in JCTVC-N0238 [2] it was proposed to signal color chromaticity information in VPS for session negotiation.

Meeting report from Vienna meeting [3] concludes that:

“Basically the remaining question is whether to add "video signal type" information at the VPS level (in VUI-like section or elsewhere).

Something like this may be desirable, but requires further study.”

This document proposes syntax and semantics for signalling video signal information including video_format, video_full_range_flag, colour_primaries, transfer_characteristics, matrix_coeffs regarding layers in VPS extension.  Currently this video signal information is signalled in SPS VUI in SHVC [4] and MV-HEVC [5] drafts.

It is asserted that it is useful for session negotiation that a client can select layers to decode based on the knowledge about video signal information including color chromaticity information. The upcoming deployment of UHDTV devices and content will use a different color gamut than legacy devices.  Specifically, HD uses the ITU-R BT.709 recommendation [6], while UHDTV will use the ITU-R BT.2020 recommendation [7].  A key difference between these systems is that the color gamut of UHDTV is significantly larger than HD. Also common consumer applications using BT.709 have an 8-bit sample depth while BT.2020 only defines 10-bit and 12-bit sample depths. Thus it is asserted that signalling information about bit-depth and colour chromaticity in VPS is important in enabling determination of UHDTV layers which use ITU-R BT.2020 during session negotiation.  Thus this document proposes to signal video signal information including color chromaticity information in VPS. Additionally a mechanism is proposed to share this information with SPS and to also enable update of the video signal information in the SPS.

# Proposal

We propose syntax and semantics for signalling video source information about layers in VPS.

Proposed changes are highlighted below compared to JCTVC-N1008 [4].

## VPS syntax & Semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| video_signal_info_idx_present_flag | u(1) |
| if( video_signal_info_idx_present_flag ) |  |
| vps_num_video_signal_info_minus1 | u(4) |
| for( i = 0; i <= vps_num_video_signal_info_minus1; i++ ) |  |
| video_signal_info( ) |  |
| if( video_signal_info_idx_present_flag ) { |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| if( vps_num_video_signal_info_minus1 > 0) |  |
| vps_video_signal_info_idx[ i ] | u(4) |
| } |  |
| … |  |
| } |  |

video_signal_info_idx_present_flag equal to 1 indicates that the syntax elements vps_num_video_signal_info_minus1, and vps_video_signal_info_idx[ i ] are present. video_signal_info_idx_present_flag equal to 0 indicates that the syntax elements vps_num_video_signal_info_minus1, and vps_video_signal_info_idx[ i ] are not present. When video_signal_info_idx_present_flag is equal to 0 vps_num_video_signal_info_minus1 is inferred to be equal to vps_max_layers_minus1 and vps_video_signal_info_idx[ i ] is inferred to be equal to i for each of i=0,…,vps_max_layers_minus1.

vps_num_video_signal_info_minus1 plus 1 specifies the number of the following video_signal_info( ) syntax structures in the VPS. When not present, the value of vps_num_video_signal_info_minus1 is inferred to be equal to vps_max_layers_minus1.

vps_video_signal_info_idx[ i ] specifies the index, into the list of video_signal_info( ) syntax structures in the VPS, of the video_signal_info( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. When i is equal to 0 or vps_num_video_signal_info_minus1 is equal to 0, the value of vps_video_signal_info_idx[ i ] is inferred to be equal to 0. The value of vps_video_signal_info_idx[ i ] shall be in the range of 0 to vps_num_video_signal_info_minus1, inclusive.

## Video Signal Information syntax & Semantics

| video_signal_info( ) { | Descriptor |
| --- | --- |
| video_vps_format | u(3) |
| video_full_range_vps_flag | u(1) |
| colour_primaries_vps | u(8) |
| transfer_characteristics_vps | u(8) |
| matrix_coeffs_vps | u(8) |
| } |  |

video_vps_format, video_full_range_vps_flag, colour_primaries_vps, transfer_characteristics_vps, matrix_coeffs_vps  are used for inference of the values of the SPS VUI syntax elements video_format, video_full_range_flag, colour_primaries, transfer_characteristics, matrix_coeffs respectively, for each SPS that refers to the VPS.

For each of these syntax elements, all constraints, if any, that apply to the value of the corresponding SPS VUI syntax element also apply.

## SPS syntax & Semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| … |  |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = EXTENDED_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| if( nuh_layer_id > 0 ) |  |
| update_video_signal_info_flag | u(1) |
| if( update_video_signal_info_flag ) { |  |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coeffs | u(8) |
| } |  |
| } |  |
| } |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| … | u(1) |
| } |  |

update_video_signal_info_flag equal to 1 specifies that the syntax elements video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs are explicitly signalled in the SPS and all the layers with nuh_layer_id greater than zero that refer to this SPS use these values instead of those corresponding values signalled in the VPS when the nuh_layer_id of the SPS is greater than 0. update_video_signal_info_flag equal to 0 specifies that the syntax elements video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs respectively, are not signalled in the SPS and all the layers that refer to this SPS use the values signalled in the VPS. When not present, the value of update_video_signal_info_flag is inferred to be equal to 1.

When a current picture with nuh_layer_id layerIdCurr greater than 0 refers to an SPS, the values of video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs are inferred or constrained as follows:

- If the nuh_layer_id of the active layer SPS is equal to 0, the values of video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs are inferred to be equal to video_vps_format, video_full_range_vps_flag, colour_primaries_vps, transfer_characteristics_vps, and matrix_coeffs_vps, respectively, of the vps_video_signal_info_idx[ j ]-th video_signal_info( ) syntax structure in the active VPS where j is equal to LayerIdxInVps[ layerIdCurr ] and the values of video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs of the active layer SPS are ignored.

NOTE X –  The values are inferred from the VPS when an active non-base layer references an SPS which is also used by the base layer, in which case the SPS has nuh_layer_id equal to 0. For an active base layer, the values in the active SPS apply.

- Otherwise (the nuh_layer_id of the active layer SPS is greater than zero), the following applies:

  - If update_video_signal_info_flag is equal to 0, the values of video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs are inferred to be equal to video_vps_format, video_full_range_vps_flag, colour_primaries_vps, transfer_characteristics_vps, and matrix_coeffs_vps, respectively, of the vps_video_signal_info_idx[ j ]-th video_signal_info( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

  - Otherwise (update_video_signal_info_flag is equal to 1), it is a requirement of bitstream conformance that the value of at least one of video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs shall be different than values of video_vps_format, video_full_range_vps_flag, colour_primaries_vps, transfer_characteristics_vps, and matrix_coeffs_vps, respectively, of the vps_video_signal_info_idx[ j ]-th video_signal_info( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

# Conclusion

This document proposes syntax and semantics for signalling video signal information including video_format, video_full_range_flag, colour_primaries, transfer_characteristics, matrix_coeffs regarding layers in VPS extension.  It is asserted that this is useful for session negotiation. Additionally a mechanism is proposed to share this information with SPS and to also enable update of the video signal information in the SPS.

We propose to adopt the proposal in the draft of SHVC and MV-HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] A. K. Ramasubramonian, Y.-K. Wang, Y. Chen, J. Boyce, S. Deshpande, “MV-HEVC/SHVC HLS: Representation format information in VPS,” JCTVC-N0092, Vienna, August 2013

[2] S. Deshpande, “On Source Representation Information Signaling in VPS,” JCTVC-N0238, Vienna, August 2013

[3] G. Sullivan, J.-R. Ohm, “Meeting report of the 14th meeting of the Joint Collaborative Team on Video Coding (JCT-VC),” JCTVC-N_Notes_dA, Vienna, AT, 25 July – 2 Aug. 2013

[4] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Draft Text 3,” JCTVC-N1008, Vienna, August 2013

[5] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 5 (ISO/IEC 23008-2:201x/PDAM2),” JCT3V-E1004, Vienna, August 2013.

[6] ITU-R Recommendation BT.709 “Parameter values for the HDTV standards for production and international programme exchange” Dec. 2010

[7] ITU-R Recommendation BT.2020 “Parameter values for UHDTV systems for production and international programme exchange” April 2012.