| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0119 WG11 Number: m30860 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0064 |
| --- | --- |

| Title: | On Highest Temporal Sub-layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

In this document :

- It is proposed to signal the information regarding maximum number of temporal sub-layers that may be present in each layer in VPS.

- It is proposed to derive HighestTemporalIdList - a list of values of highest TemporalId for each layer in the TargetDecLayerIdList.

It is proposed to invoke the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in sub-clause Error: Reference source not found when TemporalId is equal to HighestTemporalIdList[nuh_layer_id] during the  decoding process for ending the decoding of a coded picture with nuh_layer_id greater than zero as specified in F.8.1.2.

# Introduction

In JCTVC-N1008 [1] and JCT3V-E1004 [2] decoding of HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified in 8.1 General Decoding Process as follows:

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in subclause C.1, HighestTid is set as specified in subclause C.1.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

In Annex C, sub-clause C.1 HighestTid is set as follows:

- An operation point under test, denoted as TargetOp, is selected. The layer identifier list OpLayerIdList of TargetOp consists of the list of nuh_layer_id values, in increasing order of nuh_layer_id values, present in the bitstream subset associated with TargetOp, which is a subset of the nuh_layer_id values present in the bitstream under test. The OpTid of TargetOp is equal to the highest TemporalId present in the bitstream subset associated with TargetOp.

- TargetDecLayerIdList is set equal to OpLayerIdList of TargetOp, HighestTid is set equal to OpTid of TargetOp, and the sub-bitstream extraction process as specified in clause Error: Reference source not found is invoked with the bitstream under test, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to BitstreamToDecode.

In N1008 and E1004 the HighestTid is used for Marking process for sub-layer non-reference pictures not needed for inter-layer prediction.

In particular during the decoding process for ending the decoding of a coded picture with nuh_layer_id > 0 as specified in F.8.1.2 of N1008, when TemporalId is equal to HighestTid, the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in sub-clause Error: Reference source not found is invoked with latestDecLayerId equal to nuh_layer_id as input.

It is asserted that in SHVC different layers may have different frame rates. As a result a layer with a higher frame rate may have a higher value of highest temporal sub-layer compared to a layer with a lower frame-rate. In this case using current decoding process the HighestTid value will be set to be equal to the highest TemporalId in the bitstream when using subclause C.1 of Annex C to set HighestTid value. In this case currently the marking process for sub-layer non-reference pictures not needed for inter-layer prediction in F.8.1.2.1 will not get invoked for layers which have lower frame rate as the highest TemporalId value in those layers is less than HighestTid value for the bitstream. In this case sub-layer non-reference pictures in the highest temporal sub-layer of such layers could not be removed earlier. We propose changes to the decoding process which can help remove these pictures earlier.

We propose to derive highest TemporalId values per layer. HighestTemporalIdList - a list of values of highest TemporalId for each layer in the TargetDecLayerIdList is thus proposed to be derived. For this maximum number of temporal sub-layers that may be present in each layer is proposed to be signalled in VPS. We propose to invoke the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in sub-clause Error: Reference source not found when TemporalId is equal to HighestTemporalIdList[nuh_layer_id] during the  decoding process for ending the decoding of a coded picture with nuh_layer_id greater than zero as specified in F.8.1.2.

# Proposal

In this document :

- We propose to signal the information regarding maximum number of temporal sub-layers that may be present in each layer in VPS.

- We propose to derive HighestTemporalIdList - a list of values of highest TemporalId for each layer in the TargetDecLayerIdList.

We propose to invoke the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in sub-clause Error: Reference source not found when TemporalId is equal to HighestTemporalIdList[nuh_layer_id] during the  decoding process for ending the decoding of a coded picture with nuh_layer_id greater than zero as specified in F.8.1.2.

Proposed changes related to signalling the information regarding maximum number of temporal sub-layers that may be present in each layer in VPS are highlighted below compared to JCTVC-N1008 [1].

It is proposed that this signalling in VPS regarding maximum number of temporal sub-layers that may be present in each layer in VPS be also considered independently of other aspects of this proposal.

- F.7.3.2.1.1 Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_offset | u(16) |
| …. |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) |  |
| sub_layers_vps_max_minus1[ i ] | u(3) |
| max_tid_ref_present_flag | u(1) |
| … |  |
| } |  |

sub_layers_vps_max_minus1[ i ] plus 1 specifies the maximum number of temporal sub-layers that may be present in the CVS for layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. The value of sub_layers_vps_max_minus1[ i ] shall be in the range of 0 to vps_max_sub_layers_minus1 inclusive. When not present sub_layers_vps_max_minus1[ i ] shall be equal to vps_max_sub_layers_minus1.

Proposed changes related to derivation of HighestTemporalIdList - a list of values of highest TemporalId for each layer in the TargetDecLayerIdList are highlighted below compared to JCTVC-L1003 [3].

It should be noted that in a variant these proposed changes could be instead done in section 8.1 General Decoding Process of JCTVC-N1008.

C.1 General

This annex specifies the hypothetical reference decoder (HRD) and its use to check bitstream and decoder conformance.

Two types of bitstreams or bitstream subsets are subject to HRD conformance checking for this Specification. The first type, called a Type I bitstream, is a NAL unit stream containing only the VCL NAL units and NAL units with nal_unit_type equal to FD_NUT (filler data NAL units) for all access units in the bitstream. The second type, called a Type II bitstream, contains, in addition to the VCL NAL units and filler data NAL units for all access units in the bitstream, at least one of the following:

–	additional non-VCL NAL units other than filler data NAL units,

–	all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex Error: Reference source not found).

…

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic subclauses of clause Error: Reference source not found, Annexes Error: Reference source not found and Error: Reference source not found.

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled through the hrd_parameters( ) syntax structure, which may be part of the SPS syntax structure or the VPS syntax structure.

Multiple tests may be needed for checking the conformance of a bitstream, which is referred to as the bitstream under test. For each test, the following steps apply in the order listed:

- An operation point under test, denoted as TargetOp, is selected. The layer identifier list OpLayerIdList of TargetOp consists of the list of nuh_layer_id values, in increasing order of nuh_layer_id values, present in the bitstream subset associated with TargetOp, which is a subset of the nuh_layer_id values present in the bitstream under test. The OpTid of TargetOp is equal to the highest TemporalId present in the bitstream subset associated with TargetOp.

- TargetDecLayerIdList is set equal to OpLayerIdList of TargetOp, HighestTid is set equal to OpTid of TargetOp, and the sub-bitstream extraction process as specified in clause Error: Reference source not found is invoked with the bitstream under test, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to BitstreamToDecode.

- HighestTemporalIdList consists of list of values of highest TemporalId present in the bitstream subset associated with TargetOp in order for each of the layer in the TargetDecLayerIdList.

The HighestTemporalIdList could be derived as follows

for( i=0; i < number of layers in TargetDecLayerIdList;i++) {
		HighestTemporalIdList[ i ] = min( HighestTid, sub_layers_vps_max_minus1[ LayerIdxInVps[ TargetDecLayerIdList[ i ] ] );
	}

- The hrd_parameters( ) syntax structure and the sub_layer_hrd_parameters( ) syntax structure applicable to TargetOp are selected.

...

Proposed change related to invoking the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in sub-clause Error: Reference source not found are highlighted below compared to JCTVC-N1008 [1].

F.8 Decoding process

- F.8.1 General decoding process

The specifications in subclause 8.1 apply with following additions.

When the current picture has nuh_layer_id greater than 0, the following applies.

–	Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the following decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the following decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb, and SCr, respectively.

NOTE – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

–	The decoding process operates as follows for the current picture CurrPic.

–	For the decoding of the slice segment header of the first slice, in decoding order, of the current picture, the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	When If ViewScalExtLayerFlag[ nuh_layer_id ] is equal to 1, the decoding process for a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	Otherwise, when DependencyId[ nuh_layer_id] is greater than 0, the decoding process for a coded picture with nuh_layer_id greater than 0 specified in subclause H.8.1.1 is invoked.

–	After all slices of the current picture have been decoded, the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

- F.8.1.1 Decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0

Each picture referred to in this subclause is a complete coded picture.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause Error: Reference source not found. This needs to be invoked only for the first slice segment of a picture. It is a requirement of bitstream conformance that PicOrderCntVal shall remain unchanged within an access unit.

–	The decoding process for RPS in subclause Error: Reference source not found is invoked, wherein only reference pictures with a nuh_layer_id equal to that of CurrPic may be marked as "unused for reference" or "used for long-term reference" and any picture with a different value of nuh_layer_id is not marked. This needs to be invoked only for the first slice segment of a picture.

–	When FirstPicInLayerDecodedFlag[ nuh_layer_id ] is equal to 0, the decoding process for generating unavailable reference pictures specified in subclause Error: Reference source not found is invoked, which needs to be invoked only for the first slice segment of a picture.

- F.8.1.2 Decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0

PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, if LayerInitialisedFlag[ nuh_layer_id ] is equal to 0, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

The following applies:

–	If discardable_flag is equal to 1, the decoded picture is marked as "unused for reference".

–	Otherwise, the decoded picture is marked as "used for short-term reference".

When TemporalId is equal to HighestTid HighestTemporalIdList[ i ] where nuh_layer_id of the current layer is equal to TargetDecLayerIdList[ i ], the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in subclause Error: Reference source not found is invoked with latestDecLayerId equal to nuh_layer_id as input.

FirstPicInLayerDecodedFlag[ nuh_layer_id ] is set equal to 1.

- F.8.1.2.1 Marking process for sub-layer non-reference pictures not needed for inter-layer prediction

Input to this process is:

–	a nuh_layer_id value latestDecLayerId

Output of this process is:

–	potentially updated marking as "unused for reference" for some decoded pictures

NOTE – This process marks pictures that are not needed for inter or inter-layer prediction as "unused for reference". When TemporalId is less than HighestTid HighestTemporalIdList[ i ] where nuh_layer_id of the current layer is equal to TargetDecLayerIdList[ i ], the current picture may be used for reference in inter prediction and this process is not invoked.

- The variables numTargetDecLayers, and latestDecIdx are derived as follows:

–	numTargetDecLayers is set equal to the number of entries in TargetDecLayerIdList.

–	latestDecIdx is set equal to the value of i for which TargetDecLayerIdList[ i ] is equal to latestDecLayerId.

- For i in the range of 0 to latestDecIdx, inclusive, the following applies for marking of pictures as "unused for reference":

–	Let currPic be the picture in the current access unit with nuh_layer_id equal to TargetDecLayerIdList[ i ].

–	When currPic is marked as "used for reference" and is a sub-layer non-reference picture, the following applies:

–	The variable currTid is set equal to the value of TemporalId of currPic.

–	The variable remainingInterLayerReferencesFlag is derived as specified in the following:

remainingInterLayerReferencesFlag = 0
		if ( currTid  <=  ( max_tid_il_ref_pics_plus1[ LayerIdxInVps[ TargetDecLayerIdList[ i ] ] ] –1 ) )
			for( j = latestDecIdx + 1; j < numTargetDecLayers; j++ )
				for( k = 0; k < NumDirectRefLayers[ TargetDecLayerIdList[ j ] ]; k++ )
					if( TargetDecLayerIdList[ i ]  = =  RefLayerId[ TargetDecLayerIdList[ j ] ][ k ] )
						remainingInterLayerReferencesFlag = 1

When remainingInterLayerReferenceFlag is equal to 0, currPic is marked as "unused for reference".

# Conclusion

In this document :

- We propose to signal the information regarding maximum number of temporal sub-layers that may be present in each layer in VPS.

- We propose to derive HighestTemporalIdList - a list of values of highest TemporalId for each layer in the TargetDecLayerIdList.

We propose to invoke the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in sub-clause Error: Reference source not found when TemporalId is equal to HighestTemporalIdList[nuh_layer_id] during the  decoding process for ending the decoding of a coded picture with nuh_layer_id greater than zero as specified in F.8.1.2.

We propose to adopt the proposal in the draft of SHVC and MV-HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Draft Text 3,” JCTVC-N1008, Vienna, August 2013

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 5 (ISO/IEC 23008-2:201x/PDAM2),” JCT3V-E1004, Vienna, August 2013.

[3] B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 10,” JCTVC-L1003, Geneva, January 2013.