| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0120 WG11 Number: m30861 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0065 |
| --- | --- |

| Title: | On Inter-layer Reference Picture Set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

It is asserted that when different layers have different frame rates and when max_one_active_ref_layer_flag is equal to 1 or  NumDirectRefLayers[ nuh_layer_id ]  is equal to 1 or  all_ref_layers_active_flag is equal to 1 the decoder is not able to distinguish between the case that an AU had no coded picture for a direct reference layer of a current layer due to that picture not existing in the bitstream (e.g. due to the reference layer having different frame rate) versus the case that the coded picture for the direct reference layer of a current layer was lost during transmission. This document proposes solution to distinguish between these cases. Two alternative solutions are proposed.

- A slice segment header signalling and modification to the decoding process for inter-layer reference picture set is proposed.

- A VPS signalling and other semantics change is proposed

# Introduction

It is asserted that when different layers have different frame rates and when max_one_active_ref_layer_flag is equal to 1 or  NumDirectRefLayers[ nuh_layer_id ]  is equal to 1 or  all_ref_layers_active_flag is equal to 1 the decoder is not able to distinguish between the case that an AU had no coded picture for a direct reference layer of a current layer due to that picture not existing in the bitstream (e.g. due to the reference layer having different frame rate) versus the case that the coded picture for the direct reference layer of a current layer was lost during transmission. Thus in both those cases currently when max_one_active_ref_layer_flag is equal to 1 or  NumDirectRefLayers[ nuh_layer_id ]  is equal to 1 or  all_ref_layers_active_flag is equal to 1, a “No reference picture” would be inferred during the decoding process for the inter-layer reference picture set. It is asserted that this is incorrect.

This document proposes solution to distinguish between these cases. Two alternative solutions are proposed.

- Option 1: A slice segment header signalling and modification to the decoding process for inter-layer reference picture set is proposed.

- Option 2: A VPS signalling and other semantics change is proposed.

# Proposal

Two alternative options for solutions are proposed.

- Option 1: A slice segment header signalling and modification to the decoding process for inter-layer reference picture set is proposed.

We propose signalling information regarding layers present in AU in slice segment header for NumActiveRefLayers. This information is used to modify the decoding process for inter-layer reference picture set.

- Option 2: A VPS signalling and other semantics change is proposed:

We propose signaling in VPS extension information regarding the maximum number of temporal sub-layers for each layer in SHVC and MV-HEVC. This information is used to modify the derivation of NumActiveRefLayers and semantics of inter_layer_pred_layer_idc[ i ].

Proposed changes related to signalling, semantics and decoding process for option1 and option 2 are highlighted below compared to JCTVC-N1008 [1].

Option 1:

- F.7.3.6 Slice segment header syntax

- F.7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| …. |  |
| if( nuh_layer_id > 0 && all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( nuh_layer_id > 0 && (max_one_active_ref_layer_flag ¦¦ (NumDirectRefLayers[ nuh_layer_id ] == 1 ) ¦¦ all_ref_layers_active_flag ) { |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| layer_present_in_au_flag[ i ] | u(1) |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| … |  |
| } |  |

layer_present_in_au_flag[ i ] equal to 1 specifies that a picture with nuh_layer_id equal to RefPicLayerId[ i ]  is present in the current access unit. layer_present_in_au_flag[ i ] equal to 0 specifies that a picture with nuh_layer_id equal to RefPicLayerId[ i ]  is not present in the current access unit.  When not present layer_present_in_au_flag[ i ] is inferred to be equal to 1.

- H.8.1.2 Decoding process for inter-layer reference picture set

Outputs of this process are updated lists of inter-layer reference pictures RefPicSetInterLayer0 and RefPicSetInterLayer1 and the variables NumActiveRefLayerPics0 and NumActiveRefLayerPics1.

The variable currLayerId is set equal to nuh_layer_id of the current decoded picture

The lists RefPicSetInterLayer0 and RefPicSetInterLayer1 are first emptied, NumActiveRefLayerPics0 and NumActiveRefLayerPics1 are set equal to 0 and the following applies:

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	if(layer_present_in_au_flag[ i ]) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
		nuh_layer_id equal to RefPicLayerId[ i ] ) {
		an interlayer reference picture rsPic is derived by invoking the subclause H.8.1.4 with picX and
			  RefPicLayerId[ i ] given as inputs 
		if( ( ViewId[ nuh_layer_id ]  <=  ViewId[ 0 ]   &&
									ViewId[ nuh_layer_id ]  <=  ViewId[ RefPicLayerId[ i ] ] )  | |
			( ViewId[ nuh_layer_id ]  >=  ViewId[ 0 ]  &&
									ViewId[ nuh_layer_id ]  >=  ViewId[ RefPicLayerId[ i ] ] ) ) {
			RefPicSetInterLayer0[ NumActiveRefLayerPics0 ] = rsPic
			RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] is marked as "used for long-term reference"
		} else {
			RefPicSetInterLayer1[ NumActiveRefLayerPics1 ] = rsPic
			RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] is marked as "used for long-term reference"
		}
	} else
		RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] = "no reference picture"
	}
}

Option 2:

- F.7.3.2.1.1 Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_offset | u(16) |
| …. |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) |  |
| sub_layers_vps_max_minus1[ i ] | u(3) |
| max_tid_ref_present_flag | u(1) |
| … |  |
| } |  |

sub_layers_vps_max_minus1[ i ] plus 1 specifies the maximum number of temporal sub-layers that may be present in the CVS for layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. The value of sub_layers_vps_max_minus1[ i ] shall be in the range of 0 to vps_max_sub_layers_minus1 inclusive. When not present sub_layers_vps_max_minus1[ i ] shall be equal to vps_max_sub_layers_minus1.

- F.7.4.7 Slice segment header semantics

- F.7.4.7.1 General slice segment header semantics

The specifications in subclause 7.4.7.1 apply with the following modifications.

–	"When nal_unit_type has a value in the range of 16 to 23, inclusive (IRAP picture), slice_type shall be equal to 2." is replaced by "When nal_unit_type has a value in the range of 16 to 23 and nuh_layer_id is equal to 0, inclusive (IRAP picture), slice_type shall be equal to 2."

…

inter_layer_pred_enabled_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the current picture. inter_layer_pred_enabled_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the current picture.

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictures that may be used in decoding of the current picture for inter-layer prediction. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0 )
   NumActiveRefLayerPics = 0
else if( all_ref_layers_active_flag ){ 
   NumActiveRefLayerPics = NumDirectRefLayers[ nuh_layer_id ] 
   for( i = 0; i < NumDirectRefLayers[ nuh_layer_id ]; i++) {
     if( sub_layers_vps_max_minus1[ LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ i ] ] ] < TemporalId ) 
	NumActiveRefLayerPics = NumActiveRefLayerPics - 1
	}
}
else if( !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 ) {
    if( sub_layers_vps_max_minus1[ LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ 0 ] ] ] < TemporalId ) 
	       NumActiveRefLayerPics = 0
	else
	       NumActiveRefLayerPics = 1
	}
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

inter_layer_pred_layer_idc[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the syntax element inter_layer_pred_layer_idc[ i ] is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of inter_layer_pred_layer_idc[ i ] shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive. When not present, the value of inter_layer_pred_layer_idc[ i ] is inferred to be equal to I as follows: 
	for( i = 0, j = 0; i < NumDirectRefLayers[ nuh_layer_id ]; i++) {
	   if( sub_layers_vps_max_minus1[ LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ i ] ] ] >= TemporalId ) 
			inter_layer_pred_layer_idc[ j++ ] = i; 
	}

When i is greater than 0, inter_layer_pred_layer_idc[ i ] shall be greater than inter_layer_pred_layer_idc[ i − 1 ].

The variables RefPicLayerId[ i ] for all values of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, are derived as follows:

for( i = 0, j = 0; i < NumActiveRefLayerPics; i++)
	RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]

All slices of a picture shall have the same value of inter_layer_pred_layer_idc[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive.

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, either of the following two conditions shall be true:

–	The value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] is greater than TemporalId.

–	The values of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] and TemporalId are both equal to 0 and the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is an IRAP picture.

# Conclusion

It is asserted that when different layers have different frame rates and when max_one_active_ref_layer_flag is equal to 1 or  NumDirectRefLayers[ nuh_layer_id ]  is equal to 1 or  all_ref_layers_active_flag is equal to 1 the decoder is not able to distinguish between the case that an AU had no coded picture for a direct reference layer of a current layer due to that picture not existing in the bitstream (e.g. due to the reference layer having different frame rate) versus the case that the coded picture for the direct reference layer of a current layer was lost during transmission. This document proposes solution to distinguish between these cases. Two alternative solutions are proposed.

- Option 1: A slice segment header signalling and modification to the decoding process for inter-layer reference picture set is proposed.

- Option 2: A VPS signalling and other semantics change is proposed.

We propose to adopt either option 1 or option 2 in the draft of SHVC and MV-HEVC. We have preference for option 1.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Draft Text 3,” JCTVC-N1008, Vienna, August 2013

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 5 (ISO/IEC 23008-2:201x/PDAM2),” JCT3V-E1004, Vienna, August 2013.