| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0174 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0066 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Reference picture set/list derivation independent of ViewId |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution attempts to make the inter-layer reference picture set decoding process is independent of scalability dimensions in use and the same in MV-HEVC and SHVC, while initial reference picture lists obtained with the proposal can reportedly be identical to those achieved by the current MV-HEVC and SHVC draft standards.

The following changes are proposed:

- Specify zero or more partitionings of active inter-layer reference pictures in the VPS extension. Each partitioning is assigned an index. Each inter-layer reference picture partitioning specifies the division of active inter-layer reference pictures into two subsets (0 and 1, which correspond to RefPicSetInterLayer0 and RefPicSetInterLayer1, respectively, in the current MV-HEVC/SHVC draft text).

- Include a gating flag, il_rps_partition_enabled_flag, in the PPS. The gating flag controls the presence of the il_rps_partition_idx syntax element in the slice segment header.

- When there are more than one active inter-layer reference picture and when il_rps_partition_enabled_flag is equal to 1, the slice header includes an index of the inter-layer reference picture set partitioning, il_rps_partition_idx, to be used (when the number of partitionings is greater than 1).

# Introduction

In the present draft text of MV-HEVC and SHVC, the inter-layer reference pictures are selected and placed into the reference picture list(s) using the following steps:

- The VPS extension (direct_dependency_flag[ i ][ j ]) is used to specify the layers which may be used for inter-layer prediction.

- Active inter-layer reference pictures can be indicated in the slice header (inter_layer_pred_layer_idc[ i ]) and can be a subset of the layers indicated by the VPS extension. An inter-layer reference picture set, consisting of two subsets RefPicSetInterLayer0 and RefPicSetInterLayer1, is decoded accordingly. The inter-layer reference picture set decoding process depends on the ViewId value specified in MV-HEVC.

- Reference picture lists are initialized to include the pictures in the inter-layer reference picture set. The pictures in RefPicSetInterLayer0 are located in a different position within the reference picture list(s) from the position of the pictures in RefPicSetInterLayer1.

- Reference picture lists may be modified.

It is the opinion of the proponent that the following features would be desirable in the inter-layer reference picture set and list related design:

- It would be desirable to use the same decoding process for MV-HEVC and SHVC without specific references to scalability dimensions, such as ViewId. It is asserted that this would:

  - Minimize the implementation burden.

  - Make it unambiguous which process to use for combined scalability (e.g. combined multiview and SNR scalability) and make combined scalability more compatible with the currently foreseen MV-HEVC and SHVC profiles.

  - Make it unambiguous which process to use for potential other types of scalabilities, such as depth in 3D-HEVC, and make other types of scalabilities more compatible with the currently foreseen MV-HEVC and SHVC profiles.

- It would be desirable to control the reference picture list initialization to enable the same initialization results as the current MV-HEVC and SHVC drafts, respectively, in order to initialize the reference picture lists in a suitable manner for MV-HEVC and SHVC and to maintain the same RD performance.

# Proposal

The following changes are proposed:

- Specify zero or more partitionings of active inter-layer reference pictures in the VPS extension. Each partitioning is assigned an index.

Each inter-layer reference picture partitioning specifies the division of active inter-layer reference pictures into two subsets (0 and 1, which correspond to RefPicSetInterLayer0 and RefPicSetInterLayer1, respectively).

- Include a gating flag, il_rps_partition_enabled_flag, in the PPS. The gating flag controls the presence of the il_rps_partition_idx syntax element in the slice segment header.

- When there are more than one active inter-layer reference picture and when il_rps_partition_enabled_flag is equal to 1, the slice header includes an index of the inter-layer reference picture set partitioning, il_rps_partition_idx, to be used (when the number of partitionings is greater than 1) or il_rps_partition_idx is inferred to be equal to 0 (when the number of partitionings is equal to 1).

- The inter-layer reference picture set decoding process is independent of scalability dimensions in use and the same in MV-HEVC and SHVC.

# Proposed Specification Text

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| num_il_rps_partitionings | ue(v) |
| for( i = 0; i < num_il_rps_partitionings; i++ ) { |  |
| il_rps_partitioning_num_pics_minus1[ i ] | ue(v) |
| for( j = 0; j <= il_rps_partitioning_num_pics_minus1[ i ]; j++ ) |  |
| il_rps_subset0_flag[ i ][ j ] | u(1) |
| } |  |
| … |  |
| } |  |

num_il_rps_partitionings specifies the number of partitionings of active inter-layer reference pictures into lists RefPicSetInterLayer0 and RefPicSetInterLayer1.

il_rps_partitioning_num_pics_minus1[ i ] + 1 specifies the number of pictures included in the i-th partitioning.

il_rps_subset0_flag[ i ][ j ] equal to 0 specifies that RefPicLayerId[ j ] in a picture referring to the i-th partitioning is mapped to RefPicSetInterLayer1. il_rps_subset0_flag[ i ][ j ] equal to 1 specifies that RefPicLayerId[ j ] in a picture referring to the i-th partitioning is mapped to RefPicSetInterLayer0.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 ) |  |
| il_rps_partition_enabled_flag | u(1) |
| lists_modification_present_flag | u(1) |
| … |  |

il_rps_partition_enabled_flag is used to determine to presence of il_rps_partition_idx in the slice segment header. When not present, il_rps_partition_enabled_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that when il_rps_partition_enabled_flag is equal to 1 for an active PPS, num_il_rps_partitionings shall be greater than 0 in the active VPS.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 && all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( il_rps_partition_enabled_flag && NumActiveRefLayerPics > 0 && num_il_rps_partitionings > 1 ) |  |
| il_rps_partition_idx | u(v) |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| … |  |
| } |  |

il_rps_partition_idx specifies that the syntax elements il_rps_partitioning_num_pics_minus1[ il_rps_partition_idx ] and il_rps_subset0_flag[ il_rps_partition_idx ][ j ] are used in the decoding process for inter-layer reference picture set. When il_rps_partition_enabled_flag is equal to 1 and NumActiveRefLayers is greater than 0 and num_il_rps_partitionings is equal to 1, il_rps_partition_idx is inferred to be equal to 0. The length of the il_rps_partition_idx syntax element is Ceil( Log2( num_il_rps_partitionings ) ) bits. The value of il_rps_partition_idx shall be in the range of 0 to num_il_partitionings – 1, inclusive.

- G.8.1.2	Decoding process for inter-layer reference picture set

Outputs of this process are updated lists of inter-layer pictures RefPicSetInterLayer0 and RefPicSetInterLayer1 and the variables NumActiveRefLayerPics0 and NumActiveRefLayerPics1.

The lists RefPicSetInterLayer0 and RefPicSetInterLayer1 are first emptied, NumActiveRefLayerPics0 and NumActiveRefLayerPics1 are set equal to 0 and the following applies:

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
										nuh_layer_id equal to RefPicLayerId[ i ] ) {
		if( ( ViewId[ nuh_layer_id ]  <=  ViewId[ 0 ]   &&
									ViewId[ nuh_layer_id ]  <=  ViewId[ RefPicLayerId[ i ] ] )  | |
			( ViewId[ nuh_layer_id ]  >=  ViewId[ 0 ]  &&
									ViewId[ nuh_layer_id ]  >=  ViewId[ RefPicLayerId[ i ] ] ) ) {
		if( !il_rps_partition_enabled_flag  | |  il_rps_subset0_flag[ il_rps_partition_idx ][ i ] )
			RefPicSetInterLayer0[ NumActiveRefLayerPics0 ] = picX
			RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] is marked as "used for long-term reference"
		} else {
			RefPicSetInterLayer1[ NumActiveRefLayerPics1 ] = picX
			RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] is marked as "used for long-term reference"
		}
	} else
		RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] = "no reference picture"
}

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer0 or RefPicSetInterLayer1.

When il_rps_partition_enabled_flag is equal to 1, it is a requirement of bitstream conformance that NumActiveRefLayers shall be less than or equal to il_rps_partitioning_num_pics_minus1[ i ] + 1.

If the current picture is a RADL picture, there shall be no entry in RefPicSetInterLayer0 or RefPicSetInterLayer1 that is a RASL picture.

NOTE – An access unit may contain both RASL and RADL pictures.

- H.8.1.2	Decoding process for inter-layer reference picture set

Outputs of this process are updated lists of inter-layer reference pictures RefPicSetInterLayer0 and RefPicSetInterLayer1 and the variables NumActiveRefLayerPics0 and NumActiveRefLayerPics1.

The variable currLayerId is set equal to nuh_layer_id of the current decoded picture

The lists RefPicSetInterLayer0 and RefPicSetInterLayer1 are first emptied, NumActiveRefLayerPics0 and NumActiveRefLayerPics1 are set equal to 0 and the following applies:

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
		nuh_layer_id equal to RefPicLayerId[ i ] ) {
		an interlayer reference picture rsPic is derived by invoking the subclause Error: Reference source not found with picX and
			  RefPicLayerId[ i ] given as inputs 
		if( ( ViewId[ nuh_layer_id ]  <=  ViewId[ 0 ]   &&
									ViewId[ nuh_layer_id ]  <=  ViewId[ RefPicLayerId[ i ] ] )  | |
			( ViewId[ nuh_layer_id ]  >=  ViewId[ 0 ]  &&
									ViewId[ nuh_layer_id ]  >=  ViewId[ RefPicLayerId[ i ] ] ) ) {
			RefPicSetInterLayer0[ NumActiveRefLayerPics0 ] = rsPic
			RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] is marked as "used for long-term reference"
		} else {
			RefPicSetInterLayer1[ NumActiveRefLayerPics1 ] = rsPic
			RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] is marked as "used for long-term reference"
		}
	} else
		RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] = "no reference picture"
}

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer0 or RefPicSetInterLayer1.

NOTE – For the profiles defined in Annex H, RefPicSetInterLayer1 is always empty since the value of ViewId[ i ] is equal to zero for all layers.

If the current picture is a RADL picture, there shall be no entry in the RefPicSetInterLayer0 or RefPicSetInterLayer1 that is a RASL picture.

NOTE – An access unit may contain both RASL and RADL pictures.

# Examples

With the PIP inter-view prediction hierarchy and in the typical SHVC use cases, the encoder can set num_il_rps_partitionings to 0, which causes inter-layer reference picture(s) to be included in the list RefPicSetInterLayer0, as in the current MV-HEVC and SHVC drafts, respectively.

In the IBP inter-view prediction hierarchy, the encoder can perform as follows:

- In the VPS extension, the encoder can set

  - num_il_rps_partitionings equal to 1

  - il_rps_partitioning_num_pics_minus1[ 0 ] equal to 1

  - il_rps_subset0_flag[ 0 ][ 0 ] equal to 1

  - il_rps_subset0_flag[ 0 ][ 1 ] equal to 0

- In the PPS extension, the encoder can set il_rps_partition_enabled_flag equal to 1.

- No additional syntax elements are hence included in the slice segment header.

When encoding/decoding a picture in the P view, il_rps_subset0_flag[ 0 ][ 0 ] (equal to 1) causes the inter-layer reference picture from the I view to be included in RefPicSetInterLayer0.

When encoding/decoding a picture in the B view, il_rps_subset0_flag[ 0 ][ 0 ] (equal to 1) causes the inter-layer reference picture from the I view to be included in RefPicSetInterLayer0 and il_rps_subset0_flag[ 0 ][ 1 ] (equal to 0) causes the inter-layer reference picture from the I view to be included in RefPicSetInterLayer1.

Thus, with this encoder operation, the consequences are identical to the current MV-HEVC text with ViewId-specific inter-layer reference picture set decoding.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).