| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0175 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0067 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Indications related to single-loop decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Hoda Roodaki Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is proposed to replace the discardable_flag syntax element with a 2-bit indication, pic_ref_idc, having values with the following semantics:

- 3 = discardable picture.

- 2 = intra-layer non-reference picture. Specifies that the picture is not used as a reference picture for inter prediction of subsequent pictures in decoding order within the same layer and that the picture may be used as an inter-layer prediction reference.

- 1 = non-output-layer skip (NOLS) picture, which may be used as a reference for other NOLS and discardable pictures in the same layer but is not used as a reference for inter-layer prediction.

- 0 specifies that the picture may be used as a reference for inter prediction of subsequent pictures in decoding order within the same layer and may be used as a reference for inter-layer prediction.

It is asserted that encoders may use prediction structures including intra-layer non-reference pictures and/or NOLS pictures to omit the decoding of some parts of the base layer and to reduce the DPB memory usage for the decoded base layer pictures.

# Introduction

## Encoding constraints for reduced base-layer decoding complexity and/or DPB memory use

Single-loop decoding aims at reducing the decoding complexity by omitting the decoding of some parts of the base layer and at reducing the DPB memory usage for the decoded base layer pictures.

It is asserted in this contribution that either of the following encoding constraints of MV-HEVC/SHVC enables a decoder operation that targets at the output of an enhancement layer to use a smaller number of base-layer picture storage buffers and to decode a smaller number of base-layer pictures:

- Turning off inter-layer sample prediction from selected levels of base layer prediction structure:

The enhancement layer pictures use for example the prediction hierarchy of the random access common conditions and have the same TemporalId value to reach a maximum compression performance.

The base layer pictures are split into two (or more) temporal levels, even though it might not be possible to set their TemporalId value accordingly, because no temporal scalability is required to be used in the enhancement layer. No inter-layer prediction from selected higher temporal levels of the base-layer prediction structure takes place. Consequently, decoding of those base layer pictures at higher temporal levels is not required when the output of the enhancement layer is targeted.

For example, inter-layer prediction can be turned off for relative POC values 1 to 3 and 5 to 7, inclusive, in the random access common conditions. Similarly, the base layer is split into two temporal levels, the “base” temporal level consisting of pictures with relative POC values 0, 4, 8 and so on, and the “enhancement” temporal level consisting of pictures with relative POC values 1, 2, 3, 5, 6, 7, and so on. However, as the enhancement layer (with nuh_layer_id equal to 1) uses the prediction structure of the random access common conditions (with all pictures having the same TemporalId), the base-layer pictures also have the same TemporalId. The base-layer sample arrays of the “enhancement” temporal level need not be decoded, when the decoder outputs the enhancement layer.

- Intra-layer non-reference pictures:

Selected base-layer pictures are not used for inter prediction of other pictures in the same layer, while they may be used for inter-layer sample and motion prediction. These selected base-layer pictures need not be stored in he DPB after the decoding of the access unit has been finished.

Figure 1 provides an example where the relative POC values 1 to 3 and 5 to 7 are non-reference pictures within the base layer but are used as reference for inter-layer prediction. The enhancement layer GOP structure and prediction dependencies are kept the same as in the random access common conditions.

Figure 1. Example of intra-layer non-reference pictures (BL POCs 1 to 3 and 5 to 7).

Such encoding constraints could therefore be useful for reduced decoding complexity and/or DPB memory use similarly to more comprehensive single-loop decoding approaches.

## Related HEVC and MV-HEVC/SHVC features

In the HEVC design a picture is marked as “unused for reference”, when it is not included in the reference picture set (RPS) of a subsequent picture in decoding order. In other words, a picture cannot be marked as “unused for reference” immediately after its decoding.

In the MV-HEVC/SHVC design, a marking process for sub-layer non-reference pictures not needed for inter-layer prediction (clause F.8.1.2.1) has been specified and reportedly helps in saving DPB memory use. This process marks the pictures that fulfil the following as “unused for reference”:

- the picture is in the highest decoded TemporalId, i.e. the TemporalId of the picture is equal to HighestTid; and

- the picture is a sub-layer non-reference picture; and

- the picture is not used or is no longer used as inter-layer reference for any other picture (according to inter-layer prediction dependency information of the VPS extension).

As specified in clause 7.4.2.2, the value of temporal_id_plus1 is required to be the same in all VCL NAL units of an access unit. The relaxation of the constraint could be considered a non-backward-compatible change. Consequently, it is not possible for example to indicate dyadic nested temporal scalability in the temporal_id_plus1 values of the base layer and to use the non-temporally-scalable prediction structure of the common test conditions in the enhancement layer.

The vps_max_sub_layers_minus1 and sps_max_sub_layers_minus1 syntax elements specify the maximum number of temporal sub-layers that may be present in the bitstream. However, the actual number of temporal sub-layers present in the bitstream may be less than what vps_max_sub_layers_minus1 or sps_max_sub_layers_minus1 syntax specifies.

The MV-HEVC/SHVC design currently enables indicating in the VPS extension (with the max_tid_ref_pics_plus1[ i ] syntax element) whether inter-layer prediction may only take place from IRAP pictures or that inter-layer prediction may only take place from pictures up to the indicated TemporalId value.

# Overview of the proposal

It is proposed to replace the discardable_flag syntax element with a 2-bit indication, pic_ref_idc, having values with the following semantics:

- 3 = discardable picture. Specifies that the coded picture is not used as a reference picture for inter prediction and is not used as an inter-layer reference picture in the decoding process of subsequent pictures in decoding order

- 2 = intra-layer non-reference picture. Specifies that the picture is not used as a reference picture for inter prediction of subsequent pictures in decoding order within the same layer and that the picture may be used as an inter-layer prediction reference.

- 1 = non-output-layer skip (NOLS) picture. Specifies that the picture may be used as a reference for inter prediction of subsequent pictures in decoding order that have pic_ref_idc equal to 1 or 3 until the next picture in decoding order with pic_ref_idc equal to 0 and the same or lower TemporalId value than that of the current NOLS picture. Also specifies that the picture is not used as a reference for inter-layer prediction and not used as a reference for inter prediction of any picture with pic_ref_idc equal to 0 or 2.

- 0 specifies that the picture may be used as a reference for inter prediction of subsequent pictures in decoding order within the same layer and may be used as a reference for inter-layer prediction.

pic_ref_idc equal to 1 provides signalling for the use case 1 (“Turning off inter-layer sample prediction from selected levels of base layer prediction structure”).

pic_ref_idc equal to 2 provides signalling for the use case 2 (“Intra-layer non-reference pictures”). It is noted that the marking process for sub-layer non-reference pictures not needed for inter-layer prediction would cover this use case for sub-layer non-reference pictures that have TemporalId equal to highest temporal identifier selected to be decoded. However, the actual number of temporal sub-layers present in the bitstream may be less than what vps_max_sub_layers_minus1 or sps_max_sub_layers_minus1 syntax specifies, and hence it can be difficult to select the highest temporal identifier to be decoded to match the actual number of temporal sub-layers. A picture-wise indication not related to TemporalId was therefore considered useful.

# Simulation results

Use case 2 was simulated with SHM 3.0.1 using the random access common conditions for SNR scalability. As illustrated in Figure 1, every fourth picture of the base layer was a reference picture while the other pictures in the base layer were non-reference pictures. The enhancement layer prediction structure was kept unchanged compared to that of the random access common conditions. Inter-layer prediction was enabled for all access units. The SHM 3.0.1 anchor was used as the anchor for the comparison presented in Table 1.

Table 1. Experiment results (use case 2), every 4th picture is a reference picture in the BL 
(versus SHM 3.0.1).

|  | RA HEVC SNR |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U |  | V |
| Class A | 2.1% | 2.4% | 2.4% |  |
| Class B | 1.7% | 1.6% | 1.6% |  |
| Overall (Test vs Ref) | 1.8% | 1.9% | 1.8% |  |
| EL only (Test vs Ref) | -0.2% | -0.1% | -0.1% |  |

We provide the results of using nested temporal scalability and enabling inter-layer prediction every 4th picture as a reference for comparison. Such coding has the same base-layer DPB memory consumption as in the experiment presented in Table 1. The same anchor (SHM 3.0.1) was used also in these results presented in Table 2. As can be seen from the tables, the use of non-reference pictures in the base layer roughly halves the BD-bitrate penalty compared to the temporal scalability reference results.

Table 2. Reference results, nested temporal scalability, inter-layer prediction enabled for every 4th picture (versus SHM 3.0.1).

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 5.1% | 2.1% | 2.2% |
| Class B | 3.6% | 0.7% | -0.4% |
| Overall (Test vs Ref) | 4.0% | 1.1% | 0.3% |
| EL only (Test vs Ref) | 7.9% | 4.6% | 3.7% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).