| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0177 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0068 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On Layers Not Present SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung Won Kang, Jinho Lee, Hahyun Lee, Jin Soo Choi 218, Gajeong-no, Yuseong, Daejeon, Korea Truong Cong Thang Tsuruga, Ikki-Machi, Aizu-Wakamatsu, Japan 965-8580 | Tel: Email: | {jungwon, jinosoul, hanilee, jschoi}@ etri.re.kr thang@u-aizu.ac.jp |
| Source: | ETRI, University of Aizu |  |  |

_____________________________

# Abstract

Currently the layer not present SEI message provides the information of non-present layers after some modification. The current solution just focuses on the layers identified by the nuh_layer_id parameter. In practice, a CVS could be modified in temporal dimension as well. This contribution proposes to describe the (temporal) sub-layers which are not present in a CVS. The usefulness of the proposal is shown with some practical use cases.

# Introduction

Currently the layer not present SEI message provides the information of non-present layers after some modification. The current solution just focuses on the layers identified by the nuh_layer_id parameter. In practice, a CVS could be modified in temporal dimension as well. So, we propose to describe the temporal layers which are not present in a CVS.

This solution has several advantages in practice. In the following, the usefulness of the solution is described for decoders and extractors (or middle box).

For a decoder, the actual maximum number of sub layers is useful to decide whether the decoder is able to decode the given CVS, depending on the defined profiles and levels. In addition, knowing the actual maximum number of sub layers of a CVS, a client can better organize its resource to decode the CVS.

For an extractor in the networks, the information about the actual number of sub layers of a CVS helps the extractor better adapt the CVS. For example, given a CVS that has both temporal scalability and SNR scalability. The original number of sub layers is 6 and the frame rate is 60fps. The bitrate information corresponding to different layer sets and sub layers is provided in VPS VUI. After some processing on the content delivery path, the CVS is adapted so that it has 5 sub layers and the frame rate now is 30fps. Suppose that this adapted CVS is input to an extractor. If the extractor knows that its input CVS (or a period of its) has only 5 sub layers and its bitrate meet the throughput constraint, the extractor will simply pass the CVS to the receiver. However, if the extractor still thinks that the number of sub layers is 6, it will have to remove some SNR layers to meet the throughput constraint.

# Proposal

In the revised layers not present SEI message, the new value of vps_max_sub_layers_minus1 is signaled when needed. Also, another flag is used to enable/disable the layer present information in the message. The proposed syntax and semantics are as follows.

| layers_not_present( payloadSize ) { | Descriptor |
| --- | --- |
| lp_sei_active_vps_id | u(4) |
| changed_layers_flag | u(1) |
| changed_max_sub_layers_flag | u(1)` |
| if( changed_layers_flag) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) |  |
| layer_present_flag[ i ] | u(1) |
| if (changed_max_sub_layers_flag) |  |
| new_vps_max_sub_layers_minus1 | u(3) |
| } |  |

- changed_layers_flag equal to 1 indicates that the presence of at least one layer has been changed. This flag equal to 0 indicates that no layers specified in the related VPS are absent.

- changed_max_sub_layers_flag equal to 1 indicates that the maximum number of sub layers has been changed. This flag equal to 0 indicates that the maximum number of sub layers is unchanged.

- new_vps_max_sub_layers_minus1 plus 1 specifies new maximum number of sub layers of the CVS.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

University of Aizu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).