| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0179 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0069 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Improvements of Parameter Sets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung Won Kang, Jinho Lee, Hahyun Lee, Jin Soo Choi 218, Gajeong-no, Yuseong, Daejeon, Korea Truong Cong Thang Tsuruga, Ikki-Machi, Aizu-Wakamatsu, Japan 965-8580 | Tel: Email: | {jungwon, jinosoul, hanilee, jschoi}@ etri.re.kr thang@u-aizu.ac.jp |
| Source: | ETRI, University of Aizu |  |  |

_____________________________

# Abstract

This contribution proposes some improvements to the parameter sets. The first improvement is providing a flag in VPS that signals the presence of tiles in a CVS. The second improvement is the description of representation formats in VPS, including the index signaling and format signaling. Correspondingly, a revised updating mechanism of representation format in PPS is presented.

This revision provides a revision of rep_format() syntax structure according to the discussion of Parameter sets HLS BoG.

# Introduction

This contribution proposes some improvements to the parameter sets. The first improvement is providing a flag in VPS that signals the presence of tiles in a CVS. This signaling would be useful for a decoder to know more about the CVS and to better organize its resource to decode the CVS.

The second improvement is the description of representation formats in VPS. A small change in the format index signaling is proposed to have a more efficient parsing of the VPS. For rep_format() syntax structure, we propose to signal the presence of bit depth information and size information separately. Correspondingly, a revised updating mechanism of representation format in PPS is presented.

# Proposals

2.1 Tiles-enabled flag in VPS

In this part, we propose a flag in VPS that signal the presence of tiles in a coded video sequence (CVS). Specifically, a flag called vps_tiles_enabled_flag and it is used to condition the presence of tile_boundaries_aligned_flag[ i ][ j ] as follows.

| vps_vui( ){ | Descriptor |
| --- | --- |
| ... |  |
| vps_tiles_enabled_flag | u(1) |
| if (vps_tiles_enabled_flag) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < NumDirectRefLayers[ layer_id_in_nuh[ i ] ]; j++ ) { |  |
| tile_boundaries_aligned_flag[ i ][ j ] | u(1) |
| ... |  |
| } |  |

- vps_tiles_enabled_flag: equal to 1 specifies that more than one tile is present in at least one picture of the CVS. vps_tiles_enabled_flag equal to 0 specifies that only one tile is present in every picture of  the CVS.

2.2 Revision of representation formats in VPS and SPS

In this part, some revisions to improve the description of representation formats in VPS and SPS are proposed. Currently, the indexes of representation formats are provided as follows:

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0 ) |  |
| vps_rep_format_idx[ i ] | u(4) |
| … |  |
| } |  |

The above syntax is slightly revised as follows to enable more efficient VPS parsing. In this way, the VPS parsing process of a decoder or middle box does not need to build a “for” loop when vps_num_rep_formats_minus1 is equal to 0.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| if( rep_format_idx_present_flag && vps_num_rep_formats_minus1 > 0) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0 ) |  |
| vps_rep_format_idx[ i ] | u(4) |
| … |  |
| } |  |

Besides, the current representation format syntax structure may have a lot of information redundancy when, for example, only the picture size or bit depth is changed across some layers.  For that, we propose to add some flags to control the presence of different parts of syntax.

| rep_format( ) { | Descriptor |
| --- | --- |
| chroma_and_bit_depth_vps_present_flag | u(1) |
| pic_width_vps_in_luma_samples | u(16) |
| pic_height_vps_in_luma_samples | u(16) |
| if(chroma_and_bit_depth_vps_present_flag ) { |  |
| chroma_format_vps_idc | u(2) |
| if( chroma_format_vps_idc = = 3 ) |  |
| separate_colour_plane_vps_flag | u(1) |
| bit_depth_vps_luma_minus8 | u(4) |
| bit_depth_vps_chroma_minus8 | u(4) |
| } |  |
| } |  |

The semantics of the new syntax element is as follows.

- chroma_and_bit_depth_vps_present_flag: equal to 1 specifies that the syntax elements, chroma_format_vps_idc, separate_colour_plane_vps_flag, bit_depth_vps_luma_minus8, and bit_depth_vps_chroma_minus8 are present in the syntax structure.	chroma_and_bit_depth_vps_present_flag equal to 0 specifies that the syntax elements, chroma_format_vps_idc, separate_colour_plane_vps_flag, bit_depth_vps_luma_minus8, and bit_depth_vps_chroma_minus8 are inferred from the previous rep_format() syntax structure. The value of chroma_and_bit_depth_vps_present_flag of the first rep_format() syntax structure shall be equal to 1.

Correspondingly, in SPS, the syntax could be revised by replacing the update_re_format_flag by two flags, called update_chroma_size_flag and update_bit_depth_flag, that control the presence of updated information of chroma format and picture size and updated information of bit depth. This change also reduces the redundancy when, for example, only the picture size or bit depth is updated.

| Seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 ) { |  |
| update_rep_format_flag | u(1) |
| update_chroma_size_flag | u(1) |
| update_bit_depth_flag | u(1) |
| } |  |
| if( update_chroma_size_flag ) { |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| } |  |
| … |  |
| if( update_bit_depth_flag ) { |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| } |  |
| … |  |
| } |  |

The semantics of the new syntax elements is as follows.

update_chroma_size_flag equal to 1 specifies that the syntax elements chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, and pic_height_in_luma_samples are explicitly signalled in the SPS and all the layers with nuh_layer_id greater than zero that refer to this SPS use these values instead of those signalled in the VPS when the nuh_layer_id of the SPS is greater than 0. update_chroma_size_flag equal to 0 specifies that the syntax elements chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples are not signalled in the SPS and all the layers that refer to this SPS use the values signaled in the VPS. When not present, the value of update_chroma_size_flag is inferred to be equal to 1.

When a current picture with nuh_layer_id layerIdCurr greater than 0 refers to an SPS, the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples are inferred or constrained as follows:

–	If the nuh_layer_id of the active layer SPS is equal to 0, the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples are inferred to be equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS where j is equal to LayerIdxInVps[ layerIdCurr ] and the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples of the active layer SPS are ignored.

NOTE 1 – The values are inferred from the VPS when an active non-base layer references an SPS which is also used by the base layer, in which case the SPS has nuh_layer_id equal to 0. For an active base layer, the values in the active SPS apply.

–	Otherwise (the nuh_layer_id of the active layer SPS is greater than zero), the following applies:

–	If update_chroma_size_flag is equal to 0, the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples are inferred to be equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

–	Otherwise (update_chroma_size_flag is equal to 1), it is a requirement of bitstream conformance that the value of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, shall be less than or equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

update_bit_depth_flag equal to 1 specifies that the syntax elements bit_depth_luma_minus8 and bit_depth_chroma_minus8 are explicitly signalled in the SPS and all the layers with nuh_layer_id greater than zero that refer to this SPS use these values instead of those signalled in the VPS when the nuh_layer_id of the SPS is greater than 0. update_bit_depth_flag equal to 0 specifies that the syntax elements bit_depth_luma_minus8 and bit_depth_chroma_minus8 are not signalled in the SPS and all the layers that refer to this SPS use the values signaled in the VPS. When not present, the value of update_bit_depth_flag is inferred to be equal to 1.

When a current picture with nuh_layer_id layerIdCurr greater than 0 refers to an SPS, the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 are inferred or constrained as follows:

–	If the nuh_layer_id of the active layer SPS is equal to 0, the values of bit_depth_luma_minus8 and bit_depth_chroma_minus8 are inferred to be equal to bit_depth_vps_luma_minus8 and bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS where j is equal to LayerIdxInVps[ layerIdCurr ] and the values of bit_depth_luma_minus8 and bit_depth_chroma_minus8 of the active layer SPS are ignored.

NOTE 2 – The values are inferred from the VPS when an active non-base layer references an SPS which is also used by the base layer, in which case the SPS has nuh_layer_id equal to 0. For an active base layer, the values in the active SPS apply.

–	Otherwise (the nuh_layer_id of the active layer SPS is greater than zero), the following applies:

–	If update_bit_depth_flag is equal to 0, the values of bit_depth_luma_minus8 and bit_depth_chroma_minus8 are inferred to be equal to bit_depth_vps_luma_minus8 and bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

–	Otherwise (update_bit_depth_flag is equal to 1), it is a requirement of bitstream conformance that the value of bit_depth_luma_minus8 or bit_depth_chroma_minus8 shall be less than or equal to bit_depth_vps_luma_minus8 or bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

University of Aizu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).