| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0213v4 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0073v4 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On picture order count |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ye-Kui Wang Ying Chen Hendry Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA Ming Li Ping Wu ZTE Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 | Tel: Email: | 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com fhendry@qti.qualcomm.com 86-25-88014644 li.ming42@zte.com.cn ping.wu@zte.com.cn |
| Source: | Qualcomm Incorporated, ZTE |  |  |

_____________________________

# Abstract

This contribution presents an improved design for the signalling and derivation of picture order count in SHVC and MV-HEVC. It is proposed that two flags indicating reset of POC LSB and POC MSB be signalled in the slice header, in place of the one flag that is currently signalled for the POC reset. Additionally, a POC reset information SEI message is proposed to help recover the POC derivation process when pictures containing POC reset information is lost.

In v2 of this document, a variant of the original proposal is proposed, which is summarized as follows:

- Instead of using two bits, one for POC MSB reset and one for POC LSB reset, a two-bit indication is used, to fully utilize the fact that there would never be only POC LSB reset.

- The two bits, and possibly additional information for support of missing-collocated-picture scenarios as well as for resolving the error resilience issue, are signalled in the slice header extension instead of in the up-to-seven reserved slice header bits.

- A POC resetting period is specified based on the POC resetting period ID. Each non-IRAP picture that belongs to an access unit that contains at least one IRAP picture must be the start of a POC resetting period in the layer containing the non-IRAP picture. In that access unit, each picture would be the start of a POC resetting period in the layer containing the picture. POC resetting and update of POC values of same-layer pictures in the DPB are applied only for the first picture within each POC resetting period.

- For derivation of the delta POC value used for updating the POC values of the same-layer pictures in the DPB as well as for derivation of the POC MSB of the POC value of the current picture, a POC LSB value is conditionally signalled in the slice segment header.

- A loss of a picture at the start of a POC resetting period can be overcome by having one or more same-layer pictures that follow that picture in decoding order and that are assigned the value 3 of the two-bit indication. The same process for support of missing-collocated-picture scenarios apply for resolving the error resilience issue.

Text changes for the above variant of the proposal, with change marks in relative to the latest MV-HEVC WD in JCT3V-F1004v3, are included in the attachment.

The following SEI messages have a POC-dependent persistence scope:

- Recovery point SEI message

- Progressive refinement segment start SEI message

It is proposed to further restrict the end of persistence scope of such an SEI message to be earlier than the next picture, in decoding order, that is in the same layer as the picture associated with the SEI message and that updates the picture order count values of pictures in the DPB.

In v3 of this document, the following two corrections were made to the attached specification text:

- Typo correction of "poc_reset_pic_id" to "poc_reset_period_id".

- When the function getCurrMsb( ) is called when poc_reset_idc equal to 3, the second input parameter, representing the previous POC LSB, is changed from "poc_lsb_val" to "full_poc_reset_flag ? 0 : poc_lsb_val".

In v4 of this document, the proposal was further revised to address comments received during the discussion of this document at the 2nd POC conference call on Nov. 21 as well as to fix two bugs that existed in v3. The updated proposal is summarized as follows:

- Revised the POC update mechanism to update the POC values of earlier pictures of all layers in the DPB at the beginning of each access unit that requires POC reset (before the decoding of the first picture received for the access unit but after parsing and decoding of the slice header information of the first slice of that picture). However, POC derivation is still independent of information from any other layer, because it is required that the delta derived based on in-layer infomation is the same for all pictures in the access unit.

- To fix the two bugs, revised the condition for signalling of poc_lsb_val, such that it is also sent for base layer IDR pictures with poc_reset_idc equal to 2, and added the definition of FirstPicInLayerDecodedFlag[ i ] also for i equal to 0.

In the attachment containing required spec text changes, the new text changes (in relative to v3) are included in the attachment, marked by reviewer names "Version#4" and "Fix02". The changes marked by "Fixes 02" are asserted to be editoiral bug-fixing changes that are generally needed.

# Introduction

The SHVC and MV-HEVC specifications allow access units that contain both IRAP pictures and non-IRAP pictures. In the previous meeting, it was proposed that the POC reset information be signalled in the slice header in the form of poc_reset_flag, and this flag would be used to reset the POC of the current picture to 0, and also decrement the POC of the pictures in the DPB with the same value of nuh_layer_id as the current picture by a value that is equal to the POC of the current picture before reset. The flag is signalled in the reserved bits of the slice segment header, and signalled not just for the enhancement layer, but also for the base layer.

# Problem Statement

The presence of the POC reset information in the base layer only indicates that the POC derivation process would be different in case of an operating point containing only the base layer, and other operating points. This POC signalling has the following shortcomings:

- When an access unit auA contains an enhancement layer picture that is an IRAP picture with NoRaslOutputFlag equal to 1, and a base layer picture that is non-IRAP (assume two layer bitstream for simplicity), and either the value of the signalled slice_pic_order_cnt_lsb of the EL picture is non-zero, or the derived POC MSB value of the base layer when decoding starts from any preceding IRAP access unit is non-zero, then in order that POC of all the picture in an access unit be aligned, the POC has to be reset at the base layer, and additionally in the enhancement layer when the signalled slice_pic_order_cnt_lsb at the EL picture is non-zero. Under this circumstance, the POC LSB of base layer pictures following auA in decoding order will be assuming the reset value. Consider the example shown in Figure 1, with MaxPicOrderCntLsb equal to 128 for both the base and the enhancement layers. The bitstream has two layers, and the EL picture in AU 241 is an IRAP picture with NoRaslOutputFlag equal to 1. The POC, therefore, is reset for the base layer pictures. In a multi-layer codec, the signalling results in aligned POC values for pictures in an access unit. However, when the bitstream would be decoded by a version 1 HEVC decoder, the POC derived for the BL pictures would be incorrect in their relative ordering with other pictures.

Figure 1: POC reset when an AU contains a non-IRAP BL picture and an IRAP EL picture with NoRaslOutputFlag equal to 1. The numbers in green indicate the POC LSB and MSB after POC is reset at AU 241. The numbers in red indicate that the relative POC numbers are incorrect.

- When the picture containing POC reset information is lost, the derivation of the POC for succeeding pictures in the same layer may be incorrect.

# Proposal

The proposal has two components:

- In the first part, signalling of one flag each for POC LSB and POC MSB reset is proposed in the reserved slice segment header

- In the second part, it is also proposed that the POC reset information be constrained so that it does not occur at a picture that does has non-zero value of temporal ID, or that is a sub-layer non-reference picture, RASL picture or a RADL picture. Additionally, a POC reset information SEI message is proposed to be signalled to provide information for POC derivation when a picture preceding picA decoding order containing POC reset information (i.e. picture that has either POC LSB reset or POC MSB reset) is lost.

The changes with respect to the JCTVC-E1004_v6 are highlighted in green, or red strikethrough.

## POC LSB and POC MSB reset

### Slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| i = 0 |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| poc_msb_reset_flag | u(1) |
| } |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| poc_lsb_reset_flag | u(1) |
| } |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| discardable_flag | u(1) |
| } |  |
| for( i = 1; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| … |  |

### Slice segment header semantics

poc_reset_flag equal to 1 specifies that the derived picture order count for the current picture is equal to 0. poc_reset_flag equal to 0 specifies that the derived picture order count for the current picture may or may not be equal to 0. It is a requirement of bitstream conformance that when cross_layer_irap_aligned_flag is equal to 1, the value of poc_reset_flag shall be equal to 0. When not present, the value of poc_reset_flag is inferred to be equal to 0.

poc_msb_reset_flag equal to 1 specifies that the MSB value of the derived picture order count for the current picture is equal to 0. poc_msb_reset_flag equal to 0 specifies that the MSB value of the derived picture order count for the current picture may or may not be equal to 0.

When the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1 and at least one picture in the current access unit is an IRAP picture with NoRaslOutputFlag equal to 1, poc_msb_reset_flag shall be present and the value shall be equal to 1.

When not present, the value of poc_msb_reset_flag is inferred to be equal to 0.

poc_lsb_reset_flag equal to 1 specifies that the derived picture order count for the current picture is equal to 0. poc_lsb_reset_flag equal to 0 specifies that the derived picture order count for the current picture may or may not be equal to 0.

When the current picture is not an IDR picture or slice_pic_order_cnt_lsb is not equal to 0, and the picture with nuh_layer_id equal to 0 in the current access unit is an IDR picture, poc_lsb_reset_flag shall be present and the value shall be equal to 1.

When not present, the value of poc_lsb_reset_flag is inferred to be equal to 0.

When the value of poc_msb_reset_flag is equal to 0, the value of poc_lsb_reset_flag shall be equal to 0.

It is a requirement of bitstream conformance that, when there is an IRAP picture with NoRaslOutputFlag equal to 1 in the current access unit, the current picture shall have discardable_flag equal to 0 and TemporalId greater than 0, and shall not be a sub-layer non-reference picture, a RASL picture, or a RADL picture.

### Changes to the decoding process of picture order count

- F.8.3.1	Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Picture order counts are used to identify pictures, for deriving motion parameters in merge mode and motion vector prediction, and for decoder conformance checking (see subclause C.5).

Each coded picture is associated with a picture order count variable, denoted as PicOrderCntVal.

- When the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1, the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows:

- Let prevTid0Pic be the previous picture in decoding order that has TemporalId equal to 0 and nuh_layer_id equal to nuh_layer_id of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture, and let prevPicOrderCnt be equal to PicOrderCntVal of prevTid0Pic.

- The variable prevPicOrderCntLsb is set equal to prevPicOrderCnt & ( MaxPicOrderCntLsb − 1 ).

- The variable prevPicOrderCntMsb is set equal to prevPicOrderCnt − prevPicOrderCntLsb.

- The variable PicOrderCntMsb of the current picture is derived as follows:

- If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, PicOrderCntMsb is set equal to 0.

- Otherwise, PicOrderCntMsb is derived as follows:

if( ( slice_pic_order_cnt_lsb < prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − slice_pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(Error: Reference source not found-1)
else if( (slice_pic_order_cnt_lsb > prevPicOrderCntLsb )  &&
		( (slice_pic_order_cnt_lsb − prevPicOrderCntLsb ) > ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as follows:

PicOrderCntVal = ( poc_msb_reset_flag ? 0 : PicOrderCntMsb ) +
						( poc_lsb_reset_flag ? 0: slice_pic_order_cnt_lsb )				(Error: Reference source not found-2)

NOTE 1 – All IDR pictures that have nuh_layer_id equal to 0 will have PicOrderCntVal equal to 0 since slice_pic_order_cnt_lsb is inferred to be 0 for IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

When poc_reset_flag is equal to 1, the following steps apply in the order listed:

- The PicOrderCntVal of each picture that is in the DPB and belongs to the same layer as the current picture is decremented by PicOrderCntVal.

- PicOrderCntVal is set equal to 0.

When poc_msb_reset_flag is equal to 1, the PicOrderCntVal of each picture that is in the DPB and belongs to the same layer as the current picture is decremented by PicOrderCntMsb.

When poc_lsb_reset_flag is equal to 1, the PicOrderCntVal of each picture that is in the DPB and belongs to the same layer as the current picture is decremented by slice_pic_order_cnt_lsb.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive. In one CVS, the PicOrderCntVal values for any two coded pictures in the same layer shall not be the same.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCntVal of the picture picX	(Error: Reference source not found-3)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(Error: Reference source not found-4)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that are not in the range of −215 to 215 − 1, inclusive.

NOTE 2 – Let X be the current picture and Y and Z be two other pictures in the same sequence, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

### Analysis of the proposed solution

The motivation for POC LSB and POC MSB reset is given with two examples. The HEVC, SHVC and MV-HEVC working drafts have a restriction that the picture order count of all the coded pictures in an access unit must be equal. Consider the two examples given below, where there are non-aligned IRAP AUs. In both the examples with two layers, the LSB of the POC is represented with seven bits. In Figure 2, AU 241 consists of an IDR picture in the enhancement layer (EL) and a trailing picture in the base layer (BL). The table shows the value of the POC LSB and MSB derived when no reset is performed. The derived values of the POC for the BL and EL would not be equal in AUs that succeed AU 240 in decoding order. A POC MSB reset at AU 241 (in particular, in the BL picture of AU 241) would result in correct derivation of the POC values.

Figure 2: POC MSB reset when an AU contains a non-IRAP BL picture and an IRAP EL picture with NoRaslOutputFlag equal to 1. The numbers in red indicate POC mismatch within an AU. The numbers in green indicate that the POC values would be decremented when POC MSB are reset at AU 241.

In Figure 3, AU 241 contains an IDR picture at the BL and a trailing picture at the EL. Due to the restriction in HEVC WD that the IDR picture in the BL must have a POC value equal to 0 and the requirement of cross-layer POC alignment in the latest SHVC and MV-HEVC WDs, the value of the derived POC of the EL picture at AU 241 should be 0. However, the value of the derived POC values for the pictures in the BL and EL pictures would not match if there is no reset, or when only the MSB is reset. Only when both the MSB and LSB are reset for the EL picture at AU 241, the POC values of the EL and the BL pictures would match.

Figure 3: POC MSB and POC LSB reset when an AU contains a non-IRAP EL picture and an IDR BL. The numbers in red indicate POC mismatch within an AU. The numbers in green indicate that the POC values would be decremented when POC LSB and MSB are reset at AU 241. Note that signalled LSB for EL picture at AU 241 would still be 113

## POC reset information SEI message

### General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| ... |  |
| else if( payloadType = = XXX ) |  |
| three_dimensional_reference_displays_info( payloadSize ) |  |
| else if( payloadType = = XXX ) |  |
| poc_reset_info( payloadSize ) |  |
| ... |  |
| Else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| ... |  |
| Else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

### POC reset information SEI message syntax

| poc_reset_info( payloadSize ) { | Descriptor |
| --- | --- |
| poc_reset_type_flag | u(1) |
| poc_reset_value | u(32) |
| poc_reset_id | u(7) |
| } |  |

### POC reset information SEI message semantics

The POC reset information SEI message provides information that enables correct POC derivation for the associated picture even when the previous picture in decoding order in the same layer as the associated picture and that has poc_lsb_reset_flag or poc_msb_reset_flag equal to 1 is lost. The information contained in the SEI message can also be used to derive the POC values of other pictures in the DPB that are in the same layer as the associated picture.

POC-reset picture is defined as a picture that has the value of either poc_msb_reset_flag or poc_lsb_reset_flag, or both, equal to 1. The associated POC-reset picture refers to the previous picture in decoding order in the same layer as the associated picture and that has poc_lsb_reset_flag or poc_msb_reset_flag equal to 1.

The associated picture of a POC reset information SEI message refers to the picture that is associated with the SEI message.

A non-nested POC reset information SEI message is associated with the picture for which the first VCL NAL unit in decoding order is the associated VCL NAL unit of the SEI NAL unit containing the non-nested POC reset information SEI message. The nuh_layer_id of the SEI NAL unit containing a non-nested POC reset information SEI message shall be equal to the nuh_layer_id of the associated picture.

A non-nested POC reset information SEI message is associated with the first picture picA that has the same value of nuh_layer_id as the SEI NAL unit, that succeeds the SEI message in decoding order, and that is contained in the access unit containing a poc_reset_info_cancel( ) SEI message . The value of nuh_layer_id of the SEI NAL unit containing the poc_reset_info_cancel( ) SEI message shall be equal to the nuh_layer_id of the associated picture.

poc_reset_type_flag equal to 1 indicates that the POC MSB was reset and POC LSB was not reset for the associated POC-reset picture. poc_reset_type_flag equal to 0 specifies that both the POC MSB and POC LSB were reset for the associated POC-reset picture.

poc_reset_value indicates the POC value of the associated POC-reset picture before POC resetting is applied (i.e. the derived POC value assuming both poc_msb_reset_flag and poc_lsb_reset_flag are equal to 0).

NOTE − When the associated POC-reset picture is lost, this value can also be used to derive the POC values of the associated picture and other decoded pictures of the same layer in the DPB, as follows. When the value of poc_reset_type_flag is equal to 0, the POC of the associated picture can be derived by setting prevPicOrderCntLsb equal to poc_reset_value % MaxPicOrderCntLsb, and prevPicOrderCntMsb equal to 0, and following the rest of the decoding process for the picture order count, and the value of PicOrderCntVal of all the pictures in the DPB that belong to the same layer as the associated picture are decremented by poc_reset_value - poc_reset_value % MaxPicOrderCntLsb. When the value of poc_reset_type_flag is equal to 1, the POC of the associated picture can be derived by setting prevPicOrderCntLsb and prevPicOrderCntMsb both equal to 0, and following the rest of the decoding process for the picture order count, and the value of PicOrderCntVal of all the pictures in the DPB that belong to the same layer as the associated picture are decremented by poc_reset_value.

poc_reset_id specifies an identifier of a POC-reset picture in the same layer as the associated picture.

No two consecutive POC-reset pictures of a particular layer in the bitstream shall have the same value of poc_reset_id.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).