| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0214r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0074r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On parameter sets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ye-Kui Wang Ying Chen Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-3481 vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document provides an analysis of VPS size, discusses a few parameter set related issues, and proposes solutions for the issues. These issues include: 1) repetition of parameter sets within one AU, 2) parallel extensions for SPS, 3) scaling matrix prediction, 4) bitstream conformance restrictions on parameter sets that are never activated, 5) repetition of same SPS VUI in SPSs, and 6) signaling of MV-HEVC SPS extension syntax elements.

Revision 1 of this document contains a combined solution for the fifth proposal aspect on the repetition of same SPS VUIs in SPSs, integrating aspects of JCTVC-O0118, after initial review of the two documents.

# Introduction

## VPS size analysis

In the attached Excel sheets, we present an analysis of VPS size, including the break-down to different syntax structures included in the VPS.

Some observations from this analysis:

- In some typical scenarios, the bits for the profile_tier_level syntax structure are about half of the entire VPS size.

- Typical sizes of VPS with VPS extension are about 2000 bits, and with VPS extension are about 3000 bits.

## Repetition of parameter sets within one AU

### Problem

In HEVC, parameter sets such as VPS and SPS and are allowed to be repeated within a CVS as long as the parameters of a particular type and a particular value of ID have the same contents. Repetition of parameter sets in the bitstream provides error resilience to the loss of these parameter sets. This behaviour is also extended to SHVC and MV-HEVC.

In SHVC and MV-HEVC, two or more layers may share the same SPS as their active layer SPS. In this case, there may be situations where the bitstream may repeat the SPS in the bitstream, and even within an access unit. For example, in the first access unit in the bitstream that contains two layers, the first occurrence of an SPS may be before all the VCL NAL in the AU, and the second occurrence in the same AU may be after all the VCL NAL units that have nuh_layer_id equal to 0, but before all the VCL NAL units that have nuh_layer_id equal to 1. Figure 1 shows a typical example of such a bitstream. The SPS with seq_parameter_set_id equal to 0 and nuh_layer_id equl to 0 is sent twice in the first access unit.

Figure 1: Bitstream with two layers that share the same SPS that has seq_parameter_set_id equal to 0. The SPS is repeated in the access unit.

When only the base layer is extracted from this bitstream, and AU 1 has an AU delimiter, then the resultant bitstream would be non-conforming because an empty access unit (i.e. no VCL NAL units in that AU) that contains only an SPS NAL unit would follow AU 0. The extracted bitstream is shown in Figure. 2. Such a bitstream with an empty NAL unit is a non-conforming bitstream.

Figure 2: Only the base layer is extracted from the bitstream in Figure 1. The second AU is empty, i.e. no VCL NAL units in the second AU.

### Solution

Although SPS repetition is permitted, MV-HEVC/SHVC specify that the SPS NAL units that contain same value of seq_parameter_set_id in the same CVS must have the same content. If this condition is relaxed, and only SPS RBSP of two or more SPS NAL units that contain the same value of seq_parameter_set_id are constrained to be the same, the SPS NAL unit can be repeated but with different values of nuh_layer_id. In the above example in Figure 1, the second SPS in AU 0 will have nuh_layer_id equal to 1, and thus would be removed when only the base layer is extracted. Thus, the situation such as in Figure 2 would not occur. The following change to the Specification is proposed (additions highlighted in grey) in Sec. F.7.4.2.4.2.

- Any SPS NAL unit containing the value of sps_seq_parameter_set_id for the active layer SPS RBSP shall have the same content of SPS RBSP as that of the active layer SPS RBSP unless it follows the last coded picture for which the active layer SPS is required to be active and precedes the first NAL unit activating a SPS of the same value of seq_parameter_set_id.

## Parallel extensions for SPS

### Background

Range extension of HEVC will be the amendment 1 of HEVC version 1. MV-HEVC will be the amendment 2 of HEVC version 1 and SHVC may be the amendment 3 of HEVC version 1.

So MV-HEVC text will be built eventually on top of range extension specification.

In MV-HEVC, it is highly possible that it is not necessarily requires the features in range extension to be on, so the syntax elements in range extension might be unnecessary.

SHVC is an extension of MV-HEVC from the syntax structure point of view, so it could be assumed that the syntax elements in MV-HEVC are still necessary for SHVC.

To convey additional information in HEVC extensions, an SPS extension can be present in an extension specification.

In range extension the syntax table in SPS is as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| … |  |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension1_flag | u(1) |
| if( sps_extension1_flag ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| residual_dpcm_intra_enabled_flag | u(1) |
| residual_dpcm_inter_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| sps_extension2_flag | u(1) |
| } |  |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Editorially, an SPS range extension is considered as present as follows:

| sps_range_extension () { |  |
| --- | --- |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| residual_dpcm_intra_enabled_flag | u(1) |
| residual_dpcm_inter_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| sps_extension2_flag | u(1) |
| } |  |

And the SPS syntax is as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| … |  |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension1_flag | u(1) |
| if( sps_extension1_flag ) { |  |
| sps_range_extension () |  |
| } |  |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

In MV-HEVC, there is also an SPS extension, as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| … |  |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### Problem

Both of the range extension and MV-HEVC extension utilize the same sps_extension_flag to indicate the presence of the SPS extension. Regardless of which SPS extension should be put as a higher priority (most likely range extension), in a fashion as what we did in AVC extensions, sps_extension2_flag, is used to contain the other SPS extension within the first extension, the following problem exists.

- Assume MV-HEVC is the extension of range extension, the SPS range extension must be present for MV-HEVC bistreams, regardless of whether the MV-HEVC bitstream is built on top of the range extension features. This does not make sense, as MV-HEVC can be, actually is more likely to be, built on 'plain' HEVC without range extensions. Therefore the presence of the SPS range extension bits in an MV-HEVC bitstream would be purely a waste.

### Solution

We propose to use the flag sps_extension_flag simply to indicates the presence of any SPS extension, however, the existence of parallel extensions are controlled by different flags separately. The detailed syntax design is as follows.

#### Sequence parameter set syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| … |  |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

sps_extension1_flag equal to 1 specifies extension to this parameter set is present, as in sps_extension( ). sps_extension1_flag equal to 0 specifies extension to this parameter set is not present.

#### SPS extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| range_ext_flag | u(1) |
| multi_layer_ext_flag | u(1) |
| if( range_ext_flag ) |  |
| sps_range_extension( ) |  |
| if( multi_layer_ext_flag ) |  |
| sps_mv_extension( ) |  |
| sps_futher_extension_flag | u(1) |
| if( sps_further_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_further_extension_data_flag | u(1) |
| } |  |

#### SPS extension syntax

range_ext_flag equal to 1 specifies that sps_range_extension( ) is present. range_ext_flag equal to 0 specifies that sps_range_extension( ) is not present.

multi_layer_ext_flag equal to 1 specifies that sps_mv_extension( ) is present. range_ext_flag equal to 0 specifies that sps_mv_extension( ) is not present.

sps_futher_extension_flag equal to 0 specifies that no sps_further_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. When present, sps_further_extension_data_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. When not present, the value of sps_further_extension_data_flag is inferred to be equal to 0. The value of 1 for sps_further_extension_data_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of sps_further_extension_data_flag to be equal to 1 (when present) and shall ignore all sps_further_extension_data_flag syntax elements that follow the value 1 for sps_futher_extension_flag in an SPS NAL unit.

sps_further_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all sps_further_extension_data_flag syntax elements.

#### SPS range extension syntax

| sps_range_extension () { |  |
| --- | --- |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| residual_dpcm_intra_enabled_flag | u(1) |
| residual_dpcm_inter_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| sps_range_extension2_flag | u(1) |
| if(sps_range_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_range_extension_data_flag | u(1) |
| } |  |

#### SPS range extension semantics

sps_range_extension2_flag equal to 0 specifies that no sps_range_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. When present, sps_range_extension_data_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. When not present, the value of sps_range_extension_data_flag is inferred to be equal to 0. The value of 1 for sps_range_extension_data_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of sps_range_extension_data_flag to be equal to 1 (when present) and shall ignore all sps_range_extension_data_flag syntax elements that follow the value 1 for sps_range_extension2_flag in an SPS NAL unit.

sps_range_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all sps_range_extension_data_flag syntax elements.

#### SPS multiview extension syntax

| sps_mv_extension () { |  |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_shvc_reserved_zero_idc | ue(v) |
| sps_mv_extension2_flag | u(1) |
| if(sps_mv_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_mv_extension_data_flag | u(1) |
| } |  |

sps_mv_extension2_flag equal to 0 specifies that no sps_mv_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. When present, sps_mv_extension_data_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. When not present, the value of sps_mv_extension_data_flag is inferred to be equal to 0. The value of 1 for sps_mv_extension_data_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of sps_mv_extension_data_flag to be equal to 1 (when present) and shall ignore all sps_mv_extension_data_flag syntax elements that follow the value 1 for sps_mv_extension2_flag in an SPS NAL unit.

sps_mv_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all sps_mv_extension_data_flag syntax elements.

## Scaling matrix prediction

### Problem

The current SHVC and MV-HEVC specifications allow the inference of scaling list information for an SPS from a reference SPS, and for a PPS from a reference PPS. The semantics of the syntax element that is used to specify the reference SPS is given below:

sps_scaling_list_ref_layer_id specifies the value of the nuh_layer_id of the layer for which the active SPS is associated with the same scaling list data as the current SPS.

The value of sps_scaling_list_ref_layer_id shall be in the range of 0 to 62, inclusive.

When avc_base_layer_flag is equal to 1, it is a requirement of bitstream conformance that the value of sps_scaling_list_ref_layer_id shall be greater than 0.

It is a requirement of bitstream conformance that, when an SPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB and sps_infer_scaling_list_flag in the SPS is equal to 1, sps_infer_scaling_list_flag shall be equal to 0 for the SPS that is active for the layer with nuh_layer_id equal to sps_scaling_list_ref_layer_id.

It is a requirement of bitstream conformance that, when an SPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB, the layer with nuh_layer_id equal to sps_scaling_list_ref_layer_id shall be a direct or indirect reference layer of the layer with nuh_layer_id equal to nuhLayerIdB.

The reference SPS that is used for scaling list inference is the active layer SPS of another layer that is specified by sps_scaling_list_ref_layer_id. In other words, in order that the scaling list data for a particular SPS be inferred using this syntax element, the SPS of the reference layer must already be activated. Such an inference may not be error resilient because the reference SPS may not activated for that layer due to some losses, and the inference is either not possible (because there is no active reference SPS to infer from) or the SPS may infer the previously active layer SPS of the layer with nuh_layer_id sps_scaling_list_ref_layer_id. This could potentially affect the decoding of many pictures, and result in incorrect decoding.

The above discussion is applicable for any form of inference or prediction that happens between SPS (not restricted to scaling prediction, although that is the only prediction or inference that is allowed between SPSs as per the current Specification), or any two parameter sets of the same type.

### Solution

It would be much more robust to have hierarchical prediction of parameter sets, such as VPS to SPS, SPS to PPS, and PPS to slice header and so on.

## Bitstream conformance restrictions on parameter sets that are never activated

### Problem

In current SHVC WD, there are bitstream conformance constraints specified for active parameter sets PPS or SPS, e.g. as highlighted with green below.

pps_scaling_list_ref_layer_id specifies the value of the nuh_layer_id of the layer for which the active PPS is associated with the same scaling list data as the current PPS.

The value of pps_scaling_list_ref_layer_id shall be in the range of 0 to 62, inclusive.

When avc_base_layer_flag is equal to 1, it is a requirement of bitstream conformance that pps_scaling_list_ref_layer_id shall be greater than 0.

It is a requirement of bitstream conformance that, when a PPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB and pps_infer_scaling_list_flag in the PPS is equal to 1, pps_infer_scaling_list_flag shall be equal to 0 for the PPS that is active for the layer with nuh_layer_id equal to pps_scaling_list_ref_layer_id.

It is a requirement of bitstream conformance that, when a PPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB, the layer with nuh_layer_id equal to pps_scaling_list_ref_layer_id shall be a direct or indirect reference layer of the layer with nuh_layer_id equal to nuhLayerIdB.

In addition, HEVC version 1 has a constraint on never-activated parameter sets as follows:

All constraints that are expressed on the relationship between the values of the syntax elements and the values of variables derived from those syntax elements in VPSs, SPSs, and PPSs and other syntax elements are expressions of constraints that apply only to the active VPS, the active SPS, and the active PPS. If any VPS RBSP, SPS RBSP, and PPS RBSP is present that is never activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it was activated by reference in an otherwise conforming bitstream.

However, for parameter sets that are never activated, it is hard to check whether the above green-highlighted constraints are satisfied for parameter sets that are in a bitstream but never activated, even considering the above yellow-highlighted constraint. This is because if a parameter set can pass the constaints for some particular layer, it may not be activated for another layer. Multiple parameter sets can be signalled in a bitstream for a certain layer, so it cannot be concluded about the conformance unless this parameter set is activated for a particular layer.

### Solution

To overcome the issues with the bitstream conformance for never-activated parameter sets, the above yellow-highlighted constraint in HEVC version 1 is suggested to be modified as follows:

If any VPS RBSP, SPS RBSP, and PPS RBSP is present that is never activated in the bitstream, syntax elements of that particular parameter set RBSP shall have values that would conform to those specified constraints that are not dependent on the value of any syntax element of another parameter set RBSP.

## On VPS VUI and SPS VUI

### Problem

SPS VUI includes information such as sample aspect ratio, over scan, source video format (PAL/NTSC etc., sample value range, source color format), field/frame information, bitstream restrictions (including cross-layer bitstream restrictions). Most of such information in most cases, including cross-layer bitstream restrictions, is really not layer specific and is the same for all layers. Thus it is just awkward to not have such VUI information signalled in the VPS that naturally applies to all layers.

### Solution

We propose to include all SPS VUI information in VPS VUI, and if for a particular layer any VUI parameter for an SPS must have a different value than that in the VPS VUI, the entire SPS VUI is directly signalled in the SPS, otherwise, the SPS does not contain VUI parameters and the VUI is inferred from the VPS.

The proposed syntax and semantics changes are as follows, with additions highlighted in yellow.

| vps_vui( ){ | Descriptor |
| --- | --- |
| ... |  |
| sps_vui_in_vps_flag | u(1) |
| if( sps_vui_in_vps_flag ) |  |
| vui_parameters( ) |  |
| } |  |

sps_vui_in_vps_flag equal to 1 specifies that the vui_parameters( ) syntax structure is present in the VPS and applies to all SPSs that refer to the VPS and have sps_vui_from_vps_flag equal to 1. sps_vui_in_vps_flag equal to 0 specifies that the vui_parameters( ) syntax structure is not present in the VPS. When vps_vui_present_flag is equal to 0, the value of sps_vui_in_vps_flag is inferred to be equal to 0.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nuh_layer_id > 0 ) |  |
| sps_vui_from_vps_flag | u(1) |
| if( !sps_vui_from_vps_flag ) |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

sps_vui_from_vps_flag equal to 1 specifies that the vui_parameters( ) syntax structure applicable to the SPS is inferred to be identical to that in the VPS the SPS refers to. sps_vui_from_vps_flag equal to 0 specifies that the vui_parameters( ) syntax structure applicable to the SPS is not inferred from the VPS the SPS refers to. When not present, the value of sps_vui_from_vps_flag is inferred to be equal to 0. When sps_vui_in_vps_flag is equal to 0, the value of sps_vui_from_vps_flag shall be equal to 0.

vui_parameters_present_flag equal to 1 specifies that the vui_parameters( ) syntax structure as specified in Annex Error: Reference source not found is present. vui_parameters_present_flag equal to 0 specifies that the vui_parameters( ) syntax structure as specified in Annex Error: Reference source not found is not present. When not present, the value of vui_parameters_present_flag is inferred to be equal to 0.

## Signaling of MV-HEVC SPS extension syntax elements

In MV-HEVC, a motion vector constraint flag (inter_view_mv_vert_constraint_flag) is included in the SPS extension, shown as follows:

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_shvc_reserved_zero_idc | ue(v) |
| } |  |

Consider this is the only flag that is relevant to MV-HEVC in the sequence parameter set extension for MV-HEVC and such a syntax element can be considered as operation point level information, it is proposed to move this flag to video parameter set.

The sps_shvc_reserved_zero_idc is actually only relevant in SHVC and can be sufficient to be removed from MV-HEVC specification.

With the above changes, the SPS extension of MV-HEVC doesn’t need to be defined, so it is the editorially much cleaner while providing the same functionality.

The syntax change in the VPS extension is shown as follows:

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| inter_view_mv_vert_constraint_flag | u(1) |
| } |  |

# Combined solution for signaling SPS VUI in the VPS

This section contains a solution to signal the SPS VUI in the VPS, integrating aspects from Section 1.6 of this document and aspects from JCTVC-O0118.

## VPS VUI syntax

| vps_vui( ){ | Descriptor |
| --- | --- |
| ... |  |
| sps_vui_in_vps_flag | u(1) |
| if( sps_vui_in_vps_flag ) |  |
| vui_parameters( ) |  |
| video_signal_info_idx_present_flag | u(1) |
| if( video_signal_info_idx_present_flag ) |  |
| vps_num_video_signal_info_minus1 | u(4) |
| for( i = 0; i <= vps_num_video_signal_info_minus1; i++ ) |  |
| video_signal_info( ) |  |
| if( video_signal_info_idx_present_flag ) { |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| vps_video_signal_info_idx[ i ] | u(4) |
| } |  |
| … |  |
| } |  |

## VPS VUI semantics

sps_vui_in_vps_flag equal to 1 specifies that the vui_parameters( ) syntax structure is present in the VPS and applies to all SPSs that refer to the VPS and have sps_vui_from_vps_flag equal to 1. sps_vui_in_vps_flag equal to 0 specifies that the vui_parameters( ) syntax structure is not present in the VPS. When vps_vui_present_flag is equal to 0, the value of sps_vui_in_vps_flag is inferred to be equal to 0.

video_signal_info_idx_present_flag equal to 1 indicates that the syntax elements vps_num_video_signal_info_minus1, and vps_video_signal_info_idx[ i ] are present. video_signal_info_idx_present_flag equal to 0 indicates that the syntax elements vps_num_video_signal_info_minus1, and vps_video_signal_info_idx[ i ] are not present.

vps_num_video_signal_info_minus1 plus 1 specifies the number of the following video_signal_info( ) syntax structures in the VPS. When not present, the value of vps_num_video_signal_info_minus1 is inferred to be equal to vps_max_layers_minus1.

vps_video_signal_info_idx[ i ] specifies the index, into the list of video_signal_info( ) syntax structures in the VPS, of the video_signal_info( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. When video_signal_info_idx_present_flag is equal to 0 vps_video_signal_info_idx[ i ] is inferred to be equal to i for each of i=0,…,vps_max_layers_minus1. When i is equal to 0 or vps_num_video_signal_info_minus1 is equal to 0, the value of vps_video_signal_info_idx[ i ] is inferred to be equal to 0. The value of vps_video_signal_info_idx[ i ] shall be in the range of 0 to vps_num_video_signal_info_minus1, inclusive.

## Video signal information syntax and semantics

| video_signal_info( ) { | Descriptor |
| --- | --- |
| video_vps_format | u(3) |
| video_full_range_vps_flag | u(1) |
| colour_primaries_vps | u(8) |
| transfer_characteristics_vps | u(8) |
| matrix_coeffs_vps | u(8) |
| } |  |

video_vps_format, video_full_range_vps_flag, colour_primaries_vps, transfer_characteristics_vps, matrix_coeffs_vps  are used for inference of the values of the SPS VUI syntax elements video_format, video_full_range_flag, colour_primaries, transfer_characteristics, matrix_coeffs respectively, for each SPS that refers to the VPS.

For each of these syntax elements, all constraints, if any, that apply to the value of the corresponding SPS VUI syntax element also apply.

## Sequence parameter set extension syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nuh_layer_id > 0 ) |  |
| sps_vui_from_vps_flag | u(1) |
| if( !sps_vui_from_vps_flag ) |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

## Sequence parameter set extension semantics

sps_vui_from_vps_flag equal to 1 specifies that the vui_parameters( ) syntax structure applicable to the SPS is inferred to be identical to that in the VPS the SPS refers to for the syntax elements. sps_vui_from_vps_flag equal to 0 specifies that the vui_parameters( ) syntax structure applicable to the SPS is not inferred from the VPS the SPS refers to. When not present, the value of sps_vui_from_vps_flag is inferred to be equal to 0. When sps_vui_in_vps_flag is equal to 0, the value of sps_vui_from_vps_flag shall be equal to 0.

vui_parameters_present_flag equal to 1 specifies that the vui_parameters( ) syntax structure as specified in Annex Error: Reference source not found is present. vui_parameters_present_flag equal to 0 specifies that the vui_parameters( ) syntax structure as specified in Annex Error: Reference source not found is not present. When not present, the value of vui_parameters_present_flag is inferred to be equal to 0.

## VUI semantics

video_signal_type_present_flag equal to 1 specifies that video_format, video_full_range_flag and colour_description_present_flag are present. video_signal_type_present_flag equal to 0, specify that video_format, video_full_range_flag and colour_description_present_flag are not present.

It is a requirement of bitstream conformance that when nuh_layer_id is greater than 0, video_signal_type_present_flag shall be equal to 0.

It is a requirement of bitstream conformance that when the vui_parameters( ) syntax structure is signaled in the VPS VUI, the value of video_signal_type_present_flag shall be equal to 0.

When a current picture with nuh_layer_id layerIdCurr greater than 0 refers to an SPS, the values of video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs are inferred as follows:

- If the nuh_layer_id of the active layer SPS is equal to 0, the values of video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs are inferred to be equal to video_vps_format, video_full_range_vps_flag, colour_primaries_vps, transfer_characteristics_vps, and matrix_coeffs_vps, respectively, of the vps_video_signal_info_idx[ j ]-th video_signal_info( ) syntax structure in the active VPS where j is equal to LayerIdxInVps[ layerIdCurr ] and the values of video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs of the active layer SPS are ignored.

NOTE X –  The values are inferred from the VPS when an active non-base layer references an SPS which is also used by the base layer, in which case the SPS has nuh_layer_id equal to 0. For an active base layer, the values in the active SPS apply.

- Otherwise (the nuh_layer_id of the active layer SPS is greater than zero), the following applies:

  - The values of video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs are inferred to be equal to video_vps_format, video_full_range_vps_flag, colour_primaries_vps, transfer_characteristics_vps, and matrix_coeffs_vps, respectively, of the vps_video_signal_info_idx[ j ]-th video_signal_info( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).