| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0217 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0075 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Sub-DPB based DPB operations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ye-Kui Wang Ying Chen Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA Miska M. Hannuksela Visiokatu 3, 33720 Tampere, Finland Sachin Deshpande 5750 Pacific Rim Blvd Camas, WA 98607, USA. | Tel: Email: | 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com +358-7180-08000 miska.hannuksela@nokia.com +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Qualcomm Incorporated, Nokia, and Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution describes sub-DPB based operation for the decoded picture buffer (DPB). The DPB is partitioned into several sub-DPBs, and each sub-DPB is managed independently. The signalling scheme in the attached document can be applied to both modes of sub-DPB operation – layer-specific DPB and resolution-specific DPB. In the layer-specific DPB mode, each layer has its own sub-DPB, while in the resolution-specific DPB mode, all layers that have the same spatial resolution, bit depth, and color format share the same sub-DPB.

# Introduction

The current specification of DPB operations in both the SHVC and MV-HEVC are incomplete, and unclear in many places. The current design of the DPB and the other aspects of the Specification have the following shortcomings:

- There is no distinction among the picture storage buffers in the DPB in terms of the size of each picture storage buffer. For example, different pictures could have different sizes due to differences in resolution, bit depth and chroma format.

- The number of picture storage buffers depends not only on the layer set, but also on the output layer set. Therefore, the signalling of the DPB size-related parameters should be made for each output layer set.

- The information about the output layer set is only present in the VPS, and to prevent the parsing dependency of syntax elements across parameter sets, the DPB size information and the information about number of reorder picture and the maximum latency should be also signalled in the VPS (or in the VPS extension, to be more precise).

- The operation point of the decoder is input to the decoder in terms of a set of layers to be decoded (layer set) and a temporal ID value. Multiple output layer sets could be associated with the same layer set, and thus it is not clear what layers are to be output by the decoder. Instead, the input operation point should be an output layer set instead of a layer set.

This contribution presents a sub-DPB based operation of DPB. Two modes of sub-DPB operations are presented: one involves layer-specific sub-DPB operation, and the other involves resolution/chroma format/bit depth-specific sub-DPB operation (referred to, henceforth, as resolution–specific mode). Pros and cons of each mode of operation are discussed. We request the group to discuss and choose one of the modes of operation that is applied for both SHVC and MV-HEVC. This contribution also includes the definition of the target output layers, and also the specification of the operation point with a set of target output layer and a highest temporal ID value.

It should be noted that this document is specific to the operation of DPB only, and that it is independent of, although not conflicting with, the so-called layer-specific CPB model, where the CPB operations for a layer itself, not including the layers it depends on, are specified.

# Proposal

## Operation points and target output layers

An SHVC/MV-HEVC bitstream contains two or more layers, and in most cases more than one operation point. A sub-bitstream is generated from an existing bitstream based on the various operation points that are supported. In the current Specification, operation point is specified using a layer set (set of pictures to be decoded) and a highest temporal ID value. This has two shortcomings: it is unclear what pictures should be output. Several candidate output layer sets are signalled in the VPS. However, the decoder does not have a means to decide what pictures to output. Since a layer set may correspond to one or more output layer sets (and hence one or more operation points), there needs to be additional information to help the decoder determine the set of target output layers.

It is proposed that an operation point be associated with a set of target output layers and a temporal ID. A modification in the definition for operation points is proposed, along with a new definition for output layer sets.

- operation point: A bitstream that is created from another bitstream by operation of the sub-bitstream extraction process with the another bitstream, a target highest TemporalId, and a target layer identifier list as inputs, and that is associated with a set of target output layers.

- NOTE 14 – If the target highest TemporalId of an operation point is equal to the greatest value of TemporalId in the layer set associated with the target layer identification list, the operation point is identical to the layer set. Otherwise it is a subset of the layer set.

- output layer set: A layer set that is associated with a set of target output layers.

The set of target output layers corresponds to an output layer set. Hence, it is also proposed that the specification of the operation point to the decoder be done by an output layer set, and a temporal ID value. The decoding process in sub-clause 8.1 is changed to reflect this.

## Sub-DPB based operation

The DPB is logically partitioned into sub-DPBs and each sub-DPB contains picture storage buffers. Each sub-DPB is associated with a layer (in the layer-specific mode) or all layers of a particular combination of resolution, chroma format and bit depth (in the resolution-specific mode), and all pictures in the layer(s) are stored in the associated sub-DPB. The operation of sub-DPBs are independent of each other – in terms of insertion, marking, and removal of decoded pictures as well as the size of each sub-DPB, though the output of decoded pictures from different sub-DPBs are linked through their output times or picture order count values.

### Signalling in VPS extension

The signalling of the number of picture buffers in the sub-DPBs, the number of reorder pictures and the maximum latency, in the VPS is common for both the layer-specific and resolution-specific modes of sub-DPB operations. The number of sub-DPBs in each mode is derived or inferred differently, and the details are presented in later sections.

The syntax and the semantics changes to the VPS extensions are provided in the attachment.

### Layer-specific sub-DPB model

In the layer-specific sub-DPB model, each layer is allocated a sub-DPB, and thus the number of sub-DPBs would be equal to the number of layers in the layer set corresponding to the operation point. The i-th layer in the layer set associated with the specified operation point is associated with the i-th sub-DPB.

### Resolution-specific sub-DPB model

In the resolution-specific sub-DPB model, all the pictures that have the same spatial resolution, colour format and bit depth share the same sub-DPB. The number of sub-DPBs for each output layer set, denoted by the variable NumSubDpbs, is derived. The association of each layer with a sub-DPB is also defined in the process below:

For each layer in each layer set, the assignment of a sub-DPB to each layer is derived in the following process:

–	Set numTargetDecLayerIdList to be equal to numLayersInIdList[ decLayerSetIdx ], where decLayerSetIdx is in the range of 0 to vps_num_layer_sets_minus1, inclusive, and is the index to the list of layer set in the VPS corresponding to the particular layer set. For i from 0 to numTargetDecLayerIdList – 1, inclusive, the arrays spatRes, colourFormat, and bitDepthId are defined as follows:

- spatRes[ i ][ 0 ] and spatRes[ i ][ 1 ] are set equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the active layer SPS of the i-th layer in the target layer set.

- colourFormat[ i ] is set equal to the value of chroma_format_idc[ i ] of the active layer SPS of the i-th layer in the target layer set.

- bitDepthId[ i ][ 0 ] and bitDepthId[ i ][ 1 ] are set equal to the values of bit_depth_luma_minus8 and bit_depth_chroma_minus8, respectively, of the active layer SPS of the i-th layer in the target layer set.

–	The following applies:

subDpbCtr = 0
for( i = 0; i < numTargetDecLayerIdList; i++ )
	SubDpbAssigned[ k ][ i ] = - 1
for( i = 0; i < numTargetDecLayerIdList; i++ ) {
	if( SubDpbAssigned[ k ][ i ] < 0 ) {
		for( j = i + 1; j < numTargetDecLayerIdList; j++ )
			if( SubDpbAssigned[ k ][ j ] < 0 )
				if( ( spatRes[ i ][ 0 ] = = spatRes[ j ][ 0 ] )  
					&&  ( spatRes[ i ][ 1 ] = = spatRes[ j ][ 1 ] )  
					&& ( bitDepthId [ i ][ 0 ] = = bitDepthId [ j ][ 0 ] )  
					&&  (bitDepthId [ i ][ 1 ] = = bitDepthId [ j ][ 1 ] )  
					&& (colourFormat [ i ][ 0 ] = = colourFormat [ j ][ 0 ] ) )

SubDpbAssigned[ k ][ j ] = subDpbCtr
	}
	subDpbCtr ++
}
NumSubDpbsInLayerSet[ decLayerSetIdx ] = subDpbCtr

The variable NumSubDpbs[ i ] for the i-th output layer set is derived as follows:

NumSubDpbs[ i ] = NumSubDpbsInLayerSet[ output_layer_set_idx_minus1[ i ] ]

For each picture with layer ID nuh_layer_id in the decLayerSetIdx-th layer set, the sub-DPB with index SubDpbAssigned[ decLayerSetIdx ][ LayerIdInVps[ nuh_layer_id ] ] is said to be  associated with that picture and also to be associated with that layer.

### Comparison between the two sub-DPB models

#### Pros and cons of the layer-specific sub-DPB model

Pros:

- Specification of the layer-specific mode is simple.

Cons:

- Pictures across layers would not be able to share the sub-DPB space, and this could be inefficient use of the DPB memory.

- The design is not aligned with MVC and MVC+D.

#### Pros and cons of the resolution-specific sub-DPB model

Pros:

- All pictures with the same resolution are stored in the same sub-DPB, and thus optimal use the DPB is enabled.

- The design is aligned with MVC and MVC+D.

Cons:

- More texts are needed for specifying (either signalling or deriving) the number of sub-DPBs and the association of each layer with a sub-DPB.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).