| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0223 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0078 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Comments on latest MV-HEVC and SHVC draft specs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Ying Chen Adarsh K. Ramasubramonian Hendry Jianle Chen Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-2042 krapaka@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-658-5801 aramasub@qti.qualcomm.com hendry@qti.qualcomm.com 1-858-651-8028 cjianle@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document discusses the following issues in MV-HEVC Draft Text 5 and SHVC Draft Text 3 and proposes corresponding solutions: 1) cross-layer alignment of picture type, 2) unnecessary restrictions on the semantics of poc_reset_flag, 3) POC resetting at an AU that does not have all pictures present, 4) support of larger numbers of views, 5) persistence scope of an active SPS, and 6) coding of direct_dep_type_len_minus2.

# Proposal

## On indication of cross-layer picture type distribution

This section proposes to indicate cross-layer alignment of picture types for SHVC and MV-HEVC. MV-HEVC WD5 and SHVC WD3 do not require cross-layer alignment of the defined picture types across layers. However, it includes a flag cross_layer_irap_aligned_flag to indicate cross-layer alignment of IRAP picture types. It is asserted that, in addition, it is beneficial to indicate cross-layer alignment of leading pictures types, trailing picture types, TSA pictures types, and STSA pictures types.

cross_layer_irap_aligned_flag equal to 1 does not requires cross-layer alignment of leading pictures types, that is, a RASL picture and a RADL picture may be present in the same access unit. Currently, all leading pictures are discardable without affecting the decoding of trailing pictures, and RASL pictures must be discarded when random accessing from the associated IRAP picture. The mix of leading picture types (RASL and RADL) within an AU adds additional burden and require processing differently for each layer. Furthermore, cross-layer alignment of TSA and STSA pictures enables temporal sub-layer up switching capability to be valid across layers and useful information to system entities. Thus, it would useful to indicate the cross-layer alignment of picture types if that is the case.

Therefore, it is proposed to signal one flag to indicate cross-layer alignment of picture types. The proposed changes are as follows:

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| cross_layer_pic_type_aligned_flag | u(1) |
| if( !cross_layer_pic_type_aligned_flag ) |  |
| cross_layer_irap_aligned_flag | u(1) |
| ... |  |
| } |  |

cross_layer_pic_type_aligned_flag equal to 1 specifies that within a CVS that refers to the VPS, all VCL NAL units that belong to an access unit have the same value of nal_unit_type. cross_layer_pic_type_aligned_flag equal to 0 specifies that within a CVS that refers to the VPS, all VCL NAL units in each access unit may or may not have the same value of nal_unit_type.

cross_layer_irap_aligned_flag equal to 1 specifies that IRAP pictures in the CVS are cross-layer aligned, i.e. when a picture pictureA of a layer layerA in an access unit is an IRAP picture, each picture pictureB in the same access unit that belongs to a direct reference layer of layerA or that belongs to a layer for which layerA is a direct reference layer of that layer is an IRAP picture and the VCL NAL units of pictureB have the same value of nal_unit_type as that of pictureA. cross_layer_irap_aligned_flag equal to 0 specifies that the above restriction may or may not apply. When not present, the value of cross_layer_irap_aligned_flag is inferred to be equal to 1.

## Relaxing restriction on poc_reset_flag

MV-HEVC WD5 and SHVC WD3 include both the flags cross_layer_irap_aligned_flag and poc_reset_flag. When cross_layer_irap_aligned_flag is equal to 1, poc_reset_flag is not present and inferred to be equal to 0. Thus, when cross_layer_irap_aligned_flag is equal to 1, the functionality of POC reset is disabled.

However, even when cross_layer_irap_aligned_flag is equal to 1, it is possible that POC reset is still required to make POC values within one AU to be aligned. When cross_layer_irap_aligned_flag is equal to 1, it requires all pictures that directly or indirectly depend on an IRAP picture within the same AU to be IRAP pictures. For those IRAP pictures inside the AU, the POC values may be the same without POC reset. However, for other pictures within the AU which don’t have a dependency relationship (either depending on or being dependent on) with any IRAP picture in the same AU, the POC values may still be different from the those of the IRAP pictures, e.g. due to the fact that the POC MSB values of those pictures are not zero.

Hence it is proposed to remove the restriction for poc_reset_flag related to cross_layer_irap_aligned_flag. Changes to the semantics of the slice segment header and picture parameter set are shown below in red strikethrough.

### General slice segment header semantics

poc_reset_flag equal to 1 specifies that the derived picture order count for the current picture is equal to 0. poc_reset_flag equal to 0 specifies that the derived picture order count for the current picture may or may not be equal to 0. It is a requirement of bitstream conformance that when cross_layer_irap_aligned_flag is equal to 1, the value of poc_reset_flag shall be equal to 0. When not present, the value of poc_reset_flag is inferred to be equal to 0.

### Picture parameter set semantics

num_extra_slice_header_bits specifies the number of extra slice header bits that are present in the slice header RBSP for coded pictures referring to the PPS. num_extra_slice_header_bits shall be in the range of 0 to 2, inclusive, in bitstreams conforming to this version of this Specification. Other values for num_extra_slice_header_bits are reserved for future use by ITU-T | ISO/IEC. However, decoders shall allow num_extra_slice_header_bits to have any value.

When cross_layer_irap_aligned_flag equal to 0, it is a requirement of bitstream conformance that num_extra_slice_header_bits shall be greater than or equal to 1.

## Restrictions on the poc_reset behavior for an AU which does not have all pictures present

MV-HEVC WD5 and SHVC WD3 include the syntax element “poc_reset_flag” in the slice segment header to specify that the derived picture order count for the current picture is equal to 0 and that the picture order count of the pictures that are in the same layer as the current picture and in the DPB are decremented by the picture order count of the current picture before reset. However, when one picture picA has poc_reset_flag equal 1 and another picture picB in the same access unit is not present, the POC value of a picture picC in the layer containing picA may be derived to be different from the POC value of a picture picD in the layer containing picB, where picC and picD are in the same access unit, and picC may or may not be picA (and consequently picD may or may not be picB). This is because poc_reset_flag equal to 1 resets the POC value to be equal to 0. This is undesirable because all pictures in an access unit should have the same POC value.

We define a POC-anchor picture as follows:

POC-anchor picture: A picture that has TemporalId equal to 0 and nuh_layer_id equal to nuh_layer_id of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture.

To overcome the above discussed problem, the following restriction is proposed:

It is a requirement of bitstream conformance that in each access unit that contains at least one picture with poc_reset_flag equal to 1, the pictures of all layers in the bitstream shall be present in that access unit and shall be POC-anchor pictures.

## Support of larger numbers of views

Support for large number of views was proposed in JCT3V-E0092, JCT3V-E0223, and JCT3V-E0224 with the anticipation of future use case wherein huge camera arrays with e.g. 100 or even 1000 views.

While it would be good to enable support for such possible needs in the future, esp. in MV-HEVC (as it contains only up to 2 views), it is sufficient to enable the possibility of the future standard extension based on MV-HEVC or even 3D-HEVC.

If it is desired to enable such future extensibility, we suggest the following:

- Reserve nuh_layer_id 63 in the current MV-HEVC specification. This reserved value can be used as escape value to indicate that the bigger layer id value is needed.

The reserved nuh_layer_id 63 can serve a hook that can be used by future standard for the purpose of supporting large number of views. For example, future standard can specify that when nuh_layer_id is equal to 63, additional two bytes shall be present after nuh_temporal_id_plus1. This additional two bytes can cover up to 65536 views.

## Persistence scope of an active SPS

In MV-HEVC WD5 and SHVC WD3, the activation of an SPS occurs at a layer initialisation picture (LIP) and the SPS must remain active until at least the subsequent LIP picture at the same layer, or the end of the CVS.

The relevant text in the specification related to persistence of SPS is present in F.7.4.2.4.2 is highlighted (in grey) below:

- When an SPS RBSP (with a particular value of sps_seq_parameter_set_id) is not already active for a nuh_layer_id value and it is referred to by activation of a PPS RBSP for that nuh_layer_id value (in which pps_seq_parameter_set_id is equal to the sps_seq_parameter_set_id value), it is activated for that nuh_layer_id value. This SPS RBSP is called the active layer SPS RBSP for that nuh_layer_id value until it is deactivated by the activation of another SPS RBSP for the same layer. An SPS RBSP, with that particular value of sps_seq_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means. An activated SPS RBSP for a particular nuh_layer_id value shall remain active for a sequence of pictures in decoding order with that nuh_layer_id value starting from a LIP picture having that nuh_layer_id value, inclusive, until either the next LIP picture with that nuh_layer_id value, exclusive, or the end of the CVS, whichever is earlier. The nuh_layer_id value the NAL unit containing the SPS RBSP that is activated for nuh_layer_id equal to nuhLayerIdA shall be less than or equal to nuhLayerIdA. The same SPS RBSP may be the active layer SPS for more than one nuh_layer_id value.

The above text allows for an SPS to be activated at each LIP picture. However, when we have an LIP picture in the enhancement layer that is not an IRAP picture, and the base layer picture is an IDR picture with NoClrasOutputFlag equal to 0, then the enhancement layer should not be allowed to activate a new SPS at such a non-IRAP picture.

Therefore, the above text highlighted text is proposed to be modified as follows:

An activated SPS RBSP for a particular nuh_layer_id value shall remain active for a sequence of pictures in decoding order with that nuh_layer_id value starting from a LIP picture having that nuh_layer_id value, inclusive, until either the next LIP picture with that nuh_layer_id value, exclusive, or the end of the CVS, whichever is earlier. as given below:

- If the particular nuh_layer_id value is equal to 0, the activated SPS shall remain active for the entire CVS.

- Otherwise (the particular nuh_layer_id value is not equal to 0), the activated SPS shall remain active starting from an LIP picture that is an IRAP picture having that nuh_layer_id value, or an LIP picture that is not an IRAP picture and that is contained in an IRAP access unit containing an IRAP picture with nuh_layer_id equal to 0 and NoClrasOutputFlag equal to 1, until the next LIP picture that is an IRAP picture having that nuh_layer_id value, or an initial IRAP access unit containing an IRAP picture with nuh_layer_id equal to 0 and NoClrasOutputFlag equal to 1.

## Fixed length coding of direct_dep_type_len_minus2

MV-HEVC WD5 and SHVC WD3 include the syntax element “direct_dep_type_len_minus2” in the VPS extension to specify the number of bits of the direct_dependency_type[ i ][ j ] syntax element.

Due to ue(v) coding of syntax element direct_dep_type_len_minus2, accessing the information of direct_dependency_type, that is important for session negotiation and content selection, needs entropy decoding. For this information to be accessible without entropy decoding, it is proposed to code the syntax element direct_dep_type_len_minus2 as u(3) (fixed-length coded).

Detailed specification changes are provided as follows.

### Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| …….. |  |
| cross_layer_irap_aligned_flag | u(1) |
| direct_dep_type_len_minus2 | u(3) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| ………. |  |
| } |  |

### Video parameter set extension semantics

direct_dep_type_len_minus2 plus 2 specifies the number of bits of the direct_dependency_type[ i ][ j ] syntax element. In bitstreams conforming to this version of this Specification the value of direct_dep_type_len_minus2 shall be equal 0. Although the value of direct_dep_type_len_minus2 shall be equal to 0 in this version of this Specification, decoders shall allow other values of direct_dep_type_len_minus2 in the range of 0 to 730, inclusive, to appear in the syntax.

direct_dependency_type[ i ][ j ] indicates the type of dependency between the layer with nuh_layer_id equal layer_id_in_nuh[ i ] and the layer with nuh_layer_id equal to layer_id_in_nuh[ j ]. direct_dependency_type[ i ][ j ] equal to 0 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for both inter-layer sample prediction and inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 1 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer sample prediction but not for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 2 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer motion prediction but not for inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. Although the value of direct_dependency_type[ i ][ j ] shall be in the range of 0 to 2, inclusive, in this version of this Specification, decoders shall allow values of direct_dependency_type[ i ][ j ] in the range of 3 to 511232 − 2, inclusive, to appear in the syntax. The length of the direct_dependency_types[ i ][ j ] syntax element is direct_dep_type_len_minus2 + 2 bits.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).