| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0224 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0079 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On Signalling of random accessibility for IRAP pictures in non-IRAP AUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Ye-Kui Wang Adarsh K. Ramasubramonian Ying Chen Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-2042 krapaka@qti.qualcomm.com yekuiw@qti.qualcomm.com aramasub@qti.qualcomm.com cheny@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes two aspects related to random accessibility of IRAP pictures in non-IRAP access units. In particular, the first aspect proposes to indicate the presence of IRAP pictures in an access unit and the second aspect proposes to indicate which pictures in previous access units need to decoded in order to random access to the layer of the IRAP picture from the access unit.

# Introduction

MV-HEVC WD5 and SHVC WD3 do not require cross-layer alignment of IRAP pictures to benefit in the scenarios that require 1) Frequent random accessing with high efficiency at low layers than higher layers. 2) Frequent random accessing with high efficiency at higher layers than lower layers. Currently, it is only possible to random access from an initial IRAP access unit (an IRAP access unit in which the coded picture with nuh_layer_id equal to 0 has NoRaslOutputFlag equal to 1). This configuration is typical in the scenario (2) mentioned above with more IRAPs in base layer than enhancement layers. However, currently it is not possible to random access from a non-initial IRAP access unit (an IRAP access unit that is not an initial IRAP access unit) due to multi-loop decoding structure that requires complete reconstruction of its reference pictures for inter-layer texture prediction. This configuration is typical in the scenario (1) mentioned above with more IRAPs in enhancement layer than base layers.

As shown in Error: Reference source not found1, an initial IRAP AU (AU n) can also be used for random accessing. It is asserted that it may be possible to random access from non-initial IRAP AU (AU m) if the decoder can somehow know information on how many prior reference pictures in each of the reference layer are need to be decoded for decoding all non-RASL pictures that succeed the current picture at the target layer in decoding order.

This method potentially can give better coding efficiency than using fully aligned IRAP AU like AU n, where IRAP picture are inserted at all its dependent reference layers.

# Proposed Solution

In this contribution it is proposed to signal information to the decoder in an SEI message to indicate random accessibility discussed in section 1. This information can be two fold.

The first part is to signal if random accessibility is possible in the current access unit, that is, the information about the presence of an IRAP picture in AU prior to the reception of parsing of each layer picture.

It is proposed to signal specific prefix SEI message with the following syntax:

| irap_access_unit_info ( payloadSize ) { | Descriptor |
| --- | --- |
| irap_pic_exists_in_au_flag | u(1) |
| } |  |

irap_pic_exists_in_au_flag equal to 1 indicates that, when there are pictures in the access unit with nuh_layer_id greater than the nuh_layer_id of the current picture, at least one of such pictures is an IRAP picture. This flag may be present only for access units with TemporalId equal to 0.

This syntax provides early indication to the decoder system whether at least one picture in the access unit with nuh_layer_id greater than the nuh_layer_id of the current picture is an IRAP picture, without the need to parse all those pictures. Based on this information, it can be known whether layer switching/random accessing is possible at the current AU.

The second part is to signal the number of prior pictures that need to be decoded in all its reference layers, so as to be able to decode all non-RASL pictures that succeed the current IRAP picture at the target layer in decoding order. It is proposed to signal layer picture specific prefix SEI message with the following syntax:

| ref_layer_prior_pics_decode_info ( payloadSize ) { | Descriptor |
| --- | --- |
| lp_sei_active_vps_id | u(4) |
| for( i = 0; i <= NumDirectRefLayers[ layer_id_in_nuh[ nuh_layer_id ] ]; i++ ) |  |
| num_lower_layer_prior_pics[ i ] | u(8) |
| } |  |

lp_sei_active_vps_id identifies the active VPS of the CVS containing the ref_layer_prior_pics_decode_info SEI message. The value of lp_sei_active_vps_id shall be equal to the value of of vps_video_parameter_set_id of the active VPS for the VCL NAL units of the access unit containing the SEI message.

num_lower_layer_prior_pics[ i ] indicates the number of prior access units, in decoding order, in which the pictures with i-th direct reference layer of the current picture contain information that is needed to decode all non-RASL pictures that succeed the current picture at the target layer in decoding order.

This SEI message applies to an IRAP picture, which is referred to as the associated layer picture of the SEI message, also referred to as the current picture in the semantics below. When present, the SEI NAL unit containing such an SEI message shall precede the first VCL NAL unit of the associated layer picture and follow the last VCL NAL unit of the previous layer picture, all in decoding order. The nuh_layer_id of the SEI message is set to the nuh_layer_id of the associated IRAP picture.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).