| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0082 |
| --- | --- |

| Title: | 3D-HEVC HLS: On slice-level camera parameter signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yi-Wen Chen, Jian-Liang Lin, Yu-Pao Tsai, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yulin.chang, yiwen.chen, jl.lin, yupao.tsai, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, camera parameters of a layer with a layer index equal to layerId are signaled in the video parameter set (VPS) when cp_present_flag[ layerId ] coded in the VPS is equal to 1. Camera parameters of the layer can be also signaled in the slice segment header extension when cp_in_slice_segment_header_flag coded in the VPS is equal to 1. Since cp_present_flag[ layerId ] and cp_in_slice_segment_header_flag are independent, conflict between the two flags might happen. In this contribution, to solve the conflict, method 1 proposes to signal camera parameters in the slice segment header extension only when both cp_present_flag[ nuh_layer_id ] and cp_in_slice_segment_header_flag are equal to 1.

In the current 3D-HEVC, cp_in_slice_segment_header_flag is coded in the VPS and shared by all layers. When camera parameters of one or more layers are time variant and those of the rest layer(s) are time invariant, cp_in_slice_segment_header_flag is set to 1 and camera parameters have to be signaled in all slice segment header extensions of all layers. In order to signal camera parameters more efficiently, method 2 proposes to extend cp_in_slice_segment_header_flag to an array of flags, where each array element represents a flag for a layer. Only the one or more layers that have time variant camera parameters have to set their corresponding cp_in_slice_segment_header_flag[ layerId ] in VPS to 1 and signal camera parameters in their corresponding slice segment header extensions. In addition, using a different way than method 1 to solve the potential conflict between cp_present_flag[ layerId ] and cp_in_slice_segment_header_flag[ layerId ], method 2 proposes to send camera parameters of the layer with the layer index equal to layerId in VPS only when cp_present_flag[ layerId ] is equal to 1 and cp_in_slice_segment_header_flag[ layerId ] is equal to 0.

# Introduction

In the current 3D-HEVC, there are two coding tools, DoNBDV and VSP, which convert the depth values to disparity vector (DV) for prediction. This kind of coding tools, depth oriented coding tools, need camera parameters for depth to disparity conversion. In current HTM-8.0, camera parameters of a layer with layer index equal to layerId are coded in VPS when cp_present_flag[ layerId ] is equal to 1. However, camera parameters are also coded in the slice segment header extension when cp_in_slice_segment_header_flag coded in VPS is equal to 1. Since cp_present_flag[ layerId ] and cp_in_slice_segment_header_flag are independent, conflict between the two flags might happen. In this contribution, method 1 is proposed to solve the potential conflict.

Another issue in the current 3D-HEVC camera parameter signaling is about the efficiency when camera parameters of one or more layers are time variant and those of the rest layer(s) are time invariant. In this case, since cp_in_slice_segment_header_flag is coded in the VPS and shared by all layers, cp_in_slice_segment_header_flag is set to 1 and camera parameters have to be signaled in all slice segment header extensions of all layers. The layers that have time invariant camera parameters waste bits in sending the same camera information repeatedly in all their slice segment header extensions. In this contribution, method 2 is proposed to enhance the signaling efficiency in this case. In addition, method 2 also solves the potential conflict mentioned in the previous paragraph with a different way than method 1.

# Proposed methods

## Method 1: Check cp_present_flag in slice segment header extension

In order to solve the potential conflict of camera parameter signaling between VPS and slice segment header extension, we propose to code camera parameters in the slice segment header extension only when both cp_in_slice_segment_header_flag and cp_present_flag[ nuh_layer_id ] (i.e., the cp_present_flag of the current layer) are equal to 1, as shown in Table 1.

Table 1.  Slice segment header extension syntax

| slice_segment_header_extension( ) { | Descriptor |
| --- | --- |
| if( cp_in_slice_segment_header_flag && cp_present_flag[ nuh_layer_id ] ) { |  |
| for ( i = 0; i < ViewIdx; i++ ) { |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |

## Method 2: Extend cp_in_slice_segment_header_flag to an array

To enhance the efficiency of camera parameter signaling when camera parameters of one or more layers are time variant and those of the rest layer(s) are time invariant, we propose to extend cp_in_slice_segment_header_flag to an array of flags, where each array element represents a flag for a layer.

The proposed changes in VPS are shown in Table 2. In VPS, if cp_present_flag[ layerId ] is equal to 1, cp_in_slice_segment_header_flag[ layerId ] is coded; otherwise, it is not coded and inferred to be 0. Moreover, camera parameters of a layer with a layer index equal to layerId are coded in VPS only when cp_present_flag[ layerId ] is equal to 1 and cp_in_slice_segment_header_flag[ layerId ] is equal to 0. This can also solve the potential conflict of camera parameter signaling between VPS and slice segment header extension.

In Table 2, a pair missing braces is added, which is clearly a bug-fix.

The proposed changes in slice segment header extension are shown in Table 3. If the current slice belongs to a layer with time invariant camera parameters, cp_in_slice_segment_header_flag[ nuh_layer_id ] should be equal to 0 and its camera parameters have been coded in VPS; otherwise (i.e., the current slice belong to a layer with time variant camera parameters), cp_in_slice_segment_header_flag[ nuh_layer_id ] should be equal to 1 and its camera parameters are coded in the slice segment header extension.

Table 2.  Proposed VPS extension syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if ( layerId ! = 0 ) { |  |
| cp_present_flag[ layerId ] |  |
| if ( cp_present_flag[ layerId ] ) |  |
| cp_in_slice_segment_header_flag[ layerId ] | ue(v) |
| … |  |
| cp_precision | ue(v) |
| cp_in_slice_segment_header_flag | u(1) |
| if( !cp_in_slice_segment_header_flag ) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if( cp_present_flag[ layerId ] && !cp_in_slice_segment_header_flag[ layerId ] ) { |  |
| viewIdx = ViewOrderIdx[ layerId ] |  |
| for( j = 0; j < viewIdx; j++ ) { |  |
| vps_cp_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_off[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_scale_plus_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_off_plus_off[ viewIdx ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

cp_in_slice_segment_header_flag[ layerId ] equal to 1 specifies that the syntax elements of vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] are not present in the video parameter set. A value of cp_in_slice_segment_header_flag equal to 0 specifies that the vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] syntax elements are present in the slice segment header. When not present, the value of cp_in_slice_segment_header_flag[ layerId ] shall be inferred to be equal to 0.

Table 3.  Proposed slice segment header extension syntax

| slice_segment_header_extension( ) { | Descriptor |
| --- | --- |
| if( cp_in_slice_segment_header_flag[ nuh_layer_id ] ) { |  |
| for ( i = 0; i < ViewIdx; i++ ) { |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |

# Conclusion

In this contribution, method 1 proposes to add the checking of cp_present_flag in slice segment header extension to solve the conflict of camera parameter signaling between VPS and slice segment header extension. Method 2 proposes to extend cp_in_slice_segment_header_flag to an array of flags, where each array element is for a layer, to better support the case when camera parameters of one or more layers are time variant and those of the rest layer(s) are time invariant. Moreover, method 2 proposes to send camera parameters of a layer with a layer index equal to layerId in VPS only when cp_present_flag[ layerId ] is equal to 1 and cp_in_slice_segment_header_flag[ layerId ] is equal to 0, in order to solve the potential conflict of camera parameter signaling between VPS and slice segment header extension.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 5,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1005, Aug., 2013.