| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0082 |
| --- | --- |

| Title: | 3D-HEVC HLS: On slice-level camera parameter signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yi-Wen Chen, Jian-Liang Lin, Yu-Pao Tsai, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yulin.chang, yiwen.chen, jl.lin, yupao.tsai, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, camera parameters are transmitted according to the cp_present_flag for each layer. However, the camera parameter in the slice segment header extension is coded according to the flag cp_in_slice_segment_header_flag. The cp_in_slice_segment_header_flag could override the cp_present_flag in the slice segment header extension, and conflict might happen. In this contribution, it is proposed to solve the conflict between cp_present_flag and cp_in_slice_segment_header_flag. Two methods are proposed. The first method proposes to add the checking of cp_present_flag in slice segment header extension. The second method proposes to extend the cp_in_slice_segment_header_flag to a flag array, and the condition check for each corresponding flag is carried out in slice segment header extension to avoid conflict between these two flags.

# Introduction

In the current 3D-HEVC, there are two coding tools, DoNBDV and VSP, which convert the depth values to disparity vector(DV) for prediction. This kind of coding tools, depth oriented coding tools, need the camera parameters for depth to disparity conversion. In current HTM-8.0, the camera parameter is coded according to a cp_present_flag[ layerId ]. If cp_present_flag[ layerId ] equals to 1, the camera parameter is coded for the layer with layer index layerId. However, in current design, the camera parameter in the slice segment header extension is coded according to the cp_in_slice_segment_header_flag which is a flag independent of cp_present_flag. In this case, even when cp_present_flag[ layerId ] equals to 0, if cp_in_slice_segment_header_flag is enabled, the camera parameters may still be sent in the slice header extension for all layers.

# Proposed schemes

## Method 1: Add the checking of cp_present_flag in slice segment header extension

To solve this issue, it is proposed to add a constraint in the slice segment header extension to prevent the conflict between cp_present_flag and cp_in_slice_segment_header_flag.

As shown in Table 1, a condition check for cp_present_flag[nuh_layer_id] is added to further restrict the coding of the camera parameters in the slice segment header extension.

- Table 1. Slice segment header extension syntax

| slice_segment_header_extension( ) { | Descriptor |
| --- | --- |
| if( cp_in_slice_segment_header_flag && cp_present_flag[nuh_layer_id] ) { |  |
| for ( i = 0; i < ViewIdx; i++ ) { |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |

## Method 2: Extend cp_in_slice_segment_header_flag

To solve this issue, another method is proposed to extend the cp_in_slice_segment_header_flag to a flag array cp_in_slice_segment_header_flag[] corresponding to each layer. If cp_present_flag[layerId] is turned on, the cp_in_slice_segment_header_flag[layerId] is derived and coded. Otherwise, cp_in_slice_segment_header_flag[layerId] shall be inferred to be equal to 0.

As an example shown in Table 2, the cp_in_slice_segment_header_flag[layerId] is modified to resolve the conflict between cp_present_flag and cp_in_slice_segment_header_flag. In table 3, corresponding modification on slice segment header extension is shown.

- Table 2. VPS extension syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if ( layerId ! = 0 ) { |  |
| cp_present_flag[ layerId ] |  |
| if ( cp_present_flag[ layerId ] ) |  |
| cp_in_slice_segment_header_flag[ layerId ] | ue(v) |
| … |  |
| cp_precision | ue(v) |
| cp_in_slice_segment_header_flag | u(1) |
| if( !cp_in_slice_segment_header_flag ) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if( cp_present_flag[ layerId ] && !cp_in_slice_segment_header_flag[ layerId ]) { |  |
| viewIdx = ViewOrderIdx[ layerId ] |  |
| for( j = 0; j < viewIdx; j++ ) { |  |
| vps_cp_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_off[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_scale_plus_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_off_plus_off[ viewIdx ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

cp_in_slice_segment_header_flag[ layerId ] equal to 1 specifies that the syntax elements of vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] are not present in the video parameter set. A value of cp_in_slice_segment_header_flag equal to 0 specifies that the vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] syntax elements are present in the slice segment header. When not present, the value of cp_in_slice_segment_header_flag[ layerId ] shall be inferred to be equal to 0.

- Table 3. Slice segment header extension syntax

| slice_segment_header_extension( ) { | Descriptor |
| --- | --- |
| if( cp_in_slice_segment_header_flag[ nuh_layer_id ] ) { |  |
| for ( i = 0; i < ViewIdx; i++ ) { |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |

# Conclusion

In this contribution, two methods to solve the conflict between cp_present_flag and cp_in_slice_segment_header_flag are proposed. Method 1 proposes to add the checking of cp_present_flag in slice segment header extension. Method 2 proposes to extend the cp_in_slice_segment_header_flag to a flag array, and the condition check for each corresponding flag is carried out in slice segment header extension to avoid conflict between these two flags.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 5,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1005, Aug., 2013.