| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0252 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0083 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On signalling of representation format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | hendry@qti.qualcomm.com yekuiw@qti.qualcomm.com cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution discusses some issues of the current representation format signaling and proposes some modifications to address the issues. The current signaling of the representation format in VPS extension assumes the same default values for the syntax elements rep_format_idx_present_flag and vps_rep_format_idx when they are not present. However, in order to make the signalling optimal for both typical cases in MV-HEVC and SHVC, such syntax elements are proposed to be set with different default values based on the scalability dimensions.

# Problem Description

The representation format information for a video sequence – the spatial resolution, bit depth, and colour format – is signalled in the Video Parameter Set (VPS) and, if necessary, may be updated in Sequence Parameter Set (SPS) of each layer in an SHVC or MV-HEVC bitstream.

The current design of the representation format signalling has the following shortcomings:

- The first shortcoming is on the design of the syntax table. The condition checking “if (vps_num_rep_formats_minus1 > 0)” which is inside the loop “for (i = 1; i <= vps_max_layers_minus1; i++)” is not necessary. This will cause implementation of such design to be required to do the condition checking as many as the number indicated by vps_max_layers_minus1.

- The second problem is on the inference rule for vps_num_rep_formats_minus1 when rep-format_idx_present_flag is equal to 0. When rep_format_idx_present_flag is equal to 0, vps_num_rep_formats_minus1 is by default inferred to be equal to vps_max_layers_minus1. Such inference is fine for SHVC since in some typical scenarios for the scalable extension, the representation format of each layer is different. However, for MV-HEVC, it makes more sense to infer it to be equal to 0 since typically representation format does not change from layer to layer.

- The third problem in on the value of rep_format_idx_present_flag when both scalable extension and multi-view extension are used. Currently, the value of rep_format_idx_present_flag is flexible such that encoder can freely decide its value. When the value of rep_format_idx_present_flag is 0, the value of vps_num_rep_formats_minus1 shall be inferred.  Such inference is acceptable when only one type of extension is used in the bitstream (i.e., either the bitstream is an SHVC bitstream or MV-HEVC bitstream). However, when both extensions exist in a bitstream, the value of vps_num_rep_formats_minus1 should not be inferred.

- The fourth problem is on inference of vps_rep_format_idx[ i ]. Currently, it is said that when not present the value of vps_rep_format_idx[ i ] is inferred to be equal to 0. This is not correct when rep_format_idx_present_flag is equal to 0 and vps_num_rep_formats_minus1 is greater than 0, as in this case there are multiple representation formats signalled but only the first one is used and the rest are never used.

# Proposed solutions

To overcome the above problems, we proposed the following:

- To deal with the first and the fourth problem described above, it is proposed to remove the condition “if( vps_num_rep_formats_minus1 > 0 )” from within the loop “for( i = 1; i <=  vps_max_layers_minus1; i++ )”. Furthermore, the value of vps_rep_format_idx[ i ], when not present, is inferred as follows:

  - When rep_format_idx_present_flag is equal to 0 and vps_num_rep_formats_minus1 is equal to vps_max_layers_minus1, the value of vps_rep_format_idx[ i ] is inferred to be equal to i, for i ranges from 0 to vps_max_layers_minus1, inclusive.

  - When rep_format_idx_present_flag is equal to 0 and vps_num_rep_formats_minus1 is equal to 0, the value of vps_rep_format_idx[ i ] is inferred to be equal to 0, for i ranges from 0 to vps_max_layers_minus1, inclusive.

  - When rep_format_idx_present_flag is equal to 1 and i is equal to 0, the value of vps_rep_format_idx[ 0 ] is inferred to be equal to 0.

- To deal with the second problem described above, it is proposed to modify inference rule for the value of vps_num_rep_formats_minus1 when rep_format_idx_present_flag is equal to 0 (this solution also solves the fourth problem).

  - When scalability is used and multiview is not used, the value of vps_num_rep_formats_minus1 is inferred to be equal to vps_max_layers_minus1.

  - When scalability is not used and multiview is used, the value of vps_num_rep_formats_minus1 is inferred to be equal to 0.

- To deal with the third problem described above, it is proposed to add a constraint such that when the bitstream contains both scalability and multi-view, then rep_format_idx_present_flag shall be equal to 1, meaning that value of vps_num_rep_formats_minus1 shall not be inferred but be explicitly signaled.

The proposed changes are as follows, where additions are highlighted in this color and removed texts are strikethrough in red fonts.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(4) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag && vps_num_rep_formats_minus1 > 0 ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0 ) |  |
| vps_rep_format_idx[ i ] | u(4) |
| … |  |
| } |  |

rep_format_idx_present_flag equal to 1 specifies that the syntax elements vps_num_rep_formats_minus1 and vps_rep_format_idx[ i ] are present. rep_format_idx_present_flag equal to 0 specifies that the syntax elements vps_num_rep_formats_minus1 and vps_rep_format_idx[ i ] are not present.

It is a requirement of bitstream conformance that, when both scalability_mask[ 1 ] and scalability_mask[ 2 ] are equal to 1, the value of rep_format_idx_present_flag shall be equal to 1.

vps_num_rep_formats_minus1 plus 1 specifies the number of the following rep_format( ) syntax structures in the VPS. When not present, the value of vps_num_rep_formats_minus1 is inferred to be equal to vps_max_layers_minus1.

When not present, the value of vps_num_rep_formats_minus1 is inferred as follow:

–	If scalability_mask[ 1 ] is equal to 1 and scalability_mask[ 2 ] is equal to 0, vps_num_rep_formats_minus1 is inferred to be equal to 0.

–	Otherwise, when scalability_mask[ 1 ] is equal to 0 and scalability_mask[ 2 ] is equal to 1, vps_num_rep_formats_minus1 is inferred to be equal to vps_max_layers_minus1

vps_rep_format_idx[ i ] specifies the index, into the list of rep_format( ) syntax structures in the VPS, of the rep_format( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. When not present, the value of vps_rep_format_idx[ i ] is inferred to be equal to 0. The value of vps_rep_format_idx[ i ] shall be in the range of 0 to vps_num_rep_formats_minus1, inclusive.

When not present, the value of vps_rep_format_idx[ i ] is inferred as follows:

–	If rep_format_idx_present_flag is equal to 0 and vps_num_rep_formats_minus1 is equal to vps_max_layers_minus1, the value of vps_rep_format_idx[ i ] is inferred to be equal to i, for each value of i in the range of 0 to vps_max_layers_minus1, inclusive.

–	Otherwise, if rep_format_idx_present_flag is equal to 0 and vps_num_rep_formats_minus1 is equal to 0, the value of vps_rep_format_idx[ i ] is inferred to be equal to 0, for each value of i in the range of 0 to vps_max_layers_minus1, inclusive.

–	Otherwise (rep_format_idx_present_flag is equal to 1 and i is equal to 0), the value of vps_rep_format_idx[ 0 ] is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).