| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0087 |
| --- | --- |

| Title: | 2D Backwards Compatible Centralized Color-Depth Packing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang Hung-Ming Wang Yi-An Chiang 1 University Road, Tainan, Taiwan 701 | Tel: Email: | +886-6-2757575#62362 jfyang@ee.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU) |  |  |

_____________________________

# Abstract

3DTV displays have become more and more popular in the consumer market recently. However, bandwidth limitation and capability is still an important issue need to be solved before the terminal devices can be updated. To deal with this problem, the DVB has released the commercial requirements of the first phase of 3DTV in July 2010. The first problem which will be solved is the frame compatible format. The most popular frame compatible formats for transmitting stereoscopic views are side by side (SbS) and top and bottom (TaB) formats. However, there are still no related standard packing formats for videos with color and depth information. The proposed method targets on a 2D Backwards Compatible Centralized Color-Depth Packing (CCDP) for videos with color and depth information. The proposed format is compatible with ordinary 2DTV, which means it can be directly displayed without extra pre-processing. When we want to display the 3D contents, the color and depth information can be extracted easily with comparable quality and generate the desired 3D contents through rendering technique.

# Introduction

In recent years, more and more movies are produced in 3D formation, which will be viewed 3D effects through 3D glasses. It becomes a main trend that people like 3D contents and their 3D applications for entertainments. Of course, it is not satisfied for people just watching 3D movies in the cinema. They will have strong desires to watch 3D television programs in home as well. In the past five years, the 3DTV displays and 3D monitors have been promoted on the consumer market. For the current broadcasting systems, there are several exiting 2D frame compatiable side-by-side, top-and-bottom, and checkerboard stereopacking formats, which allow the broadcasters to deliver the 3D television programs through their existing channel easily. However, the existing frame compatible 3D stereo packing formats have two serious problems: 1) They can not support multiview naked-eye displays in a reasonable cost unless the 3D displays have implemented real-time stereo matching and multiple depth image-based rendering algorithms; 2) Those stereo packing formats produce serious annoying effects while showing two-view video in one frame for the traditional 2D display. These two problems still strongly affect the broadcasters to deliver more 3D contents for the users. In the commercial market, there are many stereo 3D displays and numerous number of 2D display systems. Besides, the various 3D naked-eye displays with different number of views are also available in the market but only limited to 3D research users.

To generate the multi-view contents, a possible transmission format of 3DTV broadcasting systems could be the original 2D video associated with its depth information. The 2D-plus-depth format, which will double the horizontal resolution, is a stereoscopic video coding format that is used for 3D displays proposed by Philips WOWvx 3D systems. The 2D-plus-depth with double horizontal resolution will not be frame compatible such that it cannot be used in the current digital video broadcasting systems. With 2D-plus-depth format, at the display side, the user can view multi-view contents if the depth-image-based rendering (DIBR) technique is built in. However, the issues of the bandwidth limitation, frame compatible, comfortable visualization for 2D displays are still needed to be solved for the 3D broadcasting services. With video and depth map at hands, the first problem needed to be solved is the frame compatible format and comfortable visualization of 2D displays. Currently, there are still no related standard packing formats for videos with color and depth information. Therefore, how to design an efficient packing format that can maintain the coding performance and be compatible with the existing 2D TV broadcasting systems is an important and urgent problem. All the details of the packing method are going to be explained in the next section.

# Proposed 2D Backwards Compatible CCDP

Fig. 1 shows the flowchart of the proposed packing mechanism. If we want to keep the same resolution for video and depth frames, we could downscale the vertical resolution of color and depth frames into three fourth of the original resolution. Here, we use common bilinear method to resize color frame and depth frame. For the resized depth map, since only Y channel is utilized, the color-based pixel arrangement can be adopted to make it more compact. To achieve this, we embed the Y values into U and V channels in vertical direction as shown in Fig. 2. The vertical resolution of depth map becomes one third of the original.

After rearranging, we treat YUV channels as RGB channels and perform RGB to YCbCr conversion by the following formula.

Y   =    0.2568R ＋ 0.5041G ＋ 0.0979B ＋ 16

Cb  =  －0.1482R － 0.2910G ＋ 0.4392B ＋ 128

Cr  =    0.4392R － 0.3678G － 0.0714B ＋ 128

Then, we split the depth frame averagely into two separated parts and flip them. Finally, we get the packing image by combining the first depth part, resized color frame, and the other depth part from top to bottom sequentially. The diagram of the transmitting architecture for the proposed method is shown in Fig. 3.

Fig. 1. Flowchart of the proposed 2D Backwards Compatible CCDP process

Fig. 2. Procedure of rearranging depth frame into color format vertically

# Fig. 3. Diagram of the broadcasting architecture for the proposed method

# Simulations and Discussions

To verify the proposed method, Fig. 4 shows the RD curve of average results for sequence Undo_Dancer. It is noted that only view 5 is considered in this experiment. In Fig. 4, the PSNR is calculated by averaging the Y-PSNR in color and depth image, and the bit rate is obtained by summing   all the related bits that are needed to recover the reconstructed color and depth frames. Fig. 5 individually shows the RD curves of Y, U, V in color image and Y in depth image. From Figs. 4 and 5, we can see that the proposed method can achieve comparable coding performance with the existing 3D-HTM architectures. However, the proposed method can be designed to make it compatible with the traditional digital broadcasting systems, while the CCDP frame is coded in the base-layer by the traditional video encoder, said MPEG-2 or H.264 encoder.

Fig. 4. RD curve of average results for sequence Undo_Dancer

| V_component Y_depth Y_component U_component |  |
| --- | --- |

Fig. 5. RD curves Y, U, V in color image and Y in depth image for sequence Undo_Dancer

# Conclusions

In this proposal, the 2D Backwards Compatible CCDP algorithm is proposed to solve the bandwidth for transmitting 3D videos via the existing infrastructure. The proposed packing format can be directly displayed in 2D TV display in reasonable perceptual quality without any extra pre-processing. As for the coding performance, experimental results also reveal that the proposed method can achieve comparable coding performance with the existing 3D-HTM architectures.

# Patent rights declaration(s)

National Cheng Kung University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).