| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0088 |
| --- | --- |

| Title: | 2D Backwards Compatible Centralized 2-View Packing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang, Ke-Ying Liao, Ya-Han Hu, No. 1, University Road, Tainan City, Taiwan | Tel: Email: | +886-6-275-7575 ext. 62362 jfyang@ee.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU) |  |  |

_____________________________

# Abstract

In the recent year, 3D movies have become a flourishing focused point in film industry. Various manufactories continue to introduce new 3D stereo TV displays. The 3D TV displays have become more and more popular in the consumer market recently. The TV broadcasters start to provide 3D videos in their channels; however, they did not increase more 3D programs in their channels. We all know that 3D video services with multiple views information will be the trend of future entertainments. However, traditional digital TV and stereo 3D TV receivers are still the most popular for the home users currently. The traditional 3D packing systems do not provide good services for the 2D TV players. In order to make 3D broadcasting system is compatible with 3D and traditional 2D televisions; this contribution develops a new packing format, called the centralized 2-view (C2V) 3D video packing format that provides a more unrestricted and comfortable vision experience for traditional 2D display. With C2V format, it is noted that the users can perceive the more comfortably TV programs without any un-packing process, while the traditional 2D TV receiving 3D video contents.

# Introduction / Problem Statement

3D TV displays have become more and more popular in the consumer market recently. The most popular frame compatible formats for transmitting stereoscopic views are side by side (SbS) and top and bottom (TaB) formats. However, there still has a problem for receiving the SbS and TaB formats. When TV Company broadcast the 3D contents, the ordinary 2D TV cannot use tuner to select one of the 3D views. The only solution is to be forced receiving the 3D contents having SbS and TaB format such that users may not comfortably watch 3D contents. Visual discomfort is the common problem when the audience views the stereoscopic 3D video through traditional 2D TV receivers. The target of this contribution is to propose the 2D backwards compatible centralized 2-view (C2V) packing for videos with color information. The proposed format is compatible with the ordinary 2D TV, which means it can be directly displayed without extra pre-processing. When we want to display the 3D contents, the color information can be extracted easily with comparable quality and generate the desired 3D contents through rendering technique. Fig. 1 shows the flowchart of the proposed transmission framework.

Figure 1: The flowchart of the proposed transmission framework

# Proposed Algorithm

2.1 Centralized 2-View Packing

Frame packing refers to the combination of two frames, one for the left eye and the other for the right eye, into a single “packed” frame that consists of these two individual sub-frames. In this contribution, 2D backwards compatible centralized 2-view packing includes two parts as down-sampling, and rearrangement and centralized packing. The flowchart of the proposed 2D backwards compatible centralized 2-view (C2V) packing is shown in Figure 2.

Figure 2：The flowchart of the proposed 2D backwards compatible centralized 2-View packing

In this contribution, the 2-view signals having M×N resolution are first downscaled by 2 in the vertical resolution. It is noted that we use checkerboard down-sampling. The method is to extract the pixels at odd row odd column, even row even column. The vertical alignment is also needed to deal with pixels. We combine the 2 rows (1 even and 1 odd row) into a new row with vertical alignment method. So it can generate a view of the same width and half high. After vertical checkerboard down-scaling and vertical alignment, we can get two M×N/2 frames for left and right views. The vertical checkerboard down-sampling and vertical alignment methods are shown in Fig. 3.

Figure 3: Checkerboard down-sampling and vertical alignment method

The L half-size frame, for example, is further divided into two parts called T1 and T2 with M×N/4 subframes. Then, the T1 and T2 subframes are reversed along the vertical direction called T1-reverse and T2-reverse. The reverse arrangement will make T1-reverse and T2-reverse have the best alignment with bottom view (e.g. R view). We them place T1-reverse and T2-reverse at the top and the bottom of R half-size view of the packed frame, respectively. As shown in Fig. 4, the centrailzed 2-view packing format along with test frames are ordered in T1-reverse, R view, and T2-reverse packing orders vertically.

Figure 4: Examples of rearrangement and centralized packing

2.2 Centralized 2-View De-Packing

After video decoding, the reconstruction centralized 2-view packed video can be obtained. Then, we re-extract the reconstructed frame form the center view as the right view, and reverse top and bottom sides as the left view. After de-packing process, we can get two down-sampled views which size is M×N/2. There will be many holes in the de-packed left-eye and right-eye frames. To recover these holes, an edge-dependent interpolation method [1] is used in this contribution. The hole pixels can be interpolated from either vertical or horizontal direction. To choose the appropriate direction, we will select the direction which has smaller pixel difference. The brief concept about our hole-filling is to use a weighted matrix [1, -5, 20, 20, -5, 1]/32 with the non-hole pixels to estimate the hole pixels.

# Simulations

The anchor software is HTM-8.0 as demanded [2] with 2 view + 2 depth configuration. It is noted that the bit rate is obtained by summing all the related bits that are needed to recover the reconstructed color and depth frames. That means the the bit rate comparison uses the sum of original_view1, original_view2, original_depth1, original_depth2 versus the sum of packed-view and packed-depth to evaluate the coding efficiency. The PSNR is calculated by averaging the Y-PSNR in view 1 and view 2. We calculate PSNRs of two de-packed views to evaluate the coding quality. The de-packed frame can get form decoding, de-packing and up-sampling process. Then, we compare de-packed view with anchor reconstructed view. It can generate PSNR_Y, PSNR_U, and PSNR_V shown as fig.5. The method to generate PSNR of depth is the same as the method of views. Because of the U and V of depth are both 128, we calculate PSNR_Y only. The experimental results of 2 view + 2depth configuration are shown in Error: Reference source not found1. Figure6 shows the RD curves of average results for sequence PoznaHall2 with Anchor, TaB, and proposed C2V pacing methods.

Figure 5: PSNR calculation

Figure 5: The RD curves of average results for sequence PoznaHall2

# Conclusion

This contribution develops a new packing format that achieves provides a more unrestricted and comfortable vision experience for 2D TV receivers. When traditional TV receiving 3D video contents, users can more comfortably watch TV programs without any un-packing process for the compatible propose. When we want to display the 3D contents, the color information can be extracted easily with comparable quality and generate the desired 3D contents through rendering technique.

# Reference

[1]	An-Ti Chiang, Hung-Ming Wang, Jar-Ferr Yang and Jhing-Fa Wang,“A New Stereo Packing Format  based on Checkerboard Sub-sampling for Efficient Stereo Video Coding”, 2012 International Symposium on Circuits and Systems (ISCAS 2012)

[2]	JCT3V-E1100, “Common Test Conditions of 3DV Core Experiments,“Vienna, AT, 27 July-2 Aug. 2013.

# Patent rights declaration(s)

NCKU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).