| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0265 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0089 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On signaling of enhancement layer skip picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Krishna Rapaka Ye-Kui Wang Vadim Seregin Xiang Li Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com krapaka@qti.qualcomm.com yekuiw@qti.qualcomm.com vseregin@qti.qualcomm.com lxiang@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides the syntax design and decoding process of enhancement layer skipped picture.

# Introduction

The method of enhancement layer skipped slice and skipped picture had been proposed for several meeting. It seemed agreed at 14th JCTVC meeting that some kind of skipping capability is desirable and the committee encouraged to further study whether that should operate at the slice level or picture level (or perhaps tile level), and exactly how it should operate.

# Proposed Solution

The design logic of SHVC is that only HLS syntax and decoding process is modified based on HEVC except resampling process. To avoid additional operation and complexity in the decoding process this contribution proposes supporting picture level skipping in SHVC.

The skip picture flag is signaled for an enhancement layer P or B slice when the number of active inter layer reference layer picture of the current picture is larger than 0. The slice level parameters for interlayer reference picture set is still signalled so that the encoder have the flexibility to select the reference layer picture at picture level for the skip picture.

When the skip picture flag is equal to 1, the value of the sample in the slice is set equal to the value of the collocated sample of a nearest inter-layer reference picture presented in the inter-layer reference picture.

## Slice segment header syntax and semantics modification

A new syntax element skip_picture_flag is added to the slice segment header as below,

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| .. |  |
| } |  |
| if( nuh_layer_id > 0 && ! all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if(nuh_layer_id > 0 && (slice_type = = P ¦ ¦ slice_type = = B) && NumActiveRefLayerPics > 0) { |  |
| skip_picture_flag | u(1) |
| if(! skip_picture_flag) { |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( pps_loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_deblocking_filter_disabled_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| } |  |
| if( (tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) && ! skip_picture_flag) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

skip_picture_flag equal to 1 specifies that slice_segment_data( ) is not present for the current slice.  skip_picture_flag equal to 0 specifies that slice_segment_data( ) is present for the current slice. When skip_picture_flag is not present, it is inferred to be equal to 0. .

It is a requirement of bitstream conformance that, when skip_picture_flag is equal to 1, the number of slices in the current picture is 1.

It is a requirement of bitstream conformance that, when skip_picture_flag is equal to 1, the current picture shall not be used as collocated picture for temporal motion vector prediction derivation for any picture succeeds the current picture in decoding order.

## Slice segment layer RBSP syntax modification

| slice_segment_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_segment_header( ) |  |
| if (!skip_picture_flag) |  |
| slice_segment_data( ) |  |
| rbsp_slice_segment_trailing_bits( ) |  |
| } |  |

## Decoding process for skip picture

H.8.1.1 Decoding process for a coded picture with nuh_layer_id greater than 0

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found and Error: Reference source not found specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Prior to decoding the first slice of the current picture, subclause Error: Reference source not found is invoked.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause Error: Reference source not found is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

- If skip_picture_flag is equal to 1, the reconstructed picture sample arrays of CurrPic SL, SCb, and SCr is derived as follows:

SL[ i ][ j ] = S′L[ i ][ j ], with i = 0.. PicWidthInSamplesL − 1, j = 0.. PicHeightInSamplesL – 1

SCb[ i ][ j ] = S′Cb[ i ][ j ], with i = 0.. PicWidthInSamplesC − 1, j = 0.. PicHeightInSamplesC – 1

SCr[ i ][ j ] = S′Cr[ i ][ j ], with i = 0.. PicWidthInSamplesC − 1, j = 0.. PicHeightInSamplesC − 1

Where S′L, S′Cb, and S′Cr is the sample arrays of inter-layer reference picture specified by RefPicSetInterLayer0[NumActiveRefLayerPics − 1].

- Otherwise, the processes in subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

- After all slices of the current picture have been decoded, the marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).