| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0266 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0090 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On flushing of decoded pictures from the DPB based on NoOutputOfPriorPicsFlag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ying Chen Ye-Kui Wang Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document discusses flushing of decoded pictures from the DPB based on NoOutputOfPriorPicsFlag in MV-HEVC and SHVC, reports a few problems, and proposes a solution in order to address the reported problems.

It is proposed that the flushing of pictures be made layer specific for both types of decoder conformances. The flushing process is enabled to occur at each IRAP picture with NoRaslOutputFlag equal to 1, and at each LIP picture, instead of to occur at only each IRAP picture that has NoRaslOutputFlag equal to 1 and nuh_layer_id equal to 0. The value of NoOutputOfPriorPicsFlag is derived for all IRAP pictures with NoRaslOutputFlag equal to 1 and inferred, based on the value of NoClRasOutputFlag, for all non-IRAP pictures that are LIP.

# On NoOutputOfPriorPicsFlag

## Bumping and flushing of pictures

The decoded pictures are stored in the decoded picture buffer (DPB), and these decoded pictures include those that are needed for output, or marked as "used as reference". The pictures that are not needed for output and marked as "unused for reference" are present in the DPB until they are removed by the decoding process.

In output order conformant decoders, the process of removal of pictures in the DPB often immediately succeeds the output of pictures that are marked as "needed for output". This process of output and subsequent removal is referred to as bumping.

There are also situations where the decoder may remove the pictures in the DPB without output, even though these pictures may be marked as "needed for output". For ease of description, decoded pictures that are present in the DPB at the time of decoding an IRAP picture (irrespective of the reference marking and output marking of these pictures) are referred to as lagging DPB pictures associated with the IRAP picture or associated lagging DPB pictures of the IRAP picture. Some examples of such situations, in the HEVC context, are given below:

- When a CRA picture with NoRaslOutputFlag equal to 1 is present in the middle of a bitstream (i.e. this CRA picture is not the first picture in the bitstream), its associated lagging DPB pictures would not be output and would be removed from the DPB. Such situations are likely to occur at splice points, where two bitstreams are spliced and the first picture of the latter bitstream is a CRA picture with NoRaslOutputFlag equal to 1.

- When an IRAP picture picA that has NoRaslOutputFlag equal to 1 and that is not a CRA picture (e.g. IDR) is present in the middle of a bitstream and the resolution of the picture changes at picA (with the activation of a new SPS), the associated lagging DPB pictures of picA may have to be removed from the DPB before they are output, because otherwise if they continue to occupy the DPB decoding of the pictures starting from picA becomes problematic e.g. due to buffer overflow. In this case, the value of no_output_of_prior_pics_flag associated with picA should be set equal to 1 by the encoder or splicer, or even derived to be equal to 1 by the decoder, to flush the lagging pictures without output out of the DPB.

This process of removal of associated lagging DPB pictures from the DPB without output is referred to as flushing. Even in situations not described above, an IRAP picture may specify the value of no_output_of_prior_pics_flag equal to 1, so the decoder will flush the associated DPB lagging pictures of the IRAP picture.

## Flushing of pictures in multi-layer context

The process of flushing pictures is also relevant in multi-layer bitstreams, more specifically, it is relevant to all pictures that belong to an initial IRAP access unit, and also to pictures that are IRAP pictures not belonging to an initial IRAP access unit. In SHVC WD3 and MV-HEVC WD5, an IRAP access unit is defined as an access unit containing an IRAP picture that has nuh_layer_id equal to 0 (regardless of whether other pictures in the access unit are IRAP pictures), and an initial IRAP access unit is defined as an access unit containing an IRAP picture that has nuh_layer_id equal to 0 and that has NoRaslOutputFlag equal to 1 (again regardless of whether other pictures in the access unit are IRAP pictures).

With the possibility of having non-aligned IRAP pictures in access units (i.e. an access unit may contain both IRAP pictures and non-IRAP pictures) in SHVC and MV-HEVC, the situations described in the previous section in the context of HEVC can occur in different layers of an SHVC/MV-HEVC bitstream. A CRA picture picA that has NoRaslOutputFlag equal to 1 may be present at an enhancement layer (EL) in the middle of the bitstream (i.e. not in the first access unit of the bitstream) of a bitstream that starts with an initial IRAP access unit that does not have a CRA picture in the same layer as picA. The resolution change of a picture could occur at IRAP pictures in enhancement layer at an access unit where the resolution of the base layer (BL) does not change, or vice versa. Similar situations can be made for different DPB sizes.

### Flushing of pictures in SVC and MVC

Due to the single-loop coding design of SVC, only one reconstructed picture per access unit is inserted in the DPB, except for cases when the so-called medium-granular scalability (MGS) is in use then there can be two decoded pictures from the so-called key-picture access units to be stored in the DPB. However, in each access unit only the decoded picture of the highest layer may be output. Management of the decoded picture buffer, including the flushing of pictures, therefore, happens only for pictures in the highest layer, mainly because a decoded picture of a base layer is not required to be present in the DPB so as to predict the enhancement layer.

In MVC, more than one view may be target output view, and decoded view components are need to be maintained to predict view components in other layer, even if they are not needed to predict view components in the same layer. Therefore view components from more than one view may be present in the DPB. The flag no_output_of_prior_pics_flag is signaled for each IDR view component (an IDR view component of a non-base view is signalled with non_idr_flag equal to 0), and the flushing of view components is layer specific (or view specific). In MVC, for simplicity, the IDR view components in an access unit in MVC are aligned; if one view component in an access unit is IDR, all the view components in that access unit are also IDR view components. Hence, flushing operation also is performed aligned across all the views in the bitstream, even though the operation is view/layer specific.

### Flushing of pictures in SHVC and MV-HEVC

When flushing occurs under the current design in SHVC and MV-HEVC, all the pictures in the DPB are removed without output. It is not possible that pictures of only one layer in the bistream (except in the trivial case when only the base layer is present in the bitstream) are flushed, therefore the flushing is not layer-specific.

#### Output timing conformance

The output and removal of pictures from the DPB for output timing conformance in the latest versions of MV-HEVC and SHVC working drafts are described below (the parts relevant to the flushing process are highlighted in pink). In SHVC and MV-HEVC, the removal of pictures invoked is specific to each layer, as specified in Sec. F.13.3.2.

- C.3.2	Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of access unit n (containing the current picture) and proceeds as follows:

–	The decoding process for RPS as specified in subclause Error: Reference source not found is invoked.

–	When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD, such that when the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

–	When both of the following conditions are true for any pictures k in the DPB, all such pictures k in the DPB are removed from the DPB:

–	picture k is marked as "unused for reference"

–	picture k has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e. DpbOutputTime[ k ] is less than or equal to CpbRemovalTime( m )

–	For each picture that is removed from the DPB, the DPB fullness is decremented by one.

- F.13.3.2	Removal of pictures from the DPB

The specifications in subclause C.3.2 apply separately for each set of decoded pictures with a particular value of nuh_layer_id with the following modifications.

–	Replace "The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of access unit n (containing the current picture) and proceeds as follows:" with "The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of the picture n and proceeds as follows:".

#### Output order conformance

The output and removal of pictures from the DPB for output order conformance in the latest versions of MV-HEVC and SHVC working drafts are described below (the parts relevant to the flushing process are highlighted in grey).

- F.13.5.2.2	Output and removal of pictures from the DPB

The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the current picture is removed from the CPB and proceeds as follows:

The decoding process for RPS as specified in subclause Error: Reference source not found is invoked.

–	If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

–	Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers  containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause Error: Reference source not found, and the DPB fullness is set equal to 0.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1 or with nuh_layer_id not equal to 0), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. The variable currLayerId is set equal to nuh_layer_id of the current decoded picture and when one or more of the following conditions are true, the "bumping" process specified in subclause Error: Reference source not found is invoked repeatedly while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied, until none of the following conditions are true:

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB that are marked as "needed for output" is greater than sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId (when currLayerId is not equal to 0).

–	sps_max_latency_increase_plus1[ HighestTid ] of the active SPS (when currLayerId is equal to 0) or the active layer SPS for the value of currLayerId is not equal to 0 and there is at least one picture with nuh_layer_id equal to currLayerId in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount[ currLayerId ] is greater than or equal to SpsMaxLatencyPictures[ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB is greater than or equal to sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

# Problem

The process defined for flushing pictures from DPB in SHVC WD3 and MV-HEVC WD5 have the following shortcomings:

- The flushing behavior is not aligned for the two types of decoder conformance: output order conformance and output timing conformance.

- For output timing conformance decoders, the flushing is invoked for each layer picture that is not the first picture of the layer in the bitstream and that has NoRaslOutputFlag equal to 1, and when invoked, it flushes all decoded pictures of that layer in the DPB.

- For output order conformance decoders, the flushing is only invoked for the base layer picture that is not the first picture in the bitstream and that has NoRaslOutputFlag equal to 1, and when invoked, it flushes all decoded pictures of all layers in the DPB.

- In a bitstream with two layers, when a different resolution is activated at the EL at a Layer Initialisation Picture (LIP) that is an IRAP picture and that does not belong to an IRAP AU (and the resolution of the base layer cannot change at this AU as the base layer picture in an non-IRAP AU cannot be an IRAP picture), layer specific flushing of pictures may be needed. Here only pictures from the EL but not from the BL need to be flushed. This is currently not possible for output order conformance.

- In a bitstream with two layers, let there be an access unit where the BL picture is an IDR picture and the EL picture is a non-IRAP picture, and the resolution of the BL picture is updated, while the resolution of the EL picture is not updated. In this case, flushing should be performed only for the pictures from the BL, while the EL pictures should not be flushed. Again this is currently not possible for output order conformance.

# Proposal

The flushing of pictures is made layer specific for both types of decoder conformances. The flushing process is enabled to occur at each IRAP picture with NoRaslOutputFlag equal to 1, and at each LIP picture, instead of to occur at only each IRAP picture that has NoRaslOutputFlag equal to 1 and nuh_layer_id equal to 0.

no_output_of_prior_pics_flag is signaled only for IRAP pictures, and NoOutputOfPriorPicsFlag is derived for all IRAP pictures with NoRaslOutputFlag equal to 1 and inferred, based on the value of NoClRasOutputFlag, for all non-IRAP pictures that are LIP.

The changes to the syntax, semantics and the decoding process are provided below, highlighted in green or in red strikethrough with respect to MV-HEVC WD5.

### Changes to Section C.3.2 Removal of pictures from the DPB

C.3.2	Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of access unit n (containing the current picture) and proceeds as follows:

–	The decoding process for RPS as specified in subclause Error: Reference source not found is invoked.

–	When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 a LIP (or an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0) that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture with NoRaslOutputFlag equal to 1, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS of the current picture is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture with the same layer ID as the current picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, if the current picture is an IRAP picture, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

–	Otherwise if the current picture (which is a LIP) is not an IRAP picture and if NoClrasOutputFlag associated with the IRAP picture nuh_layer_id equal to 0 in the current access unit is equal to 1, NoOutputOfPriorPicsFlag is set equal to 1.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to 0.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD, such that when the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers that are in the DPB and that contain pictures having the same value of nuh_layer_id as that of the current picture, are emptied without output of the pictures they contain, and the DPB fullness is decremented by the number of pictures removed set equal to 0.

–	When both of the following conditions are true for any pictures k in the DPB, all such pictures k in the DPB are removed from the DPB:

–	picture k is marked as "unused for reference"

–	picture k has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e. DpbOutputTime[ k ] is less than or equal to CpbRemovalTime( m )

–	For each picture that is removed from the DPB, the DPB fullness is decremented by one.

### Changes to output and removal of pictures from DPB

- F.13.5.2.2	Output and removal of pictures from the DPB

The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the current picture is removed from the CPB and proceeds as follows:

The decoding process for RPS as specified in subclause Error: Reference source not found is invoked.

–	If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 a LIP (or an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0) that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS of the current picture is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture with the same layer ID as the current picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, if the current picture is an IRAP picture, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

–	Otherwise if the current picture (which is a LIP) is not an IRAP picture and if NoClrasOutputFlag associated with the IRAP picture nuh_layer_id equal to 0 in the current access unit is equal to 1, NoOutputOfPriorPicsFlag is set equal to 1.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to 0.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers that are in the DPB and that contain pictures having nuh_layer_id equal to that of the current picture are emptied without output of the pictures they contain, and the DPB fullness is decremented by the number of pictures removed. set equal to 0.

–	Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers  containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause Error: Reference source not found, and the DPB fullness is set equal to 0.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1 or with nuh_layer_id not equal to 0), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. The variable currLayerId is set equal to nuh_layer_id of the current decoded picture and when one or more of the following conditions are true, the "bumping" process specified in subclause Error: Reference source not found is invoked repeatedly while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied, until none of the following conditions are true:

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB that are marked as "needed for output" is greater than sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId (when currLayerId is not equal to 0).

–	sps_max_latency_increase_plus1[ HighestTid ] of the active SPS (when currLayerId is equal to 0) or the active layer SPS for the value of currLayerId is not equal to 0 and there is at least one picture with nuh_layer_id equal to currLayerId in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount[ currLayerId ] is greater than or equal to SpsMaxLatencyPictures[ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB is greater than or equal to sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).