| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0273 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0091 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On multi-mode bitstream extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Ye-Kui Wang Adarsh K. Ramasubramonian 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-658-5801 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In MV-HEVC/SHVC, it is not necessary for each AU to be "complete", i.e. picture from each layer in the bitstream is present in the AU. This allows use cases like picture rate scalability in SHVC as well as optimization of the size of the bitstream by discarding some pictures not needed for decoding of pictures belong to the target output layers, e.g. pictures with discardable_flag equal to 1 in layers that are not target output layers.

It has been proposed in JCTVC-N0267/JCT3V-E0087 to enable bitstream extraction with different modes. In this contribution, multi-mode bitstream extraction is re-proposed, and furthermore, it is proposed to require sub-bitstreams extracted under all the defined extraction modes to be conforming bitstreams, and different levels of conformance are specified.

# Introduction

In SVC, MVC, MVC+D, and HEVC version 1, the sub-bitstream extraction process is specified. Each extractable sub-bitstream is required to be a conforming bitstream.

One issue that has been considered in the design of the bitstream extraction process, e.g. in MVC development, is whether to target the best sub-bitstream self-completeness (such that each sub-bitstream further extracted from a sub-stream, recursively, can be a conforming bitstream) or the optimal size of a sub-bitstream (such that some or all pictures that are not needed for correct decoding of the target output layers do not have to be included in the extracted sub-bitstream).

Targeting best self-completeness makes the bitstream extraction process easier to specify. A self-complete sub-bitstream is still "fully extractable" and is more useful if the bitstream extraction is performed by a streaming server or an intermediate network element and each subset of the bitstream may be further requested by some end users. However, on the other hand, if the extracted sub-bitstream is directly sent to end users and is not supposed to be further extracted, better transmission bandwidth efficiency can be achieved by targeting the optimal size of the bitstream.

To enable either advantage to be applied depending on applications, the sub-bitstream extraction process can be designed to adaptively choose the best extraction operation.

Therefore, we proposed to support multi-mode bitstream extractions in JCTVC-N0267/JCT3V-E0087.

However, as has been discussed in the 14th JCT-VC and 5th JCT-3V meeting, to make the extracted bitstreams meaningful with all the proposed extraction modes, the specification should explicitly allow such bitstreams to be conforming.

This proposal therefore extends the JCTVC-N0267/JCT3V-E0087 and the major aspects are summarized below:

- Defining conformance points in VPS in a way that each conformance point is not only decided by the output layer set, but also a conformance level.

- Extending of the bitstraem extraction process so that it either takes an output layer set with an extraction mode, or, as currently specified, just a number of layers to be present in the bitstream. The first method is mainly targeted for one specific operation point, although it may or may not be further extracted. The second method is useful for the extraction of a set of operation points, e.g., stereo operation points ONLY in the multiview video context.

- Indication of the conformance level for a specific bitstream, which also implies which type of bitstream extraction may have been applied to produce this stream and which bitstream extraction may be further applied.

# Proposal

## VPS design

In VPS as well as VUP VUI, the operation points are defined with output layer sets and the associated conformance level values.

For each of such operation point, an HRD parameter set may be present and the video characteristics may be signalled.

### Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_offset | u(16) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| if( NumViews > 1 ) |  |
| view_id_len_minus1 | u(4) |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| all_ref_layers_active_flag | u(1) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( NumConformancePoints=0, i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| num_present_conformance_level_minus1[ i ] | u(2) |
| NumConformancePoints += num_present_conformance_level_minus1[ i ] |  |
| for( j = 0; j <= num_present_conformance_level_minus1[ i ]; j++) { |  |
| if( num_present_conformance_level_minus1[ i ] < 2 ) |  |
| conform_level[ i ][ j ] | u(2) |
| profile_level_tier_idx[ i ][ j ] | u(v) |
| } |  |
| } |  |
| add_vps_num_hrd_parameters | u(v) |
| for( i = 0; i < add_vps_num_hrd_parameters; i++ ) { |  |
| hrd_output_layer_set_idx[ i ] | ue(v) |
| if( num_present_conformance_level_minus1[ hrd_output_layer_set_idx[ i ] ] > 0 ) |  |
| vps_ext_hrd_conformance_level_idx[ i ] | u(2) |
| add_cprms_present_flag[ i ] | u(1) |
| hrd_parameters( add_cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(4) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0 ) |  |
| vps_rep_format_idx[ i ] | u(4) |
| max_one_active_ref_layer_flag | u(1) |
| cross_layer_irap_aligned_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| single_layer_for_non_irap_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_vui_alignment_bit_equal_to_one | u(1) |
| vps_vui( ) |  |
| } |  |
| } |  |

### Video parameter set extension semantics

num_present_conformance_level_minus1[ i ] plus 1 specified the number of conformance levels for the i-th output layer set. The value of num_present_conformance_level_minus1[ i ] is in the range of 0 to 2, inclusive.

conform_level[ i ][ j ] specifies the j-th conformance level present for the i-th output layer set. When not present, conform_level[ i ][ j ] is inferred to be equal to j.

Let outLayerSet be the output layer set and presentLayerset be the layer set present for the output layer set to be decoded, let maxTid be the greatest values of TemporalId of pictures in layers of the presentLayerset.

A conformance level value equal to 0 indicates that the highest TemporalId value of each layer in presentLayerset is equal to maxTid, and each access unit contains one picture from each layer in presentLayerset.

A conformance level value equal to 1 indicates that the highest TemporalId value of each layer in outLayerSet is euqal to maxTid, and each access unit contains one picture from each layer in outLyaerSet; in addition, for each layer A within presentLayerset but not in outLayerSet, if the highest TemporalId of pictures in this layer is maxTidPreLayer, in each access unit with a TemporalId smaller than or equal to maxTidPreLayer, a picture in layer A is present in the access unit.

A conformance level value equal to 1 indicates the highest TemporalId value of each layer in outLayerSet is euqal to maxTid, and each access unit contains one picture from each layer in outLyaerSet.

profile_level_tier_idx[ i ][ j ] specifies the index, into the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to i-th output layer set with the j-th conformance level. The length of the profile_level_tier_idx[ i ] syntax element is Ceil( Log2( vps_num_profile_tier_level_minus1 + 1 ) ) bits. The value of profile_level_tier_idx[ 0 ][ 0 ] is inferred to be equal to 0. The value of profile_level_tier_idx[ i ][ j ] shall be in the range of 0 to vps_num_profile_tier_level_minus1, inclusive.

- add_vps_num_hrd_parameters specifies the number of hrd_parameters( ) syntax structures present in the VPS extention of the VPS RBSP.

- hrd_output_layer_set_idx[ i ] specifies the index, into the output layer sets specified by the VPS, of the layer set to which the i-th hrd_parameters( ) syntax structure in the VPS extension applies. hrd_output_layer_set_idx[ i ] is in the range of 0 to 1023, inclusive, to appear in the syntax.

- vps_ext_hrd_conformance_level_idx[ i ] specifies the index to the conformance level specified in VPS extesnton for the hrd_output_layer_set_idx[ i ]-th output layer set. When num_present_conformance_level_minus1[ hrd_output_layer_set_idx[ i ] ] is equal to 0, vps_ext_hrd_conformance_level_idx[ i ] is derived to be equal to 0.

- The conformance level of the hrd_layer_set_idx[ i ] ]-th output layer set, associated with the i-th hrd_parameters( ) syntax structure in the VPS is inferred to be equal to 0.

add_cprms_present_flag[ i ] equal to 1 specifies that the HRD parameters that are common for all sub-layers are present in the i-th hrd_parameters( ) syntax structure in the VPS extension. add_cprms_present_flag[ i ] equal to 0 specifies that the HRD parameters that are common for all sub-layers are not present in the i-th hrd_parameters( ) syntax structure in the VPS extension and are derived to be the same as the ( i − 1 )-th hrd_parameters( ) syntax structure in the VPS extension. add_cprms_present_flag[ 0 ] is inferred to be equal to 1.

### VPS VUI syntax

| vps_vui( ){ | Descriptor |
| --- | --- |
| bit_rate_present_vps_flag | u(1) |
| pic_rate_present_vps_flag | u(1) |
| if( bit_rate_present_vps_flag ¦ ¦ pic_rate_present_vps_flag ) |  |
| for( i = 0; i <= vps_number_layer_sets_minus1 NumConformancePoints; i++ ) |  |
| for( j = 0; j <= vps_max_sub_layers_minus1; j++ ) { |  |
| if( bit_rate_present_vps_flag ) |  |
| bit_rate_present_flag[ i ][ j ] | u(1) |
| if( pic_rate_present_vps_flag ) |  |
| pic_rate_present_flag[ i ][ j ] | u(1) |
| if( bit_rate_present_flag[ i ][ j ] ) { |  |
| avg_bit_rate[ i ][ j ] | u(16) |
| max_bit_rate[ i ][ j ] | u(16) |
| } |  |
| if( pic_rate_present_flag[ i ][ j ] ) { |  |
| constant_pic_rate_idc[ i ][ j ] | u(2) |
| avg_pic_rate[ i ][ j ] | u(16) |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < NumDirectRefLayers[ layer_id_in_nuh[ i ] ]; j++ ) { |  |
| tile_boundaries_aligned_flag[ i ][ j ] | u(1) |
| ilp_restricted_ref_layers_flag | u(1) |
| if( ilp_restricted_ref_layers_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < NumDirectRefLayers[ layer_id_in_nuh[ i ] ]; j++ ) { |  |
| min_spatial_segment_offset_plus1[ i ][ j ] | ue(v) |
| if( min_spatial_segment_offset_plus1[ i ][ j ] > 0 ) { |  |
| ctu_based_offset_enabled_flag[ i ][ j ] | u(1) |
| if( ctu_based_offset_enabled_flag[ i ][ j ] ) |  |
| min_horizontal_ctu_offset_plus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

## Picture level HRD parameters

For signaling of picture level HRD parameters in the buffering period SEI message, the picture timing SEI message, and the decoding unit information SEI message, either the current scalable nesting SEI message is modified such that a nested SEI message of any of these three types of SEI message can be clearly indicated applicable to which of the conformance points.

For better backward compatibility, a new SEI message is proposed, as follows.

### New scalable nesting SEI message syntax

| new_scalable_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| nesting_num_cps_minus1 | ue(v) |
| for( i = 0; i <= nesting_num_cps_minus1; i++ ) { |  |
| nesting_cp_max_tid_plus1[ i ] | u(3) |
| nesting_cp_idx[ i ] | ue(v) |
| nesting_cp_cl[ i ] | u(2) |
| } |  |
| while( !byte_aligned( ) ) |  |
| nesting_zero_bit /* equal to 0 */ | u(1) |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| } |  |

### New scalable nesting SEI message semantics

The new scalable nesting SEI message provides a mechanism to associate SEI messages with bitstream subsets corresponding to various operation points. A new scalable nesting SEI message contains one or more SEI messages.

A new scalable nesting SEI message shall not be contained in another SEI message.

A new scalable nesting SEI message shall not contain any other SEI messages than the buffering period SEI message, the picture timing SEI message, and the decoding unit information SEI message.

nesting_num_cps_minus1 plus 1 specifies the number of the following nesting_cp_idx[ i ] syntax elements. The value of nesting_num_cps_minus1 shall be in the range of 0 to 2047, inclusive. Alternatively, the value of nesting_num_cps_minus1 shall be in the range of 0 to 3071, inclusive.

The variable nestingNumCps is set equal to nesting_num_cps_minus1 + 1.

nesting_cp_max_tid_plus1[ i ] is used to specify the variable maxTemporalId[ i ]. The value of nesting_cp_max_tid_plus1[ i ] shall be greater than or equal to nuh_temporal_id_plus1 of the current SEI NAL unit. The variable maxTemporalId[ i ] is set equal to nesting_cp_max_tid_plus1[ i ] − 1.

nesting_cp_idx[ i ] specifies the index of the output layer set. The value of nesting_cp_idx[ i ] shall be in the range of 0 to 1023, inclusive.

nesting_cp_cl[ i ] specifies the conformance level of the nesting_cp_idx[ i ]-th output layer set specified by the active VPS that the SEI messages contained in this SEI messge apply to. The value of nesting_cp_cl[ i ] shall be greater than 0 and less than 3. Alternatively, the value of nesting_cp_cl[ i ] shall be less than 3.

The SEI messages contained in the new scalable nesting SEI message apply to sub-bitstreams subBitstream[ i ] for all i values in the range of 0 to nestingNumCps − 1, inclusive, where each sub-bitstream subBitstream[ i ] is the output of the sub-bitstream extraction process (see below) with the bitstream, maxTemporalId[ i ], nesting_cp_idx[ i ], and nesting_cp_cl[ i ] as inputs.

nesting_zero_bit shall be equal to 0.

### Selection of HRD parameters

An operation point is identified by an index to the target output layer set specified by the VPS, a target HighlestTid and a conformance level.

Once a target operation point is selected for bitstream conformance test, the bitsteam extraction process (see below) is invoked with the bitstream under test, HighlestTid, the indext to the target output layer set, and the conformance level as inputs, and the output sub-bitstream is assigned to BitstreamToDecode.

The hrd_parameters( ) syntax structure applicable to the target operation opoint, as specified in the VPS by hrd_output_layer_set_idx[ i ] and vps_ext_hrd_conformance_level_idx[ i ] are chosen, and the sub_layer_hrd_parameters( ) syntax structure applicable to the target operation opoint are further chosen based on the HighlestTid.

For access unit 0, the buffering period SEI message that is applicable to the target operation point (as specified by the new scalable nesting SEI message) is selected.

For each access unit in BitstreamToDecode starting from access unit 0, the picture timing SEI message that is associated with the access unit and applies to the target operarion point (as specified by the new scalable nesting SEI message) is selected, and when SubPicHrdFlag is equal to 1 and sub_pic_cpb_params_in_pic_timing_sei_flag is equal to 0, the decoding unit information SEI messages that are associated with decoding units in the access unit (as specified by the new scalable nesting SEI message) and apply to the target operation point are selected.

## Bitstream extraction process

- F.10	Sub-bitstream extraction process general

Inputs to this process are a bitstream, a target highest TemporalId value tIdTarget, and one of the following:

–	An index to output layer set outputLayerSetIdx and an bitstream extraction mode subExtModeIdc.

–	A target output layer identifier list inputOptLayerIdList an bitstream extraction mode subExtModeIdc.

–	A target layer identifier list layerIdListTarget.

If the last input inclues an index to output layer set outputLayerSetIdx and a bitstream extraction mode subExtModeIdc, the target ouput layer identifier list TargetOptLayerIdList and layerIdListTarget are derived as follows:

lsIdx = outputLayerSetIdx;
	layerIdListTarget = LayerSetLayerIdList[ lsIdx ].
	for( k = 0, j = 0; j < NumLayersInIdList[ lsIdx ];j++) {
		layerIdListTarget[ j ] = LayerSetLayerIdList[ lsIdx ][ j ]
		if( output_layer_flag[ lsIdx ][ j ] )
			outputLayerSetIdx[ k++ ] = LayerSetLayerIdList[ lsIdx ][ j ]
	}

Otherwise, if the last input inclues a target output layer identifier list inputOptLayerIdList a bitstream extraction mode subExtModeIdc, the target ouput layer identifier list TargetOptLayerIdList is set to be equal to inputOptLayerIdList and the layerIdListTarget is derived to include the nuh_layer_id of any direct or indirect dependent layer of each layer as included in TargetOptLayerIdList, unless such a layer identifier has been included. Such a process is similar as in MVC bitstream extraction process.

Otherwise, TargetOptLayerIdList is set to be empty and subExtModeIdc is set to 0.

The Sub-bitstream extraction process as specified in section F.10.1 applies with tIdTarget, layerIdListTarget and subExtModeIdc as inputs andif subExtModeIdc is not equal to 0, TargetOptLayerIdList if subExtModeIdc as an additional input.

Let the curernt bitstream has a confromance level of confLevel.

When index to output layer set or target output layer identifier list is used as an input for bitstream extraction, any extraction mode (with value equal to 0 through 2) may apply. After the extraction process, the sub-bitstream has a conformance level equal to max (subExtModeIdc, confLevel).

When a target layer identifier list is used as input for bitstream extraction, after the extraction process, the sub-bitstream has a conformance level  equal to confLevel (i.e., the conformance level doesn’t change).

Below is the sub-bitstream extraction process based on the process defined in HEVC version 1, with changes highlighed.

- F.10.1	Sub-bitstream extraction process

- Inputs to this process are a bitstream, a target highest TemporalId value tIdTarget, and a target layer identifier list layerIdListTarget, an extraction mode subExtModeIdc, and a target output layer identifier list TargetOptLayerIdList if subExtModeIdc is not equal to 0.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream of the process specified in this subclause with tIdTarget equal to any value in the range of 0 to 6, inclusive, and layerIdListTarget equal to the layer identifier list associated with a layer set specified in the active video parameter set shall be a conforming bitstream when subExtModeIdc is less than 3.

NOTE 1 – A conforming bitstream contains one or more coded slice segment NAL units with nuh_layer_id equal to 0 and TemporalId equal to 0.

The output sub-bitstream is derived as follows:

–	When one or more of the following two conditions are true, remove all SEI NAL units that have nuh_layer_id equal to 0 and that contain a non-nested buffering period SEI message, a non-nested picture timing SEI message, or a non-nested decoding unit information SEI message:

–	layerIdListTarget does not include all the values of nuh_layer_id in all NAL units in the bitstream.

–	tIdTarget is less than the greatest TemporalId in all NAL units in the bitstream.

NOTE 2 – A "smart" bitstream extractor may include appropriate non-nested buffering picture SEI messages, non-nested picture timing SEI messages, and non-nested decoding unit information SEI messages in the extracted sub-bitstream, provided that the SEI messages applicable to the sub-bitstream were present as nested SEI messages in the original bitstream.

–	Remove all NAL units with TemporalId greater than tIdTarget or nuh_layer_id not among the values included in layerIdListTarget.

–	When subExtModeIdc is greater than 0, further remove NAL units with nuh_layer_id not among the values included in TargetOptLayerIdList and with a TemporalId greater than max_tid_il_ref_pics_plus1[ LayerIdxInVps[ nuh_layer_id ] ] – 1.

–	When subExtModeIdc is greater than 1, further remove NAL units with nuh_layer_id not among the values included in TargetOptLayerIdList and discardable_flag equal to 1.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).