| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0275v6 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0092v6 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: on POC value derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that the contribution presents a design that enables non-aligned IRAP pictures across layers, is error-resilient, does not change the POC values of the base layer and keeps the POC values static during their persistence.

Version 6 of the contribution includes the following changes relative to version 5:

- The POC MSB cycle is conditionally signalled in the slice segment header extension enabling its use also in base-layer slices (whereas in version 5 the POC MSB cycle was signalled within the slice segment header for enhancement layer slices only). It is asserted that this change enables pruning of the base layer so that it only contains IRAP pictures, for which the POC difference can be greater than MaxPicOrderCntLsb / 2. Such pruning can be used for example when only IRAP pictures of the base layer may be used as references for inter-layer prediction (as indicated by max_tid_il_ref_pics_plus1[ i ][ j ]).

- The layer-tree POC MSB cycle is conditionally signalled in the slice segment header extension for additional error resilience and for enabling independent layer-tree POC derivation for each of multiple independent layers in the same layer tree.

- Inter-layer dependencies in POC and layer-tree POC derivation are constrained further as follows:

  - The POC reference picture prevTid0Pic is selected from direct reference layers (rather than from direct or indirect reference layers as in version 5).

  - A non-IRAP picture from a direct reference layer is not prevTid0Pic, when max_tid_il_ref_pics_plus1[ i ][ j ] indicates that only IRAP pictures of the reference layer may be used as references for inter-layer prediction.

  - In version 5, when a layer tree contained more than one independent layer, TID0 pictures of the independent root layer (with the smallest value of nuh_layer_id among all independent layers of the layer tree) could serve as prevTid0Pic. In version 6, pictures of an independent root layer are no longer candidates for prevTid0Pic (unless the independent root layer is the current layer or a direct reference layer for the current layer).

As a consequence, it is asserted that POC and layer-tree POC derivation have the same or a subset of the reference pictures than what inter-layer prediction has.

The proposal includes the following ingredients:

- A layer tree is a set of layers that have inter-layer dependencies with each other.

- Slice header extensions optionally contain signalling for a layer-tree POC value, which is intended to be used in the IRAP pictures of the independent layers and may be used, for additional error resilience, in other pictures too.

- A layer-tree POC value, LayerTreePicOrderCntVal is derived for each picture based on the mentioned slice header signalling, if present, and based on PicOrderCntVal differences, otherwise.

- Both the POC value and the layer-tree POC value are derived relative to the POC value and the layer-tree POC, respectively, of the previous TID0 picture on the current layer or any direct reference layer. The layer-tree POC values of all pictures in the same layer tree in the same access unit are equal. This selection of the previous TID0 picture enables down- and up-switching of the number of layers, because the referenced TID0 picture is not chosen from the same layer, when the current picture is an IRAP picture in a dependent layer (i.e. layer up-switch point).

- The function DiffPicOrderCnt( ) is specified to use LayerTreePicOrderCntVal values.

Possibilities to combine certain aspects of the POC reset approach (JCTVC-P0041v3/JCT3V-G0031v3) to this contribution and vice versa are discussed. The following proposals are made:

- PrevPicOrderCnt derivation in the POC reset approach is proposed to be modified to happen among the current layer as well as direct and indirect reference layers. This is asserted to improve loss resilience and reduce bitrate overhead.

- POC MSB signalling similar to that of JCTVC-P0041v3/JCT3V-G0031v3 has been incorporated in this contribution.

# Version History

Version 2 of the contribution included an illustrative figure and updated the specification text to handle properly an access containing a base-layer IDR picture while some enhancement layer pictures are absent as well as enhancement-layer IRAP picture POC derivation.

Version 3 of the contribution (dated 13 Nov. 2013) updates the design and the proposed specification text to address comments received during the JCT-VC/JCT-3V review as follows:

- Reference picture sets for enhancement layers are proposed to use layer-wise picture order count to avoid the first ue(v)-coded delta POC value in the reference picture set syntax to be a relatively long codeword and to reduce the likelihood of having to use the MSB cycle signalling to differentiate between long-term pictures.

- To avoid the vulnerability of the loss of the enhancement-layerTID0 picture preceding a base-layer IDR picture in decoding order in the derivation of the layer-wise POC values correctly, the layer_delta_pic_order_cnt_minus1 syntax element in the slice header was replaced by layer_pic_order_cnt_lsb, which provides the LSBs of layer-wise POC value. This LSB value is used for layer-wise POC derivation similarly to how slice_pic_order_cnt_lsb is used for POC derivation.

It is asserted that consequently the proposed scheme is tolerant of losses of enhancement-layer TID0 pictures as long as there is a received TID0 picture for which ( prevLayerPicOrderCntLsb − layer_pic_order_cnt_lsb ) results into the same half of the total range of 0 to MaxPicOrderCntLsb – 1, inclusive, regardless of whether prevLayerPicOrderCntLsb is assigned to be the LayerPicOrderCntLsb of the received TID0 picture or the TID0 picture that the encoder used for the layer-wise POC derivation.

- Version 3 fixes a bug in the previous versions where PicOrderCntMsb was not correctly updated for the cases in which:

  - A first access unit contains a base-layer IRAP picture with NoRaslOutputFlag equal to 1 but no pictures in some enhancement layer(s) and later access units, in decoding order, precede the first access unit in output order.

  - A first access unit contains a base-layer IRAP picture with NoRaslOutputFlag equal to 1 but no pictures in an enhancement layer and the first picture of the enhancement layer is contained in an  access unit for which the PicOrderCntMsb value is greater than 0. This case could happen, for example, in the adaptive resolution change (ARC) use case.

The bug was fixed by assigning prevTid0Pic for the first picture within an enhancement layer to be the previous picture in decoding order that has TemporalId equal to 0 and nuh_layer_id equal to 0 or any nuh_layer_id value among the direct and indirect reference layers of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture.

Version 3 of the contribution also provides a summary table how POC or layer-wise POC is used for different POC-related functionalities and describes how the contribution performs the agreed evaluation criteria of multi-layer POC derivation.

Version 4 of the proposal included the following ingredients:

- A layer tree is defined to consist of one independent layer (without any reference layers) and all layers that directly or indirectly use the independent layer as reference.

- Slice header extensions optionally contain signalling for a layer-tree POC value, which is intended to be used in the IRAP pictures of the independent layers and may be used, for additional error resilience, in other pictures too.

- A layer-tree POC value, LayerTreePicOrderCntVal is derived for each picture based on the mentioned slice header signalling, if present, and based on PicOrderCntVal differences, otherwise.

- Both the POC value and the layer-tree POC value are derived relative to the POC value and the layer-tree POC, respectively, of the previous TID0 picture on the current layer or any lower layer in the same layer tree on which the decoding of the current picture may depend on. The layer-tree POC values of all pictures in the same layer tree in the same access unit are equal. This selection of the previous TID0 picture enables down- and up-switching of the number of layers, because the referenced TID0 picture is not chosen from the same layer, when the current picture is an IRAP picture in a dependent layer (i.e. layer up-switch point).

- The function DiffPicOrderCnt( ) is specified to use LayerTreePicOrderCntVal values.

Possibilities to combine certain aspects of the POC reset approach (JCTVC-O0213/JCT3V-F0073) to this contribution and vice versa were discussed in version 4. The following proposals were made:

- PrevPicOrderCnt derivation in the POC reset approach is proposed to be modified to happen among the current layer as well as direct and indirect reference layers. This is asserted to improve loss resilience.

- If it is preferred to include base-layer IDR/BLA picture loss detection means in the slice segment header syntax, enhancement-layer slice headers in this contribution can contain POC resetting information. This signalling shall be used for a picture that is in a different layer tree than the layer tree containing the base layer and which is the first picture, in decoding order, in a layer tree after a base-layer IDR or BLA picture. The signalling may also be used in other pictures for additional error resilience.

Version 5 of the contribution updated the layer tree definition so that multiple independent layers in the same layer tree are enabled. A layer tree index is derived for each layer and identifies which layer tree the layer belongs to.

# Introduction

It is asserted that the poc_reset_flag design (JCTVC-P0041v3/JCT3V-G0031v3) has the following implications:

- RASL pictures should be additionally constrained as follows: In HEVC v1 there is a requirement that any RASL picture associated with a CRA picture shall follow, in output order, any IRAP picture that precedes the CRA picture in decoding order. In the fourth AHG10 teleconference it was pointed out that outputting all the pictures in the DPB that are marked as "needed for output" at the beginning of the first access unit of a POC resetting period would cause an additional constraint on RASL pictures. That is, any RASL picture associated with a CRA picture shall follow, in output order, any picture that precedes the CRA picture in decoding order.

- The POC values are no longer static. This would complicate the use of POC values to label pictures in post-processing operations, which is required for example to conclude the persistence of some SEI messages.

  - Background: the persistence of the following SEI messages is specified similarly to the following: [the message pertains until] “the PicOrderCntVal of the next slice to be decoded is greater than currPicOrderCntVal + pic_order_cnt_delta, where currPicOrderCntVal is the value of PicOrderCntVal of the picture in the access unit containing the SEI message”

    - Recovery point SEI message

    - Progressive refinement start SEI message

    - Region refresh information SEI message

  - The structure of pictures information SEI message also has similar persistence than the SEI messages named above and additionally uses POC within the persistence scope to identify pictures.

  - It has been proposed in JCTVC-O0213v4/JCT3V-F0073v4 that the use of the above-mentioned SEI messages would be constrained in scalable bitstreams so that the persistence would not cross a POC resetting period. However, such a constraint would disable the use of these SEI messages in a flexible manner. For example, a region refresh information SEI message of the base layer would not be allowed to span for multiple POC resetting periods, even if the POC reset were caused by an independent layer, e.g. an auxiliary picture layer.

- The use of encoding tools related to these SEI messages would be constrained according to the POC resets caused by other layers, which may be for example auxiliary picture layers or other independent layers. For example, a recovery point for the base layer could not be indicated, if a picture in another layer resets the POC values in between the current picture (for which the SEI message is present) in the recovery point. Similarly, a gradual decoding refresh period for the base layer cannot be indicated if another layer resets POC values within the period. This is an undesirable limitation of the encoder flexibility as the other layers may be independent of the base layer and may be coded separately from the base layer (e.g. the other layers may be auxiliary picture layers added to the bitstream after the whole base layer has been already encoded).

  - Additionally, pan-scan rectangle, tone mapping information, frame packing arrangement, and display orientation SEI messages cease to persist when a picture with a greater POC value than that of the current picture are output.

- It is unclear how non-static POC values are used with feedback-based error recovery. PicOrderCntVal of a picture changes its value dynamically in the encoder and in the decoder. If a picture picA that were identified by PicOrderCntVal in a feedback message, picA no longer has the same value of PicOrderCntVal in the encoder if a POC reset happened since picA in decoding order.

- In JCTVC-P0041v3/JCT3V-G0031v3 the signalling of POC MSB (poc_msb_val) is required for the EL CRA pictures to enables layer down- and up-switching. It is noted that when poc_msb_val takes part in POC derivation, each IRAP picture in all layers, including the base layer, shall have poc_msb_val present for correct POC derivation. That is because the base layer decoding can start from any IRAP picture (by setting NoRaslOutputFlag and NoClrasOutputFlag equal to 1). As the successive EL CRA pictures, in decoding order, include an absolute POC MSB value, an absolute POC MSB value needs to be signaled for the base layer BLA and CRA pictures too in order to have the POC values aligned across layers.

- As explained in the previous paragraph, poc_msb_val is present when the current picture is a BLA or CRA picture. However, according to JCTVC-P0041v3/JCT3V-G0031v3, if the first picture in an enhancement layer is not a BLA or CRA picture, its POC MSB is set to zero (when poc_reset_idc is equal to 1 or 2 for the picture) or derived through LSB differences relative to POC MSB equal to zero (when poc_reset_idc is equal to 3). Consequently, POC values would not be aligned across layers. Thus, an additional constraint is needed to have poc_msb_val present for the first picture in each enhancement layer following a base layer BLA or CRA picture in decoding order.

- The first picture in each enhancement layer following a base layer BLA or CRA picture in decoding order may precede the BLA or CRA picture in output order. Consequently, negative poc_msb_val may need to be signaled and the syntax should be appended with the sign of poc_msb_val or change to use se(v) for poc_msb_val.

- An approach that POC values can be derived from their reference layers has been presented in this contribution and, earlier, as part of JCTVC-O0275v4/JCT3V-F0092v4 and its attachment "PrevPicOrderCnt change to JCTVC-O0213v4_JCT3V-F0073v4_attachment.doc" relative to the POC reset approach. It is asserted that POC MSB derivation from reference layers would significantly reduce the number of pictures where the poc_msb_val has to be included and hence reduce the bitrate overhead without sacrificing loss robustness. poc_msb_val would have to be present only for independent-layer IRAP pictures rather than IRAP pictures in any layers or pictures aligned with base-layer IRAP pictures.

- It is unclear which constraints encoders have to follow when determining the start of a new POC resetting period. Should encoders establish a new POC resetting period whenever there is an IRAP picture in any layer?

A design that would not change the POC values of the base layer and would keep the POC values static is targeted in this contribution.

It is remarked that PicOrderCntVal is used in the decoding process only as input to the DiffPicOrderCnt( ) function. This opens the possibility to re-specify the DiffPicOrderCnt( ) function to use other inputs than PicOrderCntVal.

# Overview of the Proposal

In the example of Figure 3, there is access unit with an IDR picture in the base layer, which resets the POC value to 0. The layer-tree POC values are anyhow continued by including layer_tree_poc_lsb with the base-layer IDR picture. In this example layer_tree_poc_lsb is equal to 10, and since the derived layer-tree POC for the POC reference picture is 1032 and consequently the layer-tree POC LSB is equal to 8 (when MaxPicOrderCntLsb is equal to 16), the layer-tree POC value is incremented by 2 for the base-layer IDR picture.

Figure 3. An example of layer-tree POC signalling and derivation.

Table 1 attempts to summarize whether this contribution proposed to use POC or layer-wise POC for different POC related functionalities.

Table 1. A summary of the use of POC and layer-tree POC for different POC related functionalities.

| POC-related functionality | POC or layer-tree POC used? How? |
| --- | --- |
| Decoding process (e.g. motion vector scaling in TMVP) | No changes in the base-layer decoding. For decoding of enhancement layers, the difference of layer-tree POCs is used. For decoding of the enhancement layers, the function DiffPicOrderCnt( picA, picB ) is specified to use layer-tree POCs. |
| Access unit boundary determination | It is not clear if POC related information is needed for AU boundary detection, see JCTVC-P0043/JCT3V-G0032. If POC related information is needed for AU boundary detection, it is sufficient to use pic_order_cnt_lsb, while PicOrderCntVal can likewise be used. Consecutive pictures having the same pic_order_cnt_lsb are contained in the same access unit. An AU boundary can be concluded when i) there is a picture with first_slice_segment_in_pic_flag equal to 1 and nuh_layer_id equal to 0, or ii) slice_pic_order_cnt_lsb differs in adjacent pictures. |
| Reference picture set signalling | POC for the base layer. Layer-tree POC for enhancement layers. |
| Picture output order determination in the bumping process of the HRD | POC. CVSs are output in their decoding order. Within a CVS, the output order of access units is determined through the POC value. (See Section 4.4 for more details.) |
| SEI message semantics Use of SEI messages by post-processing units | POC for the base layer. Layer-tree POC for enhancement layers. (See Section 4.4 for more details.) |
| Feedback based error recovery | POC for the base layer. Layer-tree POC for enhancement layers. (See Section 4.5 for more details.) |

The detailed proposal is indicated with change marks in the accompanying specification text document.

# Evaluation of the Proposal

## Evaluation Criteria

The agreed evaluation criteria for multi-layer POC derivation proposals as reported in the BoG report JCTVC-O0349/JCT3V-F0248 are copied below:

- Solution does not cause any problems in the no loss case, including sub-bitstream extraction and layer switching

- Understanding of what picture coding patterns were disallowed

- Error resiliency aspects: prioritize good POC behavior for typical use cases, and prioritizing when the decoded pictures themselves were likely to be useful

  - POC corruption caused by picture loss should be detectable

  - POC problems shouldn’t persist beyond sample accuracy problems, after a loss

- Specification consistency

- POC values can be determined for feedback based error recovery (classified as an additional evaluation criteria)

The proposal is asserted to meet the evaluation criteria favorably as described in the following sub-sections.

## Constraints in Picture Coding Patterns

It is asserted that there are no known constraints on picture coding patterns. Considerations on the following scenarios have been given:

- Unaligned IRAP pictures across layers, IDR picture in the base layer:

- Unaligned IRAP pictures across layers, IDR picture in the enhancement layer:

- Unaligned IRAP pictures across layers, layer down-switching followed by layer up-switching at an IDR picture in the enhancement layer:

- Only base-layer IRAP pictures used as references for inter-layer prediction, removal of non-IRAP pictures from the base layer.

Encoders have two options, either to signal POC MSB for the base-layer IRAP pictures or to make slice_pic_order_cnt_lsb codeword longer such that the POC interval between adjacent IRAP pictures is less than or equal to MaxPicOrderCntLsb / 2. These options are illustrated below.

- EL picture not present in an access unit with BL IDR picture

- Two independent layers and a dependent enhancement layer in the same layer tree

- Two independent layers, which are never present in the same access unit

## Error Resiliency

It is asserted that the derivation of LayerTreePicOrderCntVal is similarly loss-tolerant as the derivation of PicOrderCntVal for the base-layer pictures.

## Specification Consistency

- Use with SEI messages the semantics of which depend on picture order count values:

  - According to the contribution PicOrderCntVal and LayerTreePicOrderCntVal are static (i.e. are required to be derived only when the picture is decoded and need not be updated when later pictures are decoded).

  - SEI messages applying to base-layer pictures or entire access units are proposed to use PicOrderCntVal in their semantics (i.e. no change to version 1 is needed).

  - It is proposed that SEI messages with nuh_layer_id greater than 0 or included in a scalable nesting SEI message use LayerTreePicOrderCntVal in their semantics when the SEI message applies to enhancement-layer pictures.

  - Consequently, it is asserted that the semantics of SEI messages are unambigious and post-processing units handling the SEI messages need not update the POC values of past pictures (according to POC derivation of later pictures) but can rely on the POC values of the pictures at the time they are decoded.

- Determination of output order:

  - The determination of output order basically follows the base-layer output order determination. In other words, access units of a CVS are contiguous in output order.

  - The output order determination is first based on the CVS order, i.e. all access units of an earlier CVS are output before any access unit of a later CVS. Within a CVS, the access unit output order is determined by PicOrderCntVal. All the pictures of an access unit (with PicOutputFlag equal to 1) are output contiguously.

  - The proposed text for the output order determination for the HRD is included in the accompanying document "JCTVC-O0275 DPB operation (relative to JCTVC-O0217_JCT3V-F0075_Changes to Spec E1004-v6).doc".

## Use with Feedback Based Error Recovery

It is proposed that PicOrderCntVal is used for identifying base-layer pictures and LayerTreePicOrderCntVal is used for identifying enhancement-layer pictures in the feedback messages.

As the PicOrderCntVal and LayerTreePicOrderCntVal are static, their use in labeling of pictures in the feedback messages is unambiguous. Furthermore, as PicOrderCntVal and LayerTreePicOrderCntVal are derived in a loss-tolerant manner, the correct interpretation of the feedback messages (by the encoder that receives the feedback messages) is asserted to be guaranteed.

For example, the Reference Picture Selection Indication (RPSI) payload of RTP/AVPF (IETF RFC 4585) could be specified for MV-HEVC/SHVC streams as follows:

The field "Native RPSI bit string defined per codec" is a base16 [RFC4648] representation of the 8 bits consisting of 2 most significant bits equal to 0 and 6 bits of nuh_layer_id, as defined in [HEVC], followed by the 32 bits representing the value of the PicOrderCntVal (in network byte order) when nuh_layer_id is equal to 0 or the 32 bits representing the value of the LayerTreePicOrderCntVal (in network byte order), as defined in [HEVC], for the picture that is requested to be used for reference when encoding the next picture.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).