| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0093 |
| --- | --- |

| Title: | 3D-CE3.h: Results on simple merge candidate list construction for 3DV |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gun Bang, *Young Su Heo, * Yoon Jin Lee, *Gwang Hoon Park, Gwang Soon Lee, Namho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-6791 {gbang, gslee, namho} @etri.re.kr +82-31-201-3680 {* medist, * rainmake, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution presents the results of 3D-CE3.h simple merge candidate list construction for 3DV proposed in the JCT3V-E0213[1]. The JCT3V-E0213 proposed a simplified and clarified method for constructing MCL (Merge Candidate List) for dependent view or depth. The proposed method is a simple and low-complexity way to implement the MCL construction module for the 3D-HEVC by re-using the existing MCL construction module of the HEVC. The simulation results of the texture and the synthesized BD performance show that the proposed process has the same coding performances compared to the anchor 3D-HTM version 8.0, respectively. Whereas, the merge process in 3D-HEVC is far-simplified and also systematically-clarified as a role of enhancement layer codec in a view of the codec implementation.

# Introduction

In the coding system of current 3D-HEVC, the independent base-view for the texture is encoded using the base encoder, e.g., original HEVC module. And the depth map and the dependent-views for the texture are encoded using the 3DV enhancement encoder, e.g., 3D-HEVC module, as shown in Fig. 1.

The 3D-HEVC module has been integrated with many tools that can offer the additional benefits for the high coding-efficiency on the 3D videos, without considering H/W implementation comparability and constraints on the existing HEVC module. Therefore, on the other hand, the 3D-HEVC specific modules make more complex coding structure than the original HEVC. It significantly increases the computational complexity and H/W implementation complexity. This design may be un-suitable in terms of H/W implementation, since the chip for 3D-HEVC module should be re-designed and re-implemented.

Fig. 1. Coding system of the current HTM (3D-HEVC)

This contribution proposes to clarify the merge candidate list (MCL) construction process by simplifying the current 3D-HEVC MCL for the enhancement texture view and the corresponding depth map.

There exist two kinds of the MCL construction process, as shown in Fig. 2. One is the MCL construction process for the independent (base) texture view. The other is the MCL construction process for the dependent views and depth map. The MCL construction process for the dependent views and depth map is more complexly-combined than the MCL construction process for the independent texture view.

Fig. 2. Current MCL construction process for the current 3D-HEVC

In the MCL construction process for the current 3D-HEVC, new merging candidates with high complexity for 3D-videos have been added in the MCL process of HEVC. And there exists the dependencies between merging candidate, e.g., combined-bi merging candidates using VSP information of the neighboring blocks.

Fig. 3. Current MCL construction process in 3D-HEVC

Figure 3 shows the current MCL construction process in the 3D-HEVC. For the dependent views, the inserting processes for the extra candidates (green-squared) are added and also the traditional processes (yellow-squared) in HEVC should be changed. Therefore, the implementation should be duplicated in the traditional candidates for the enhancement views and the corresponding depth map. This design may not be efficient for the implementation of the added and combined MCL process for 3D-HEVC in terms of H/W implementation, since the chip for 3D-HEVC module should be re-designed and re-implemented independent with the base view MCL construction process.

This contribution proposes a low-complexity way to implement the MCL construction process for the 3D-HEVC by re-using existing MCL construction module in the HEVC, as shown in Fig. 4. This design is suitable for implementing the new MCL processes, since the existing MCL construction module in the HEVC can be re-used.

Fig. 4 Proposed MCL construction process

For the dependent views (or the enhancement views), the MCL construction process in HEVC (shown in Fig. 5) is used without any changes. Then as shown in Fig. 6, the additional MCL construction processes are carried out with the MCL1 and AvailableFlagN as inputs.

Fig. 5. MCL construction process in HEVC

Fig. 6. Proposed inserting process for extra candidates for dependent view

Following is a description of the inserting process for the extra candidates for the dependent views.

- Check the VSP flags of the spatial merging candidates (SMC) A1, B1, B0, A0 and B2 in the merge candidate list (MCL1).

- If the current PU is in the depth map, the texture candidate(T) which is derived from the corresponding texture is identified. If the current PU is in the texture, the disparity vector is derived and the corresponding block of the current PU based on the disparity vector is identified. For T or IvMC, if these are different from all candidates in the list, and derived to be available, is inserted to the head of the merge candidate list.

- The inter-view disparity vector candidate (IvDC), which is converted from the disparity vector, is inserted to the back of the valid candidate from IvMC to B0 if these are different from all candidates in the list.

- If the BVSP is enabled for the current slice, the BVSP merging candidate is inserted to the back of the last valid candidate from IvMC to IvDC,.

- If the MCL is not full, the zero motion vectors are filled as a default.

In this contribution, the proposed merge process has been simplified and optimized in comparison with JCT3V-E0213 [1] as shown in Table 1.

- When texture candidate, temporal inter-view candidate and disparity inter-view candidate are inserted to the MCL, the left and the above candidates are only used for the redundancy checking. Therefore, even for the worst case, no additional operations are required for the redundancy checking.

- Consequently, the coding loss is reduced from 0.1% to 0.0% (NO Coding Loss).

Table 1. A comparison between E0213 and F0093.

|  | Complexity |  |  | Coding results |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Additional operations | Additional memory access | Etc. | Video only | Video /total BR | Synthesized/ total BR |
| E0213 vs. HTM 7.0 | - 6 redundancy check - Less calculations on virtual candidates generation (20→12) | No additional memory access | - Early-termination is not allowed | 0.1% | 0.1% | 0.1% |
| F0093 vs. HTM 8.0 | - 0 redundancy check - Less calculations on virtual candidates generation (20→12) | No additional memory access | Provide 2-step early-termination (implementation issue) | 0.0% | 0.0% | 0.0% |

# Simplified Scheme

As mentioned in the section 1, the proposed process has been simplified and optimized. In addition, the new candidates, which was adopted by JCT3V-E0126[2] at the last meeting, are also considered in the contribution.

When the merge candidate list (MCL) construction process is invoked, the conventional HEVC MCL construction process is performed. Herein, if the current picture is on the dependent view, the following steps are performed (the parts modified from JCT3V-E0213 are highlighted in gray (removed) and yellow (added)):

- Check VSP flags of spatial merging candidates (SMC) A1, B1, B0, A0 and B2 which are in the merge candidate list.

- If the current PU is in the depth map, the texture candidate(T) which is derived from the corresponding texture is identified. If the current PU is in the texture, the disparity vector is derived and the corresponding block of the current PU based on the disparity vector is identified. For T or IvMC, if these are different from all candidates in the list the A1 and B1, and derived to be available, is inserted to the head of the merge candidate list.

- The inter-view disparity vector candidate (IvDC) which is converted from the disparity vector is inserted to the back of the last valid candidate from IvMC to B0, if these are different from all candidates in the list the A1 and B1.

- If the BVSP is enabled for the current slice, the BVSP merging candidate is inserted to the back of the last valid candidate from IvMC to IvDC.

- If IvMC is derived to be the additional candidate, it is compared with the IvMC in step 2. If it is not equal to the IvMC in step 2 or if the additional candidate is a DSMV (when the additional IvMC is unavailable), the generated additional candidate is inserted to the back of the last valid candidate from IvMC to B2 (due to JCT3V-E0126 [2])

- If the MCL is not full, the zero motion vectors are filled as a default.

## Clarification

Grey represents the marked text to be removed, and green represents the marked text to be inserted.

- Reusability of the HEVC is increased.

  - Derivation process for spatial merge candidates in HEVC is reused as shown in Fig. 7.

Fig. 7. Proposed derivation process for spatial merge candidates

  - Derivation process for combined bi-predictive merging candidates is reused as shown in Fig. 8.

Fig. 8. Proposed derivation process for combined bi-predictive merging candidates

- Simplified WD text.

- Easy implementation via reusing the existing HEVC codec module.

## Simplification

The following description shows the simplified parts of the derivation process for merging candidates in this contribution:

- Combined bi-predictive candidates (JCT3V-E0213)

  - As shown in Table 2 and Table 3, the number of combining for the virtual candidates in the worst case can be reduced from 20 to 12 by reusing HEVC process. It has no loss on the entire coding efficiency while the number of operation is reduced. In addition, current 3D-HEVC checks whether candidates is BVSP-coded or not, and then BVSP-coded candidates are not used for combination. By reusing HEVC process, checking the BVSP flags is removed from the derivation process for combined bi-predictive merging candidates.

Table 2. Current table for combined bi-predictive candidates

| combIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L0CandIdx | 0 | 1 | 0 | 2 | 1 | 2 | 0 | 3 | 1 | 3 | 2 | 3 | 0 | 4 | 1 | 4 | 2 | 4 | 3 | 4 |
| L1CandIdx | 1 | 0 | 2 | 0 | 2 | 1 | 3 | 0 | 3 | 1 | 3 | 2 | 4 | 0 | 4 | 1 | 4 | 2 | 4 | 3 |

Table 3. Proposed table for combined bi-predictive candidates (Same to the HEVC)

| combIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L0CandIdx | 0 | 1 | 0 | 2 | 1 | 2 | 0 | 3 | 1 | 3 | 2 | 3 |
| L1CandIdx | 1 | 0 | 2 | 0 | 2 | 1 | 3 | 0 | 3 | 1 | 3 | 2 |

# Analysis of early-termination

Figure 10 shows the schematic diagram of the proposed method. Figure 10(a) shows the existing 3D-HEVC MCL module design and also Fig. 10(b) shows the proposed MCL module design.

(a)     							    (b)

Fig. 9. Proposed merge process

Fig. 10. Hard-wired scheme for the proposed merge process

For the H/W implementation, the hard-wired scheme might be used to boost coding speed as shown in Fig. 10. Parallel process may be applied as follows;

(1) In the derivation process (DPBC module) for basic candidates such as {A1, B1, Bo, A0, B2, Col}, the data of all candidates are ready to be processed and then the derivation are performed in parallel before they are inserted into the MCL0 module.

(2) For the extra candidates such as {IvMC, IvDC, BVSP, DSMV}, the data of all extra candidates are also ready to be processed and then the derivation are parallelly-performed in the DPEC module before they are inserted into the MCLD module.

(3) The merging candidates derived from DPBC module are inserted into the MCL0 module and then the MCL for view 0 is came out.

(4) Both the merging candidates derived from DPEC module and the MCL produced from MCL0 module, as well as the data for the basic candidates are inserted into the MCLD module, and then finally the MCL for view 1 or 2 are generated.

The early-termination can never be happened because every candidate is not derived sequentially in the parallelized H/W implementation, as shown in (1) and (2). Therefore, the early-termination is the minor implementation issue of this proposed method.

For the sequential process, the proposed method provides 2-step early termination. 1st early termination is utilized on insertion process of conventional HEVC candidates, and 2nd early termination is utilized on insertion process of 3DV candidates. Compared to early termination provided by current 3D-HEVC, negligible number of derivation is additionally needed in the 2-step early termination.

# Simulation Results

Table 4 shows the simulation results of the proposed method. Common test conditions specified in JCT3V-E1100[4] is used for the evaluation. The simulation results show that the proposed process method has the same coding performances in aspect of texture and synthesized BD performances. Also, the encoding time and the decoding time are almost same to the anchor.

Table 4. Simulation results (proposed vs. HTM 8.0)

# Conclusions

In this contribution, the clarified and simplified MCL construction process for the dependent texture view and the corresponding depth map is proposed. The simulation results show that the proposed algorithm has the same coding performances in comparison with the HTM 8.0, while providing the MCL construction process for 3D-HEVC much simpler and also clarify the hardware implementation to reduce the complexity, significantly.

# Reference

[1]	G. Bang, Y. S. Heo, K. Y. Kim, G. H. Park, G. S. Lee, N. Hur, “3D-CE3.h releated: Simple merge candidate list construction for 3DV”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E0213, 5th Meeting, July, 2013.

[2] V. Thirumalai, L. Zhang, Y. Chen, M. Karczewicz, “CE3.h: Merge candidates derivation from vector shifting”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E0126, 5th Meeting, July, 2013.

[3] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1001, 5th Meeting, July, 2013.

[4] D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1100, 5th Meeting, July, 2013.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).