| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-F0100 |
| --- | --- |

| Title: | [AHG11] Super Multiview Videos in NICT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Information and Discussion |  |  |
| Author(s) or Contact(s): | Mehrdad Panahpour Tehrani Takanori Senoh Makoto Okui Kenji Yamamoto Naomi Inoue | Tel: Email: | mehrdad.ptehrani@nict.go.jp senoh@nict.go.jp okui.m@nict.go.jp k.yamamoto@nict.go.jp na-inoue@nict.go.jp |
| Source: | NICT |  |  |

_____________________________

# Abstract

This document introduces some videos that we have captured or generated for SMV (Super-Multi-View) 3D display.

# Introduction

Super/ultra-multiview 3D displays are emerging widely, anticipated as the next generation of auto-stereoscopic display.

Several displays are currently being developed, such as 3DTV by MERL[2], REI (Ray Emergent Imaging) by NICT [3], Holovizio by Holographica [4], SMV256 by Tokyo University of Agriculture and Technology [5], Light Field 3D Display by Samsung[6], IP (Integral Photography) by NHK [7] and 3D VIVANT[8], Projection type IP by Seoul National University display [9], 360 degree LFD by USC [10], Seelinder by Nagoya University and The University of Tokyo [11],  Holo Table by Holy Mine [12], fVisiOn by NICT [13], Walk-through by KDDI[14], etc.

Since these super multiview 3D displays require a huge number of multi-view images in real-time, a new coding standard is essential to realize their services and products in the market [15]. In 105th MPEG a new AhG on FTV was established [1] to address these new use cases, and their coding standard.

NICT has developed REI. REI is fed by about 200 full HD images per frame to enhance the image relity of conventional HDTV or current 3D cinema. The viewpoint can be freely chosen by relocating in front of the display, as if we relocate in front of a real object. Life-sized objects as humans and cars can be fit or rendered in the 16:9, 200-inch large screen display. REI provides dense and fluid horizontal motion parallax.  For this display we have generated and captured several videos that we briefly introduce them in this document.

Previously, in 105th MPEG meeting [16] [17], NICT content has been provided, i.e. two videos “shark” and “microworld” to JCT-3V. These test sequences contain only 3-view and 3-groundtruth-depth-map.

# Super multiview videos

The super multiview videos can be categorized into three types that are shown in the following. All videos have some common parameters as follows. Figure 1 shows how the viewpoints of camera are configured.

- Resolution: 1960 x 1080 pixel (can be converted to 1920x1088)

- Color: 24 bit RGB image (can be converted to 420 YUV)

- Frame rate: 60 fps

- Camera setup: 1D parallel

Figure 1   Viewpoint from cameras

## Type 1: Generated by Computer Graphic Tools

This type of test sequence is generated by computer graphic (CG) tools. The generated test sequences have nearly 200views, 1D convergent, and ground truth depth maps are also available. Figure 2 and Figure 3 show examples of this type of test sequences.

Figure 2   Multiview Image Test Sequence: Snapshot of multiview images generated by CG tools

Figure 3   Multiview Video+Depth Test Sequence: Snapshot of multiview images generated by CG tools

## Type 2: Captured by a Moving Still-Camera

A controlled moving camera setup is used to capture the object from different angles, while it can horizontally move on a rail, and the viewpoints remain convergent by rotating the camera around the optical center. The camera interval and converging point of all viewpoints can be set to the desired distance and angle, respectively. The camera baseline for this set up is 3.4m. Several images were captured by this device, and mostly contain about 200 still views. Figure 4 shows the moving camera stage. Figure 5 shows snapshots of several images.

Figure 4   Capturing system for the moving camera

Figure 5   Multiview Image Test Sequence: Snapshot of multiview images captured by a moving camera

## Type 3: Captured by Movie-Camera Array

Two set of array was used for capturing, which are 64 and 32 camera arrays. The camera array is synchronized, calibrated and multiview video are rectified. The specification of the videos are listed below. Figure 6 shows the camera arrays. Figure 7 shows snapshot of several videos captured by the camera arrays.

The raw camera parameters are as follows:

- Resolution: 1960 x 1080 pixels

- Color: 10 bits YCrCb 4:2:2

- Frame rate: 60 fps

- Camera Movement: 1D convergent

- 64 cameras array:

  - Camera interval: 5.5cm

  - Distance to Object: 4.5m   or  6.0m

- 32 cameras array:

  - Camera interval: 5.5cm

  - Distance to Object: 1.3m

Figure 6   Capturing system of camera array

Figure 7   Multiview Video Test Sequence: Snapshot of multiview images captured by camera array

# Summary

This document briefly reviewed several videos that NICT has captured or generated for SMV displays.

# References

[1] “List of AHGs Established at the 105th Meeting in Vienna, Austria”, ISO/IEC JTC1/SC29/WG11/w13649, July-Aug. 2013.

[2] W. Matusik, H. Pfister, “3D TV: A Scalable System for Real-Time Acquisition, Transmission and Autostereoscopic Display of Dynamic Scenes”, ACM SIGGRAPH, ISSN: 0730-0301, Vol. 23, Issue 3, pp. 814-824, August 2004.

[3] S, Iwasawa, M. Kawakita, and N. Inoue, “REI: an automultiscopic projection display,” Proc. 3DSA 2013, Selected paper 1, Jun (2013).

[4] http://www.holografika.com/

[5] Y. Takaki and N. Nago, "Multi-projection of lenticular displays to construct a 256-view super multi-view display," Opt. Express 18, 8824-8835 (2010).

[6] J. Lee, J. Park, D. Nam, S. Young Choi, D. Park, C. Y. Kim, “Optimal Projector Configuration Design for 300-Mpixel Light-Field 3D Display”, SID Symposium Digest of Technical Papers, Volume 44, Issue 1, pages 400–403, June 2013.

[7] J. Arai, F. Okano, M. Kawakita, M. Okui, Y. Haino, M. Yoshimura, M. Furuya, and M. Sato, “Integral Three-Dimensional Television Using a 33-Megapixel Imaging System,” JOURNAL OF DISPLAY TECHNOLOGY, VOL. 6, NO. 10, pp.422-430, OCTOBER 2010.

[8] JCT3V-C0210:  3D Holoscopic Video Test Material, FP7 3D VIVANT Consortium, Jan. 2013.

[9] Hong J, Kim Y, Park SG, Hong JH, Min SW, Lee SD, Lee B., "3D/2D convertible projection-type integral imaging using concave half mirror array.", Opt Express. 2010 Sep 27;18(20):20628-37. doi: 10.1364/OE.18.02062

[10] http://gl.ict.usc.edu/Research/3DDisplay/

[11] T. Yendo, T. Fujii, M. Tanimoto, M. Panahpour Tehrani, “The Seelinder: Cylindrical 3D display viewable from 360 degrees”, Journal of Visual Communication and Image Representation (2010) ,Volume: 21, Issue: 5-6, Publisher: Elsevier Inc., Pages: 586-594

[12] http://www.holymine3d.com/prod/prod03.html

[13] Shunsuke Yoshida, “fVisiOn: glasses-free tabletop 3D display to provide virtual 3D media naturally alongside real media,” Proc. SPIE 8384, Three-Dimensional Imaging, Visualization, and Display 2012, 838411 (1-9), Apr. 25, 2012.

[14] A. Ishikawa, M. Panahpour Tehrani, S. Naito, S. Sakazawa A. Koike, “Free Viewpoint Video Generation for Walk-through Experience Using Image-based Rendering”, Proceeding MM '08 Proceedings of the 16th ACM international conference on Multimedia Pages 1007-1008.

[15] Mehrdad Panahpour Tehrani, Takanori Senoh, Makoto Okui, Kenji Yamamoto, Naomi Inoue, Toshiaki Fujii, Hiroya Nakamura, “Proposal to Consider A New Work Item and Its Use Case - REI : An Ultra-multiview 3D Display -”, ISO/IEC JTC1/SC29/WG11/m30022, July-Aug. 2013.

[16] S. Shimizu, S. Sugimoto, and H. Kimata, “AHG11: 3D Test Materials from NICT-3D data set,” JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-C0101, Jan 2013, Geneva, Switzerland.

[17] S. Sugimoto, and S. Shimizu, “AHG11: NICT-3D test sequences”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-E0210, July 2013, Vienna, Austria.

# Patent rights declaration(s)

NICT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).