| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-F0101 |
| --- | --- |

| Title: | [AHG15] Introduction of Super Multiview Video Systems for Requirement Discussion |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Information and Discussion |  |  |
| Author(s) or Contact(s): | Mehrdad Panahpour Tehrani Takanori Senoh Makoto Okui Kenji Yamamoto Naomi Inoue Toshiaki Fujii | Tel: Email: | mehrdad.ptehrani@nict.go.jp senoh@nict.go.jp okui.m@nict.go.jp k.yamamoto@nict.go.jp na-inoue@nict.go.jp fujii@nuee.nagoya-u.ac.jp |
| Source: | NICT, Nagoya University |  |  |

_____________________________

# Abstract

This document proposes to the new AhG on FTV [1] to proceed with standardizing a new codec that can be used for most of super-multiview systems. In this document, we briefly review each super multiview displays or technologies. Each of these systems is targeting different application and use case scenario. We briefly summarize our suggestion on how the new AhG [1] may proceed toward finding a new standard for compression of the contents for SMV technologies.

# Introduction

Super/ultra-multiview 3D displays are emerging widely, anticipated as the next generation of auto-stereoscopic display.

Several displays are currently being developed, such as 3DTV by MERL[2], REI (Ray Emergent Imaging) by NICT [3], Holovizio by Holographica [4], SMV256 by Tokyo University of Agriculture and Technology [5], Light Field 3D Display by Samsung[6], IP (Integral Photography) by NHK [7] and 3D VIVANT[8], Projection type IP by Seoul National University display [9], 360 degree LFD by USC [10], Seelinder by Nagoya University and The University of Tokyo [11],  Holo Table by Holy Mine [12], fVisiOn by NICT [13], Walk-through by KDDI[14], etc.

Since super multiview 3D displays require a huge number of multi-view images in real-time, a new coding standard is essential to realize their services and products in the market [15]. In 105th MPEG a new AhG on FTV was established to address these new use cases, and their coding standard.

# Super-Multiview Video

This section briefly explains the system listed above. Furthermore, we categorize them into two groups based on the way the 3D content is displayed or provided to the users.

## Group 1: Front View 3D Display

### 3D TV by MERL [2]

MERL developed a 3D TV prototype system with real-time acquisition, transmission, and 3D display of dynamic scenes. It is a distributed, scalable architecture to manage the high computation and bandwidth demands. The system consists of an array of cameras, clusters of network-connected PCs, and a multi-projector 3D display. Multiple video streams are individually encoded and sent over a broadband network to the display. The 3D display shows high-resolution (1024x768) stereoscopic color images for multiple viewpoints without special glasses. The system is implemented with rear-projection and front-projection lenticular screens.

Figure 1. 3DTV Prototype by MERL [2]

### REI by NICT [3]

REI (Ray Emergent Imaging) is fed by about 200 full HD images per frame to compete with conventional HDTV or current 3D cinema [1][2]. Users have natural 3D depth impression without using special glasses. The viewpoint can be freely chosen by relocating in front of the display, as if we relocate in front of a real object. Such life-sized objects as humans and cars can be fit or rendered in the 16:9, 200-inch large screen display. REI provides dense and fluid horizontal motion parallax and limited flipping effect appears in the observed images. Currently, the system contains full capture to display chain. A new hardware codec for the system is under development that competes with current standards.

Figure 2. REI (Ray Emergent Imaging) by NICT [3]

### Holovizio by Holographica [4]

HoloVizio shows 3D images in which spatial points of the image are addressed individually. Therefore, position of the displayed 3D object does not depend on the viewing position but it remains fixed in the space. The system main specifications are 180 degrees FOV, and continuous motion parallax.

Figure 3.  Holovisio by Holographica [4]

### SMV256 by Tokyo University of Agriculture and Technology [5]

A new super multi-view (SMV) display system that enables the number of views to be increased is developed by Takagi Lab. All three-dimensional (3D) images generated by multiple multi-view flat-panel displays are superimposed on a common screen using a multi-projection system. The viewing zones of the flat-panel 3D display are produced in the pupils of the projection lenses and then imaged to the observation space by a screen lens. Sixteen flat-panel 3D displays having 16 views were used to construct a SMV display having 256 views. The 3D resolution was 256 × 192. The screen size was 10.3 inches. The horizontal interval of the viewing zones was 1.3 mm.

Figure 4.  SMV (Super Multi-View) display by Tokyo University of Agriculture and Tech [5]

### Light Field 3D Display by Samsung [6]

Samsung developed a projection-based 300-Mpixel light-field display that has a 100-inch screen and a 40° viewing angle. The projector configuration is modified to achieve a high-quality 3D image, by analyzing the luminance characteristics of the display and optimizing the design to minimize the variation in the brightness of projected images.

Figure 5. Light Field 3D Display by Samsung [6]

### IP (Integral Photography) by NHK [7]

NHK developed integral three-dimensional (3D) television using an ultra high definition imaging system. The system uses a device having 7680 pixels in the horizontal direction and 4320 pixels in the vertical direction for each of the red, green, and blue channels. A lens array comprising 400 lenses is configured in the horizontal direction and one comprising 250 lenses is configured in the vertical direction. The system is designed to ensure a maximum spatial frequency of 11.3 cycles/degree and a viewing angle of 24 degrees when the display is observed from three times the display height. The setup has simultaneously maintained the balance between the maximum spatial frequency and the viewing angle by shortening the focal length of the elemental lens while narrowing the pitch of the elemental lens.

Figure 6.  IP (Integral Photography) by NHK [7]

### IP by 3D VIVANT [8]

3D holoscopic imaging, first proposed by G. M. Lippmann in 1908 [4], relies on the use of a series of micro-lenses at the picture surface to create the impression of depth and provide the user with full motion parallax without requiring special glasses. The process uses an array of small spherical micro-lenses, known as a “fly’s eye” lens array, to both record and display the 3D holoscopic image. At the display side, this array is designed so that when viewed from different angles, different images will be visible. This happens because, for a given direction, only one pixel from each micro-image is visible by the user. This technology can be used for creating 3D images on a flat panel display. If motion pictures are considered, instead of still images, this results in 3D holoscopic video. At the capture side, the operation is basically the same, but the micro-lens array is applied to the sensor.

Figure 7.  IP by 3D VIVANT[8]

### Projection type IP by Seoul National University display [9]

This system implements 3D/2D convertible feature in the projection-type integral imaging by using concave half mirror array. The concave half mirror array has the partially reflective characteristic to the incident light. And the reflected term is modulated by the concave mirror array structure, while the transmitted term is unaffected. With such unique characteristic, 3D/2D conversion or even the simultaneous display of 3D and 2D images is also possible. Experiments verified the 3D/2D conversion and the display of 3D image on 2D background with the fabricated prototype. This system can be useful for the 3D industry such as the 3D theatres.

Figure 8.  Projection type IP by Seoul National University display [9]

## Group 2: All-around 3D Display

### 360 degree LFD by USC  [10]

The system contains set of rendering techniques for an autostereoscopic light field display that is able to present interactive 3D graphics to multiple simultaneous viewers, 360 degrees around the display. The display consists of a high-speed video projector, a spinning mirror covered by a holographic diffuser, and FPGA circuitry to decode specially rendered DVI video signals. The display uses a standard programmable graphics card to render over 5,000 images per second of interactive 3D graphics, projecting 360-degree views with 1.25 degree separation up to 20 updates per second.

Figure 9. 360 degree LFD by USC [10]

### Seelinder by Nagoya University and The University of Tokyo [11]

The basic structure of the display is shown in Figure 2. It has two spinning cylinders, one inside the other. The outer cylinder is a parallax barrier that has a series of vertical slits spinning rapidly while the inner cylinder spins slower in the opposite direction with a series of one-dimensional LED arrays on its surface. If the slit width of the outer cylinder is sufficiently small, the light through the slit becomes a thin flux, whose direction is scanned rapidly by the spinning of the outer cylinder. By synchronously changing the intensity of each LED in the arrays to the spinning, rays of different direction have different colours with time-multiplexing. Moreover, the inner cylinder also rotates slower, and the LED array’s position is a little different when the next slit comes, and so rays are shot to each direction from each position. In this way, a cylindrical light field display is realized and can be displayed.

Figure 10.  Seelinder by Nagoya University and The University of Tokyo [11]

### Holo Table by Holy Mine [12]

A a portable 360-degree viewable 3D display, “Holo-Table” was developed. It displays more than 500 views with 1024x768 pixels at 30-60 fps. The feature of this display is not only large number of views but also high density of views. Because of these features, users can see a 3D scene from any directions with smooth motion parallax.

Figure 11.  Holo Table by Holy Mine [12]

### fVisiOn by NICT [13]

fVisiOn is a novel glasses-free tabletop 3D display, i.e. floating virtual 3D objects appearing on a flat tabletop surface. The developed novel technique can float standing 3D image on a blank flat tabletop surface, and allow multiple viewers to observe the 3Dfrom omnidirection of 360° in seated condition. It is designed to be a friendly interface for multiple users for varied tabletop tasks by featuring our glasses-free method and observation style. For generation of the 3D images, fVisiOn employs a newly developed special optical device as a screen and a series of micro projectors arranged circularly. The combination of those devices reproduces a light field in a certain volume on the table. The entire 3D imaging mechanism is installed underneath the table. fVisiOn can display virtual 3D images beside printed documents and physical mock-ups. This primal prototype can float the 3D images of a height of approximately 5 cm on the tabletop surface like a centrepiece in the center of the table.

Figure 12.  fVisiOn by NICT [13]

# Use Cases

We considered several applications for each of the systems. We would like to initiate discussion on requirements that can meet most of the available display technologies. SMV displays use cases can be as follows: Public Viewing of Sport Events such as Olympic and World Cups, Future Cinema, Theme Park, Education, Medicine, Car Show Rooms, Science Signage, Product design, Education, Surgery, Security.

# Super Multiview System

In order realize the use cases mentioned above, a full chain of capturing / transmission / display is required. In such applications, the whole 3D content shall be sent to the user at once. Since such applications have a large viewing field, the object of interest shall be capture correspondingly from a large viewpoint. Therefore, the captured content has a large number of viewpoints. Current standard or on-going standards for 3D video coding, are only dealing with contents that have a limited number of viewpoints. Therefore, there is an immediate need to start working on a new standard that address such a use case.

MVD (Multiview Video + Depth) compression can be an initial candidate. However, due to large scale of the system, the data size for compression is still huge. Therefore, use of other data formats seems to be emerging. Currently, survey on alternative 3D format is being progressed by AhG on Alternative 3D Formats, indicating the possibility for new 3D video representation. We believe a better 3D format shall be investigated to realize such use case.

In this scenario, we would consider a system as follows. The pre-processor task is to capture the scene, and perform geometry/color corrections followed by depth estimation. At this stage the input data format to encoder is MVD. In between, and at the encoder side, the data conversion unit performs a data conversion for SMV applications. The compression unit, compress the contents given the novel data format. At the decoder side the invers process are performed, i.e. decompression and inverse data conversion. The post-processor task is to perform related geometry/color correction, followed by virtual view synthesis, for perfect displaying of the contents by the display.

Figure 14. Total system configuration

The new coding standard shall enable the low latency and real-time capture / transmission / Display of the content with a large multiview video data.

# Discussion and Our Suggestions for Discussion on Requirements

In this document, we briefly discussed several state-of-arts of 3D display technologies. Depending on each application, or use case, the capturing format, data representation shall be corresponded to the display technology. In order to achieve a standard that meets most of use cases, we propose to include following work items or tasks in during the discussions on requirements for SMV coding technologies.

- Informative Technologies:

  - Required technology for capturing the contents.

  - New Depth estimation, and view synthesis technologies (better than current DERS, and VSRS).

  - Required technology for translating the transmitted contents for a type display to another type of display.

- Normative Technologies:

  - Suitable 3D formats.

  - New Coding technology for the 3D format in (2.a)

# References

[1] “List of AHGs Established at the 105th Meeting in Vienna, Austria”, ISO/IEC JTC1/SC29/WG11/w13649, July-Aug. 2013.

[2] W. Matusik, H. Pfister, “3D TV: A Scalable System for Real-Time Acquisition, Transmission and Autostereoscopic Display of Dynamic Scenes”, ACM SIGGRAPH, ISSN: 0730-0301, Vol. 23, Issue 3, pp. 814-824, August 2004.

[3] S, Iwasawa, M. Kawakita, and N. Inoue, “REI: an automultiscopic projection display,” Proc. 3DSA 2013, Selected paper 1, Jun (2013).

[4] http://www.holografika.com/

[5] Y. Takaki and N. Nago, "Multi-projection of lenticular displays to construct a 256-view super multi-view display," Opt. Express 18, 8824-8835 (2010).

[6] J. Lee, J. Park, D. Nam, S. Young Choi, D. Park, C. Y. Kim, “Optimal Projector Configuration Design for 300-Mpixel Light-Field 3D Display”, SID Symposium Digest of Technical Papers, Volume 44, Issue 1, pages 400–403, June 2013.

[7] J. Arai, F. Okano, M. Kawakita, M. Okui, Y. Haino, M. Yoshimura, M. Furuya, and M. Sato, “Integral Three-Dimensional Television Using a 33-Megapixel Imaging System,” JOURNAL OF DISPLAY TECHNOLOGY, VOL. 6, NO. 10, pp.422-430, OCTOBER 2010.

[8] JCT3V-C0210:  3D Holoscopic Video Test Material, FP7 3D VIVANT Consortium, Jan. 2013.

[9] Hong J, Kim Y, Park SG, Hong JH, Min SW, Lee SD, Lee B., "3D/2D convertible projection-type integral imaging using concave half mirror array.", Opt Express. 2010 Sep 27;18(20):20628-37. doi: 10.1364/OE.18.02062

[10] http://gl.ict.usc.edu/Research/3DDisplay/

[11] T. Yendo, T. Fujii, M. Tanimoto, M. Panahpour Tehrani, “The Seelinder: Cylindrical 3D display viewable from 360 degrees”, Journal of Visual Communication and Image Representation (2010) ,Volume: 21, Issue: 5-6, Publisher: Elsevier Inc., Pages: 586-594

[12] http://www.holymine3d.com/prod/prod03.html

[13] Shunsuke Yoshida, “fVisiOn: glasses-free tabletop 3D display to provide virtual 3D media naturally alongside real media,” Proc. SPIE 8384, Three-Dimensional Imaging, Visualization, and Display 2012, 838411 (1-9), Apr. 25, 2012.

[14] A. Ishikawa, M. Panahpour Tehrani, S. Naito, S. Sakazawa A. Koike, “Free Viewpoint Video Generation for Walk-through Experience Using Image-based Rendering”, Proceeding MM '08 Proceedings of the 16th ACM international conference on Multimedia Pages 1007-1008.

[15] Mehrdad Panahpour Tehrani, Takanori Senoh, Makoto Okui, Kenji Yamamoto, Naomi Inoue, Toshiaki Fujii, Hiroya Nakamura, “Proposal to Consider A New Work Item and Its Use Case - REI : An Ultra-multiview 3D Display -”, ISO/IEC JTC1/SC29/WG11/m30022, July-Aug. 2013.

[16] S. Shimizu, S. Sugimoto, and H. Kimata, “AHG11: 3D Test Materials from NICT-3D data set,” JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-C0101, Jan 2013, Geneva, Switzerland.

[17] S. Sugimoto, and S. Shimizu, “AHG11: NICT-3D test sequences”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-E0210, July 2013, Vienna, Austria.

# Patent rights declaration(s)

NICT, Nagoya University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).