| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0108 |
| --- | --- |

| Title: | 3D-CE4: Results on improved advanced residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Jian-Liang Lin2, Yu-Lin Chang2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, jl.lin, yulin.chang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of JCT3V-E0175 and JCT3V-E0185 in 3D-HEVC. First, JCT3V-E0175 presents an adaptive disparity vector (DV) derivation method for advanced residual prediction (ARP). Three DV candidates can be selected to find more accurate pseudo residues. Second, JCT3V-E0185 proposes advanced temporal residual prediction, where the concept of ARP is also applied to disparity compensated prediction. Experimental results show that a combination of the two proposed methods can achieve 0.6% BD-rate saving for the synthesized views in the common test condition.

# Introduction

In the current 3D-HEVC [1], advanced residual prediction (ARP) [2] is adopted to replace the simple inter-view residual prediction for its better coding efficiency. In ARP, the base view is considered as a mirror of the current view. A mirror block is located with an estimated disparity vector (DV) and a mirror reference block is found for the mirror block with the same motion vector (MV) as the current block. Pseudo residues are generated by subtracting the reconstructed samples in the mirror block and in the mirror reference block. Then pseudo residues are used to predict the real residues of the current block. Fig. 1 depicts a sketch of ARP. In ARP, a coding unit (CU) can choose to use pseudo residual prediction or not. If pseudo residual prediction is used, pseudo residues can be weighted by ½ or not. The encoder can find the best ARP choice for each CU according to the rate-distortion optimization (RDO) criterion, and signal the choice in the bit-stream.

It plays a critical role to locate the mirror block in the base-view accurately since pseudo residues are similar to the real residues only if they are generated by similar samples. In ARP, the estimated DV is derived by a neighbouring block disparity vector (NBDV) procedure. The first encountered DV from a temporal or spatial neighbouring block is utilized by ARP as the estimated DV for the current block. Since such an estimated DV is not accurate enough, pseudo residual prediction may be inefficient in some cases.

Fig. 1.  A sketch of ARP, where PR represents pseudo residue.

# Proposed method

2.1 Adaptive disparity vector derivation

An adaptive disparity vector derivation (ADVD) method is proposed to improve ARP coding efficiency. In ADVD, three DV candidates are derived from temporal/spatial neighbouring blocks. Only two spatial neighbours are checked as depicted in Fig. 2. A new DV candidate is appended into the list only if it is not equal to any DV candidate already in the list. If the DV candidate list is not fulfilled after exploiting neighbouring blocks, default DVs will be added. An encoder can find the best DV candidate used in ARP according to RDO criterion, and signal the index to the decoder.

For further improvement, aligned temporal DV (ATDV) is proposed as a new DV candidate. ATDV is obtained from the aligned block, which is located by a scaled MV to the collocated picture, as depicted in Fig. 3. Two collocated pictures are utilized, which are also used in the NBDV derivation. ATDV is checked before DV candidates from neighbouring blocks when it is used.

2.2 Advanced temporal residual prediction

An advanced temporal residual prediction (ATRP) method is proposed for the disparity compensated prediction. In ATRP, the disparity parameters of the current PU are applied to the corresponding block in a temporal reference picture in the same view to generate the pseudo residual in temporal direction. The corresponding block in the temporal reference picture is located by a derived motion vector (DMV). DMV is the MV of the block which the current DV is pointing to in the reference view. The concept of proposed ATRP is illustrated in Fig. 4, and the derivation of DMV is illustrated in Fig. 5.

Fig. 2 Spatial neighboring blocks used to derive DV/MV candidates.

Fig. 3 Aligned temporal DV and temporal DV.

Fig. 4 The concept of the proposed ATRP.

Fig. 5 Derivation of DMV.

2.3 The combination of ADVD and ATRP

When ADVD and ATRP are combined, three MV candidates are derived for ATRP just like three DV candidates are derived for ARP in ADVD. DMV is put into the MV candidate list if it can be found. Then spatial/temporal neighbouring blocks are checked to find more MV candidates like finding a merging candidate. Also, only two spatial neighbours are checked as depicted in Fig. 2. If the MV candidate list is not fulfilled after exploiting neighbouring blocks, default MVs will be added. An encoder can find the best MV candidate used in ATRP according to RDO criterion, and signal the index to the decoder, similar to what is done in ADVD for ARP.

# Experimental results

The proposed methods are integrated on HTM-8.0 [3]. All the experiments are conducted under the common test condition [4]. To evaluate the proposed methods more clearly, five experiments are tested and Table 1 shows the configurations. Table 2-Table 6 demonstrates the performance of test 1-test 5 respectively.

From Table 2 and Table 3, it can be observed that ADVD without ATDV can achieve 0.3% BD-rate saving for the synthesized views and the number goes up to 0.4% when ½ weighting is removed. These results are expectable. Because ½ weighting introduces additional overhead bits, but it is not so useful when the estimated DV is accurate enough.

Table 4 shows that ATRP itself can contribute 0.2% BD-rate saving for the synthesized views.

In Table 5, BD-rate saving for the synthesized views is still 0.4% when ATDV is added as a new candidate for ADVD. But the BD-rate saving on V1/V2 is improved from 1.2% to about 1.5%.

Finally, Table 6 demonstrates a combination of ADVD with ATDV and ATRP. It can be seen that the gains are additive, resulting in 0.6% BD-rate saving for the synthesized views.

Table 1. Configurations for the 5 tests.

|  | ADVD | ATDV | ½ weighting | ATRP |
| --- | --- | --- | --- | --- |
| Test 1 | On | Off | On | Off |
| Test 2 | On | Off | Off | Off |
| Test 3 | Off | Off | On | On |
| Test 4 | On | On | Off | Off |
| Test 5 | On | On | Off | On |

Table 2. Performance of test 1

Table 3. Performance of test 2

Table 4. Performance of test 3

Table 5. Performance of test 4

Table 6. Performance of test 5

# Conclusion

In this proposal, an adaptive disparity vector derivation method and an advanced temporal residual prediction method are proposed to improve ARP. Experimental results show that 0.6% BD-rate saving for the synthesized views can be achieved by these two together.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1001, Aug, 2013.

- L. Zhang, Y. Chen, X. Li, M. Karczewicz, “CE4: Advanced residual prediction for multiview coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0177, April, 2013.

- HTM-8.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1100, Aug, 2013.