| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0109 |
| --- | --- |

| Title: | 3D-CE1.h related: A simplified block partitioning method for view synthesis prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang, Kai Zhang, Jicheng An, Jian-liang Lin, Shawmin Lei 1MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, jl.lin, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a simplified block partitioning method for view synthesis prediction (VSP) in 3D-HEVC. In the current 3D-HEVC, the partition size (4x8 or 8x4) is determined for each 8x8 block in a PU. It is proposed to determine the partition size only once for a PU. Experimental results show that the proposed method causes no coding efficiency loss while the partition size determination process for VSP can be simplified.

# Introduction

The current 3D-HEVC [1] adopts view synthesis prediction (VSP) for texture video coding in dependent viewsError: Reference source not found. VSP locates the corresponding virtual depth for the current PU with neighbouring disparity vector (NBDV) at first. Then prediction signals are generated with a disparity vector derived from the virtual depth for each partition in the PU.

An adaptive method proposed in JCT3V-E0207 [2] is adopted to partition PU in VSP. Each 8x8 block can be partitioned into two 8x4 partitions, or two 4x8 partitions independently as depicted in Fig. 1. To determine the partitioning way, a determination process is conducted for each 8x8 block in the PU. The current design involves two problems.

- The determination process is invoked several times for a PU when the PU size is larger than 8x8. For example, if the PU size is 64x64, the determination process will be invoked 64 times.

- The texture memory access is irregular over a PU, which is unfriendly to paralleling process.

- To avoid the switch between depth access and texture access, it is a common-used implementation to store the flags indentifying the partition for 8x8 blocks in additional memory before the prediction signals generation.

Figure 1.  An example of partitioning structure for a 32x32 PU in 3D-HEVC.

# Proposed method

To address the problems above, it is proposed to determine the partition size only once for a PU in VSP. The whole PU is partitioned into 8x4 partitions only, or it is partitioned into 4x8 partitions only as depicted in Fig. 2. To determine the partitioning way, a horSplitFlag is calculated for the PU as,

horSplitFlag = (( refDepPels[LT] > refDepPels[RB] ) = = ( refDepPels[RT] > refDepPels[LB] ))

where refDepPels[X] represents corresponding depth values at 4 corners of the current PU as depicted in Fig. 3.	If horSplitFlag is equal to 1, the current PU is partitioned into 8x4 partitions; otherwise, it is partitioned into 4x8 partitions.

Figure 2.  Partitioning structures for a 32x32 PU as proposed.

Figure 3.  Four corners of the current PU.

# Experimental result

The proposed method is built on the top of the reference software HTM-8.0 [3]. And the experiments are conducted according to the common test condition [4]. Error: Reference source not found shows simulation results of the proposed block partitioning method. It can be seen that there is no coding loss for video 1, video 2 or the synthesis views.

Table 1.  The results of the proposed method under CTC

TABLE 1 shows a summary of the partition determination complexity analysis. The details can be found in the attached EXCEL sheet. The analysis shows that the complexity is reduced significantly when the PU size is larger than 8x8.

TABLE 1: Complexity analysis

TABLE 13 shows a summary of the memory cost. The details can be found in the attached EXCEL sheet. The analysis shows that the memory cost is reduced significantly when the PU size is larger than 8x8.

TABLE 3: Memory Cost Analysis for the Flags Indentifying Partition

# Conclusion

In this contribution, VSP with either 4x8 or 8x4 partition for the whole PU is proposed. In this method, the partitioning way for a PU in VSP is determined by analyzing depth values at four corners of the PU. There are no overall performance losses, and decoder complexity is reduced by removing some computational comparison and calculation operations. We therefore recommend adopting the proposed method.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1001, July, 2013.

- Shinya Shimizu, Shiori Sugimoto, “3D-CE1.h: Adaptive block partitioning for VSP,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E0207, July, 2013.

- HTM-8.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0

- D. Rusanovksyy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-E1100, July, 2013.