- H.8.5.4.2 Derivation process for a disparity sample array

- Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view order index refViewIdx specifying a reference view,

- a view order index depthViewIdx specifying the view the depth should be derived from

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit.

- variable nSubBlkW and nSubBlkW specifying the conversion precision of the corresponding depth samples,

Outputs of this process are:

- a (nPSW)x(nPSH) array disparitySamples of disparities values.

Let refDepPels be an array of reconstructed depth samples of the depth view component with ViewIdx equal to depthViewIdx. The luma location (xTL, yTL) of top-left luma sample of a block in refDepPels is derived by

  - xTL = xP + ( ( mvDisp[ 0 ] + 2 )  >>  2 )		(Error: Reference source not found-1)
yTL = yP + ( ( mvDisp[ 1 ] + 2 )  >>  2 )		(Error: Reference source not found-2)

    - The variables xP0, yP0, xP1, yP1, are derived as specified in the following:

- xP0 = Clip3( 0, pic_width_in_luma_samples – 1, xTL)
yP0 = Clip3( 0, pic_height_in_luma_samples – 1, yTL) 
xP1 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + nPSW – 1)
yP1 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + nPSH – 1)

- The variable nSubSubBlkW is set equal to nSubBlkW and the variable nSubSubBlkH is set equal to nSubBlkH.

- When splitFlag is equal to 1, nSubSubBlkW and nSubSubBlkW are modified as follows:

  - The variable horSplitFlag is derived as specified in the following.

    - horSplitFlag = ( refDepPels[ xP0 ][ yP0 ] >  refDepPels[ xP1 ][ yP1 ] )
	  = =  ( refDepPels[ xP1 ][ yP0 ] > refDepPels[ xP0 ][ yP1] ) )	(Error: Reference source not found-3)

    - nSubSubBlkW = horSplitFlag ? nSubSubBlkW : ( nSubSubBlkW  >>  1 )	 (Error: Reference source not found-4)

    - nSubSubBlkH = horSplitFlag ? ( nSubSubBlkH  >>  1 ) : nSubSubBlkW	 (Error: Reference source not found-5)

The array disparitySamples of size (nPSW)x(nPSH) is derived as specified in the following:

- For sBy in the range of 0 to ( ( nPSH / nSubBlkH) –1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nPSW / nSubBlkW) –1 ), inclusive, the following applies:

    - The variables xB, y,B, xP0, yP0, xP1, yP1, are derived as specified in the following:

xB = sBx * nSubBlkW
			yB = sBy * nSubBlkH
			xP0 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xB )
			yP0 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + yB ) 
			xP1 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xB + nSubBlkW – 1 )
			yP1 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + yB + nSubBlkH – 1 )

    - The variable nSubSubBlkW is set equal to nSubBlkW and the variable nSubSubBlkH is set equal to nSubBlkH.

    - When splitFlag is equal to 1, nSubSubBlkW and nSubSubBlkW are modified as follows:

      - The variable horSplitFlag is derived as specified in the following.

        - horSplitFlag = ( refDepPels[ xP0 ][ yP0 ] >  refDepPels[ xP1 ][ yP1 ] )
	  = =  ( refDepPels[ xP1 ][ yP0 ] > refDepPels[ xP0 ][ yP1] ) )	(Error: Reference source not found-6)

        - nSubSubBlkW = horSplitFlag ? nSubSubBlkW : ( nSubSubBlkW  >>  1 )		(Error: Reference source not found-7)

        - nSubSubBlkH = horSplitFlag ? ( nSubSubBlkH  >>  1 ) : nSubSubBlkW		(Error: Reference source not found-8)

    - The derivation process for a disparity sample block as specified in subclause Error: Reference source not found is invoked with the luma location ( xB, yB ), variables nSubBlkW and nSubBlkH, the array of reconstructed depth samples refDepPels, the luma location ( xTL, yTL ), the variables nSubSubBlkW and nSubSubBlkW, the view order index refViewIdx, and the array disparitySamples as the inputs, and the output is the modified array disparitySamples.