| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0110 |
| --- | --- |

| Title: | 3D-CE3: Sub-PU level inter-view motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Kai Zhang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, kai.zhang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution confirmed the coding efficiency of the SPIVMP (sub-PU level inter-view motion prediction) method firstly proposed in JCT3V-E0184 on top of HTM8.0. The experimental results show that the proposed SPIVMP method using 8x8 sub-PU can achieve 5.9% and 6.0% BD-rate savings for video 1 and video 2, respectively, and 1.3% BD-rate saving for synthesized views with no encoding time increase and 2.9% decoding time increase.

# Introduction

The basic concept of the inter-view prediction of motion parameters is illustrated in Fig. 1. For deriving the motion parameters of temporal inter-view motion vector candidate (TIVMC) for a current prediction unit (PU) in a dependent view, a disparity vector (DV) is derived for the current PU [2][3]. By adding the DV to the middle position of the current PU, a reference sample location is obtained. The prediction block in the already coded picture in the reference view that covers the sample location is used as the reference block. If this reference block is coded using motion compensated prediction (MCP), the associated motion parameters can be used as the TIVMC for the current PU in the current view.

Fig. 1.  The concept of inter-view motion prediction

The corresponding area in the reference view may have plentiful motion information; however, only the motion information in the middle position of the area is used for the current PU in the dependent view. That will decrease the performance.

# Proposed method

In light of the aforementioned problem, a sub-PU level inter-view motion prediction (SPIVMP) method was proposed in JCT3V-E0084 [1] as shown in Fig. 2. The temporal inter-view merge candidate is derived as follows.

- First, divide the current PU into multiple sub-PUs with a smaller size.

- Second, add the DV to the middle position of each sub-PU to obtain a group of reference sample locations, and for each reference sample location, a prediction block in the already coded picture in the reference view that covers that sample position is used as a reference block.

- Third, for each reference block, if it is coded using MCP, the associated motion parameters can be used as candidate motion parameters for the corresponding sub-PU of the current PU. The TIVMC of the current PU is composed of the candidate motion parameters of all the sub-PUs.

Fig. 2.  The concept of proposed SPIVMP

The sub-PU size is signalled in video parameter set (VPS). In VPS, one syntax element log2_sub_PU_size_minus2 is added. The sub-PU size is derived as follows:

Sub-PU width = Sub-PU height = 2( log2_sub_PU_size_minus2+ 2 )

# Experimental results

The proposed SPIVMP has been implemented on top of HTM8.0, and the experiments are conducted following the common test conditions [4]. Sub-PU sizes equal to 4x4, 8x8, and 16x16 have been tested, and the results are shown in Table 1, Table 2, and Table 3 respectively.

Table 1.  Coding performance of SPIVMP with sub-PU size equal to 4x4

Table 2.  Coding performance of SPIVMP with sub-PU size equal to 8x8

Table 3. Coding performance of SPIVMP with sub-PU size equal to 16x16

# Conclusion

This contribution confirmed the coding efficiency of the SPIVMP (sub-PU level inter-view motion prediction) method firstly proposed in JCTVC-E0084 on top of HTM8.0. The experimental results show that the proposed method using 8x8 sub-PU size can achieve 5.9% and 6.0% BD-rate saving for video 1 and video 2, and 1.3% BD-rate saving for synthesized views with no encoding time increase and 2.9% decoding time increase. It is suggested to adopt this method to the next version of HTM.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. An, K. Zhang, J.-L. Lin, S. Lei, “CE3.h related: Sub-PU level inter-view motion prediction,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E0184, August, 2013.

- L. Zhang, Y. Chen, M. Karczewicz, “CE5.h: Disparity vector generation results,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-A0097, July, 2012.

- Y.-L. Chang, C.-L. Wu, Y.-P. Tsai, S. Lei, “CE1.h: Depth-oriented neighboring block disparity vector (DoNBDV) with virtual depth retrieval,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0131, January, 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1100, August, 2013.