| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0111 |
| --- | --- |

| Title: | 3D-CE1: Simplified view synthesis prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen1, Jian-Liang Lin1, Na Zhang3, Jicheng An2, Kai Zhang2, Shawmin Lei1 1MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2 MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 3 Department of Computer Science and Technology, Harbin Institute of Technology, Harbin, China 150001 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, jicheng.an, kai.zhang, shawmin.lei} @mediatek.com hitnzhang@hit.edu.cn |
| Source: | MediaTek Inc, Harbin Institute of Technology |  |  |

_____________________________

# Abstract

This contribution presents the results of CE1 experiments related to the simplification of view synthesis prediction (VSP) proposed in JCT3V-E0171. In HTM-8.0, a PU may need to access multiple depth blocks because the NBDV used by DoNBDV could be different from the NBDV used in VSP. Moreover, NBDV and the VSP mode flags are currently needed to be kept in a line memory to be accessed by the CUs at lower coding tree unit (CTU) row through VSP inheritance. In JCT3V-E0171, two simplifications were proposed to reduce the memory bandwidth of the depth data access and to remove the line memory used to store the NBDV and VSP mode flags. Experimental results reportedly show that both of the simplifications introduce no overall BD-rate change compared to HTM-8.0.

# Introduction

In HTM-8.0 [1], VSP uses the DV derived by NBDV process (we will denote this DV as NBDV in the following context) to locate a depth block for performing the backward warping prediction. In the current design, when a spatial neighbor of the current PU is coded as VSP mode, the NBDV and VSP mode could both be inherited from the spatial neighbor. Then the NBDV of the spatial neighbor will be used to fetch a depth block in the depth image of the reference view for performing VSP. In addition, depth-oriented NBDV (DoNBDV) also needs the NBDV of current CU to fetch a depth block to refine the DV. As a result, it may need to access different depth blocks in different reference views for performing VSP process and depth-oriented NBDV (DoNBDV) of the current PU as shown in Fig. 1.

Fig. 1.  The depth data accessed by DoNBDV and VSP in HTM-8.0

Moreover, since VSP could be inherited through the spatial merging candidates, a line memory is needed to store the BVSP mode flags and the NBDVs of neighboring blocks belonging to the upper CTU row (also termed LCU: largest coding unit).

# Proposed schemes

Two simplifications for VSP were proposed in last meeting [2] and have been further studied in CE1 [3]:

- Unify the depth data accessed by DoNBDV and VSP

If a spatial neighbor is coded using VSP mode, the spatial merging candidate derived from the spatial neighbor also uses the derived NBDV of the current CU instead of the NBDV of the spatial neighbor to fetch a depth block in the reference view. In this way, the depth data accessed by DoNBDV and VSP are unified as shown in Fig. 2. With this simplification, the memory bandwidth of accessing depth data is reduced.

Fig. 2.  The depth data accessed by DoNBDV and the proposed VSP

- Reduce memory requirement for NBDV and BVSP mode flag

A spatial neighbor coded with VSP mode is referred as a VSP candidate for spatial merging candidate derivation only when the spatial neighbor belongs to the current CTU row. If the spatial neighbor coded with VSP mode crosses the CTU row boundary, it will be considered as a common DCP candidate. With this simplification, the line buffer used to store the NBDV and VSP mode flag could be removed.

# Reduction of the memory access and memory buffer

The section gives an analysis about the saving of worst case memory access and the reduction of the memory buffer for the proposed two simplifications.

In HTM-8.0, when the picture is divided into 8x8 CUs with 2NxN partitioning and each PU is coded as VSP mode which is inherited from the spatial neighboring block, each PU may refer to different depth blocks in which one is for DoNBDV and the other one is for VSP. The fetched depth pixels for coding a 1920X1080 picture would be 1920x1080x2 = 4147200 pixels. Since the proposed first simplification unifies the depth block used for DoNBDV and VSP, the fetched depth blocks required for each PU is reduced from two to one and the fetched depth pixels for coding a 1920x1080 picture is reduced from 4147200 to 2073600 pixels which is 50% reduction of the bandwidth to access depth data.

In HTM-8.0, in cases that the NBDV of current CU could be inherited by the following CUs/PUs of below CTU row, NBDVs and VSP mode flags are required to be stored in the unit of smallest CU (8x8) and smallest PU (4x4), respectively. The line buffer of the NBDV for a 1920x1080 sequence would be (1920/8) x (32+5) + (1920/4) x 1=9360bits. With the proposed simplifications, the line buffer can be total removed without degrading the coding performance.

# Experimental results

The proposed two simplifications have been integrated into HTM-8.0 [1], and all the simulations are conducted under common test conditions [4]. The detailed experimental results of simplification 1, simplification 2 and the combined results are shown in Table 1, Table 2, and Table3, respectively. It can be seen that both of the simplifications introduce no overall BD-rate change compared to HTM-8.0.

Table 1.  Experimental results of the proposed simplification 1

Table 2.  Experimental results of the proposed simplification 2

Table 3.  Experimental results of the combination of two proposed simplifications

# Conclusion

This contribution reports the CE1 test results of two simplifications on VSP. Both of the simplifications introduce no overall BD-rate change compared to HTM-8.0 while the memory bandwidth and memory requirement are reduced.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Harbin Institute of Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- HTM-8.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0

- N. Zhang, Y.-W. Chen, J.-L. Lin, J. An, K. Zhang, S. Lei, S. Ma, D. Zhao, W. Gao, “3D-CE1.h related: Simplified view synthesis prediction,” JCT3V-E0171, July, 2013.

- S. Shimizu, “Description of Core Experiment 1 (CE1) on View Synthesis Prediction,” JCT3V-E1101, July, 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1100, July, 2013.