- The modifications related to clipping alignment are highlighted in Green

- The modifications related to reference picture selection are highlighted in purple

- H.8.5.2.2.6 Bilinear sample interpolation and residual prediction process

The process is only invoked if res_pred_flag is equal to 1.

Inputs to this process are:

- a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,prediction list utilization flags, predFlagL0 and predFlagL1,

- the prediction list indication X,

- the prediction list utilization flag predFlagLX,

- the reference index refIdxLX,

- the motion vectors mvLX, mvCLX

Output of this process are:

- the (nPbW)x(nPbH) array predSamplesLXL,

- the (nPbW / 2)x(nPbH / 2) arrays predSamplesLXCb and predSamplesLXCr.

- If ( DiffPicOrderCnt( RefPicListX[ refIdxLX ] , RefPicList1[ 0 ] ) < DiffPicOrderCnt( RefPicListX[ refIdxLX ] , RefPicList0[ 0 ] ) )

- RefPicSelected = RefPicList1[ 0 ]

- Otherwise,

- RefPicSelected = RefPicList0[ 0 ]

The motion vector mvLX is scaled as specified in the following:

  - tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td		(Error: Reference source not found-1)

  - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(Error: Reference source not found-2)

  - mvLX =	Clip3( −32768, 32767, Sign( distScaleFactor * mvLX ) * 
	( ( Abs( distScaleFactor * mvLX ) + 127 ) >> 8 ) )	(Error: Reference source not found-3)

where td and tb are derived as:

  - td = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) )	(Error: Reference source not found-4)

  - tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ 0 ] ) )	(Error: Reference source not found-5)

The motion vector mvCLX is set equal to mvLX.

The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr  are derived as specified in the following:

- The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX equal to 0 as input.

- The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr. as the inputs.

The arrays currIvSamplesLXL, currIvSamplesLXCb, and currIvSamplesLXCr  are derived as specified in the following:

- Let currIvRefPic be the picture with PicOrderCnt( currIvRefPic ) equal to PicOrderCntVal and ViewIdx equal to RefViewIdx[ xP ][ yP ].

- Let the reference picture sample arrays currIvRefPicLXL, currIvRefPicLXCb, and currIvRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture currIvRefPic.

- The arrays currIvSamplesLXL, currIvSamplesLXCb, and currIvSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the width nCS and height nCS, the motion vector mvLX equal to MvDisp[ xP ][ yP ] and the motion vector mvCLX equal to MvDisp[ xP ][ yP ], and the reference arrays with currIvRefPicLXL, currIvRefPicLXCb and currIvRefPicLXCr as the inputs.

The arrays refIvSamplesLXL, refIvSamplesLXCb, and refIvSamplesLXCr  are derived as specified in the following:

- Let refIvRefPic be the picture with PicOrderCnt( refIvRefPic ) equal to PicOrderCnt( RefPicSelected) and ViewIdx equal to RefViewIdx[ xP ][ yP ].

- Let the reference picture sample arrays refIvRefPicLXL, refIvRefPicLXCb, and refIvRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture refIvRefPic.

- The arrays refIvSamplesLXL, refIvSamplesLXCb, and refIvSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the width nCS and height nCS, the motion vector mvLX equal to ( mvLX + MvDisp[ xP ][ yP ] ) and the motion vector mvCLX equal to ( mvCLX + MvDisp[ xP ][ yP ] ), and the reference arrays with refIvRefPicLXL, refIvRefPicLXCb and refIvRefPicLXCr as the inputs.

The variable shiftVal is set equal to ( iv_res_pred_weight_idx − 1 ).

The modified prediction samples predSamplesLXL[ x ][ y ] with x = 0..( nPbW ) − 1 and y = 0..( nPbH ) − 1 are derived as specified in the following:

  - predSamplesLXL[ x ][ y ] =	 ( Clip3( 0, ( 1 << bitDepth ) − 1, predSamplesLXL[ x ][ y ] + 
	( ( currIvSamplesLXL[ x ][ y ] − refIvSamplesLXL[ x ][ y ] )  >>  shiftVal ) ) )	(Error: Reference source not found-6)

The modified prediction samples predSamplesLXCb[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 )−1 are derived as specified in the following:

  - predSamplesLXCb[ x ][ y ] =	 ( Clip3( 0, ( 1 << bitDepth ) − 1, predSamplesLXCb[ x ][ y ] + 
	( ( currIvSamplesLXCb[ x ][ y ] − refIvSamplesLXCb[ x ][ y ] )  >>  shiftVal )))	(Error: Reference source not found-7)

The modified prediction samples predSamplesLXCr[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 ) − 1 are derived as specified in the following:

  - predSamplesLXCr[ x ][ y ] =	 ( Clip3( 0, ( 1 << bitDepth ) − 1, predSamplesLXCr[ x ][ y ] + 
	( ( currIvSamplesLXCr[ x ][ y ] − refIvSamplesLXCr[ x ][ y ] )  >>  shiftVal )))	(Error: Reference source not found-8)