| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0113 |
| --- | --- |

| Title: | 3D-CE5 related: Bin reduction for SDC residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In current 3D-HEVC, the residual coding of intra SDC modes may account for large proportion of the overall context coded bins. This is due to the fact that the binary codeword for the SDC residual magnitude could be very long (e.g. around 200 bins for the worst case) and most of the bins are context coded. To improve the throughput of the CABAC, the number of context coded bins should be reduced. Besides, it is also very critical to reduce the worst case number of context coded bins for the hardware design. This contribution proposes to constrain the residual magnitude of intra SDC within a predefined range so that the bin number for coding the syntax element associated with the SDC residual could be significantly reduced. Experimental results reportedly show that constraining the SDC residual magnitude within a small range (e.g. “1” or “5”) causes no coding efficiency loss.

# Problem statement

The intra Simplified Depth Coding (SDC) approach is an alternative coding mode in 3D-HEVC for depth coding. For an intra SDC-coded depth block, it is intra predicted by Planar prediction or depth modelling mode (DMM). The residual is not coded as quantized transform coefficients but one or two constant residual values are signaled. In summary, following information is signaled for an intra SDC-coded blocks:

- The type of segmentation/prediction of the current block. Possible values are

  - Planar (1 segment)

  - DMM Mode 1 (2 segments)

- For each resulting segment, a residual value (in the sample domain) is signaled in the bitstream.

In intra SDC coding, the residual value is mapped to a residual of index by using a Depth Lookup Table (DLT). To code the residual of index for each segment within the intra SDC coded block, a residual flag (or termed significance flag) is first transmitted to indicate if there is residual signal for the segment. If a segment has residual, a sign flag is then transmitted along with the magnitude of the residual.

The decoder could reconstruct the residual value after receiving the sign flag and the magnitude of the residual. The binarization process to generate the binary codeword for the syntax element of the residual magnitude is described in subclause H.9.3.3.10 in the draft text [1], and an example of binary codeword for the residual magnitude is demonstrated in Table 1. Note that, the prefix-codeword of the residual magnitude is context coded and the suffix-codeword of the residual magnitude is bypass coded.

Table 1.  The binary codeword for the residual magnitude for intra SDC mode (when the number of depth values in DLT is 256)

As shown in Table 1, the number of the bins for coding residual magnitude is 200 for the worst case (e.g. the number of depth values in DLT is 256) and thus the residual coding of intra SDC will account for a high percentage of the overall context coded bins in this case. In context-based adaptive binary arithmetic coding (CABAC), it is difficult to parallelize context coded bins. To improve the throughput of the CABAC, the number of context coded bins should be reduced. Moreover, hardware is designed for the worst case, it is also important to reduce the worst case number of the context coded bins.

# Proposed scheme

In this proposal, to reduce the number of the context coded bins with simple modifications, we propose to constrain the residual magnitude for intra SDC mode within a predefined range N (N is an non-negative integer). Since the absolute residual values are constrained within a range, the truncated unary (TU) binarization is employed for the syntax elements associated with the residual magnitude. Table 2 shows an example codewords for the residual magnitude when N is set equal to 5.

When the predefined range N is set equal to 1, we could further avoid transmitting the syntax elements associated with the residual magnitude. We could simply use the sign flag to indicate either “+1” or “-1” is signaled as the residual value. When the predefined range N is set equal to 0, it means no residual is signaled. The intra SDC mode could be considered as an intra Skip mode.

Table 2.  The truncated unary binarization for the residual magnitude of intra SDC mode with N=5

# Experimental results

The proposed schemes are integrated into HTM-8.0 [2], and the tests are conducted under the common test conditions (CTC) [3] and all-intra conditions (AI) using the attached all-intra configurations. The anchor is HTM-8.0. The experimental results under CTC of N equals to “5”, “1”, and “0” are shown in Table 3, Table 4, and Table 5, respectively. Constraining the SDC residual magnitude within “5”, “1”, and “0” under CTC introduce 0.0%, 0.0% and 0.2% coding efficiency loss under CTC, respectively.

Moreover, the experimental results under AI conditions are shown in Table 6, Table 7, and Table 8. Constraining the SDC residual magnitude within “5”, “1”, and “0” under AI conditions introduce 0.0%, 0.1% and 0.4% coding efficiency loss under CTC, respectively. Compared to the anchor, the three tested cases could significantly reduce the bin numbers for coding SDC residual magnitude with negligible or limited coding performance drops.

Table 3.  The results of constraining SDC residual magnitude within N=5 under CTC

Table 4.  The results of constraining SDC residual magnitude within N=1 under CTC

Table 5.  The results of constraining SDC residual magnitude within N=0 (no residual is transmitted) under CTC

Table 6.  The results of constraining SDC residual magnitude within N=5 under AI conditions

Table 7.  The results of constraining SDC residual magnitude within N=1 under AI conditions

Table 8.  The results of constraining SDC residual magnitude within N=0 under AI conditions

# Conclusion

In this contribution, it is proposed to constrain the SDC residual magnitude within a small range to significantly reduce the number of the context coded bins for coding SDC residual. The results show that the proposed modifications bring almost no change to the coding performance by constraining the SDC residual within “5” or “1”.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1001, July, 2013.

- HTM-8.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1100, July, 2013.