| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0114 |
| --- | --- |

| Title: | 3D-CE3 related: A bug-fix for the disparity-based Skip and Direct modes in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { jl.lin, yiwen.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The disparity-based skip/direct mode directly inherits the motion information, i.e., reference index(es) and motion vector(s), of a corresponding block in the base view as the motion information of the current block. However, an inherited reference index may exceed the maximum reference index of the current block, which may cause the encoder and decoder to crash. Therefore, it is proposed to add one additional check to see if the inherited reference index is valid. If the reference index is valid, the reference index and MVs of the corresponding block are directly inherited. Otherwise, the inherited inter-view motion information is viewed as unavailable. This bug-fix does not affect the coding performance under common test conditions.

# Problem statement

In the disparity-based skip and direct modes of 3D-AVC [1], motion information for coding current block Cb is derived from motion information of the corresponding block in the base view. The correspondence between Cb and corresponding block in the base view is established through a disparity vector which is applied at the central sample of block Cb. The MVP derivation for skip and direct modes is shown in Figure 1. An inter-view MVP (reference index and motion vectors) is first derived from the corresponding block in the base view indicated by a disparity vector. If the corresponding block in the base view is unavailable, the spatial MVP derived from the spatial blocks (A, B and C) with reference index equal to 0 is then used. The disparity derivation procedure for this mode is specified in Section 1.4.1 of “3D-AVC Test Model 7” [2] and the flow chart of the inter-view motion information derivation is provided in Figure 2.

Figure 1.  The disparity-based inter-view MVP derivation

Figure 2.  Flow chart of inter-view MVP derivation for skip/direct mode

Since the motion information, i.e., reference index(es) and motion vector(s), of the corresponding block in the base view (or reference view) are directly inherited by the current block. However, as shown in Figure 3, the inherited reference index may exceed the maximum reference index of the current block and the encoder or decoder may crash due to this invalid reference index. Therefore, a mechanism to deal with the described situation is needed.

Figure 3.  The inherited RefIdx exceeds current reference list size

# Proposed schemes

We propose to additionally check whether the inherited reference index exceeds the maximum reference index of the current block. When the inherited reference index exceeds the maximum reference index, the inter-view motion predictor is viewed as unavailable. The modification to the text is illustrated in the next section and the modified parts are highlighted. This bug-fix does not affect the coding performance under common test conditions. The detailed results can be seen in JCT3V-F0114.xls.

# Draft text modification

J.8.2.1.5 Derivation process for the motion vector in inter-view reference

Inputs to this process are mbPartIdx, subMbPartIdx, and listSuffixFlag.

Outputs of this process are the motion vector mvCorrespond and the reference index refIdxCorrespond.

Inter-view reference picture InterViewPic and an offset vector dV are derived as follows:

–	If TextureFirstFlag is equal to 0, the following ordered steps apply:

–	The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

–	The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

–	The inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

–	The following applies to derive an inter-view reference picture or inter-view only reference picture InterViewPic and to set the variable interViewAvailable:

interViewAvailable = 0
for( cIdx = 0; cIdx <= num_ref_idx_l0_active_minus1  &&  !interViewAvailable; cIdx++ )
	if ( view order index of RefPicList0[ cIdx ] is equal to 0 ) {
		InterViewPic = RefPicList0[ cIdx ]		(J-1)
		interViewAvailable = 1
	}

–	When interViewAvailable is equal to 1, the depth-based disparity value derivation process in subclause Error: Reference source not found is invoked with depthPic equal to DepthCurrPic, textureX equal to x1 + dx1 + dx2, textureY equal to y1 + dy1 + dy2, tBlWidth equal to the width the sub-macroblock partition CurrMbAddr\mbPartIdx\subMbPartIdx, tBlHeight equal to the height the sub-macroblock partition CurrMbAddr\mbPartIdx\subMbPartIdx, srcViewId equal to view_id and refViewId equal to the view_id of InterViewPic as inputs and the output assigned to dV[ 0 ] and dV[ 1 ] is set to 0.

–	Otherwise (TextureFirstFlag is equal to 1), the following ordered steps apply:

–	dV is set to (DvMBX, 0) and interViewAvailable is set to InterViewRefAvailable.

–	When interViewAvailable is equal to 1, InterViewPic is set to be the texture view component of the base view.

The refIdxCorrespond and mvCorrespond are set as follows.

–	If interViewAvailable is equal to 0, refIdxCorrespond is set to -1, and mvCorrespond[ 0 ] and mvCorrespond[ 1 ] are both set to 0.

–	Otherwise, the following step applies in order.

–	The variable luma4x4BlkIdx is derived as  (4 * mbPartIdx + subMbPartIdx).

–	The inverse 4x4 luma block scanning process as specified in subclause Error: Reference source not found is invoked with luma4x4BlkIdx as the input and ( x, y ) as the output. In addition, ( xCorrespond, yCorrespond ) is set equal to ( x + ( dV[ 0 ] >> 4 ), y + ( dV[ 1 ] >> 4 ) ) and mbAddrCorrespond is set equal to ( ( CurrMbAddr / PicWidthInMbs ) + ( dV[ 1 ] >> 6 ) ) * PicWidthInMbs + ( CurrMbAddr % PicWidthInMbs ) + ( dV[ 0 ] >> 6 ).

–	Set mbTypeCorrespond to the syntax element mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic. When mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, subMbTypeCorrespond is set to be the syntax element sub_mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic.

–	Set mbPartIdxCorrespond to the macroblock partition index of the corresponding partition and subMbPartIdxCorrespond to the sub-macroblock partition index of the corresponding sub-macroblock partition. The derivation process for macroblock and sub-macroblock partition indices as specified in subclause Error: Reference source not found is invoked with the luma location equal to ( xCorrespond, yCorrespond ), the macroblock type equal to mbTypeCorrespond, and when mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, the list of sub-macroblock types subMbTypeCorrespond as the inputs and the outputs are the macroblock partition index mbPartIdxCorrespond and the sub-macroblock partition index subMbPartIdxCorrespond.

–	The motion vector mvCorrespond and the reference index refIdxCorrespond are derived as follows.

–	If the macroblock mbAddrCorrespond is coded as Intra prediction mode, both components of mvCorrespond are set equal to 0 and refIdxCorrespond is set equal to –1.

–	Otherwise (the macroblock mbAddrCorrespond is not coded as Intra prediction mode), the prediction utilization flags predFlagLXCorrespond is set equal to PredFlagLX[ mbPartIdxCorrespond ], the prediction utilization flag of the macroblock partition mbAddrCorrespond\mbPartIdxCorrespond of the picture InterViewPic. In addition, the following applies.

–	When predFlagLXCorrespond is equal to 1 and RefIdxLX[ mbPartIdxCorrespond ] is less than or equal to num_ref_idx_lX_active_minus1, the mvCorrespond and the reference index refIdxCorrespond are set equal to MvLX[ mbPartIdxCorrespond ][ subMbPartIdxCorrespond ] and RefIdxLX[ mbPartIdxCorrespond ], respectively, which are the motion vector mvLX and the reference index refIdxLX that have been assigned to the (sub-)macroblock partition mbAddrCorrespond\mbPartIdxCorrespond\subMbPartIdxCorrespond inside the picture InterViewPic.

# Conclusion

In this proposal, a fix for the disparity-based skip and direct modes in 3D-AVC is proposed. The proposed method can fix the problem when the inherited reference index exceeds the maximum size of the reference picture list.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- M. M. Hannuksela, Y. Chen, T. Suzuki, “3D-AVC Draft Text 6,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D1002, April, 2013.

- D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki, “3D-AVC Test Model 7,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1003, July, 2013.