| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0118 |
| --- | --- |

| Title: | AHG4: Report on 3D-AVC Software Integration of Depth View Info SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Koki Wakunami, Yasuyuki Ichihashi, Hisayuki Sasaki, Kenji Yamamoto Masayuki Tanimoto | Tel: Email: | +81-42-327-7262 {senoh, k.wakunami, y-ichihashi, sasaki, k.yamamoto}@nict.go.jp +81-52-781-1883 tanimototentative3@yahoo.co.jp |
| Source: | National Institute of Information and Communications Technology (NICT), Nagoya Industrial Science Research Institute (NISRI) |  |  |

_____________________________

# Abstract

This contribution reports on 3D-AVC software integration of depth view info SEI message into ATM9.0.

# Introduction

At the 5th JCT-3V meeting, it was determined to integrate the depth view info SEI message process into ATM reference software. According to this decision, the software was integrated in the 3D-ATM v.9.0. This report describes how to run the software and shows the test results.

# Experiments

The instruction how to run the depth view info SEI message process is described in the following text. Currently the SEI message is outputted in a text file. Global View and Depth (GVD) software for HP mode is also in this directory.

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/branches/GVD/Note.txt

The followings are the test results for 3-view case. The results are compared to ATM v.9.0 HP mode. Since GVD data structure is different from normal 3-view case, the difference values from the anchor have not much meaning. It should be noted the enc. time includes global depth generation and encoding, residual view generation, and encoding the global depth/base view/residual view. The dec. time includes all texture view and global depth. The view synthesis time will be improved by parallelizing the process. The detail is in the attached excel file.

Table 1 Summary of GVD performance comparing to ATMv.9.0 HP mode for 3-view case.

# Conclusions

Integration of depth view info. SEI message process in ATM v.9.0 and its test results are reported.

# References

[1] M. M. Hannuksela, Y. Chen, T. Suzuki, JR. Ohm, G. J. Sullivan, “3D-AVC Draft Text 7”, JCT3V-E1002, Vienna, AU, Aug. 2013.

[2] D. Rusanovskyy, FC. Chen, L. Zhang, T. Suzuki, “3D-AVC Test Model 7”, JCT3V-E1003, Vienna, AU, Aug. 2013.

# Patent rights declaration(s)

NICT (National Institute of Information and Communications Technology) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).