| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0120 |
| --- | --- |

| Title: | CE1.h related : Simplified VSP Block Partitioning Process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taesup, Kim Sehoon, Yea #221, Yangjae-dong, Seocho-gu, Seoul, Korea | Tel: Email: | +82-2-6666-6666 taesup.kim@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to simplify view synthesis prediction by reducing complexity on VSP block partitioning process. Currently, each 8x8 block composing the given PU compares its corresponding 4 corners’ depth values to decide the block partitioning type (8x4 or 4x8 block)[1]. However, this 8x8 block level process is simplified by the proposed PU level process. This process constrains the given PU with VSP mode to have only one block partitioning type and determine it on PU level rather than 8x8 block level. This results in reducing the number of comparing operations. The experimental results reportedly show 0.03% gain on coded and synthesis views.

# Introduction

In the current 3DV-HEVC (HTM 8.0)[2], the view synthesis prediction (VSP) is an inter-view prediction tool that is more flexible than the conventional disparity compensated prediction (DCP). Rather than using the PU-level prediction as DCP, it uses block-level (8x4 or 4x8 block in CTC) inter-view prediction by taking the corresponding disparity information from the reference picture’s coded depth map. Currently, the block size has been an issue to reduce the complexity of VSP, and the adaptive block size (8x4 or 4x8 block) is used by adaptively splitting each 8x8 block as depicted in Figure 1. However, in this contribution, this adaptive block partitioning process further simplified by changing 8x8 block level splitting process into PU block level splitting process.

Figure 1. The block partition type is adaptively determined on 8x8 block level, and the given PU could have different partition types.

# Proposed Method

To simplify the current VSP block partitioning process, it is assumed that the given PU with VSP has to be split into only single partition type (8x4 or 4x8 block), and proposed to select the partition type on PU level instead of 8x8 block level. Therefore, the given PU with VSP determines single partition type only once by referring its corresponding virtual depth maps as shown in Figure 2.

Figure 2. The proposed method uses single partition type for the given PU with VSP, and determines the partition type by referring PU’s corresponding virtual depth map.

The simplified VSP block partitioning process could be compared to the current method in terms of the number of times for selecting the partition type as shown in Figure 3. Regardless of the given PU size, the proposed method always needs only single time for selecting the partition type.

Figure 3. The number of times for selecting partition type for the given PU with VSP.

# Experimental Results

Table 1. Experimental results on the proposed method simplifying the VSP block partitioning process.
(Anchor: HTM 8.0 CTC)

# Conclusion

In this contribution, the VSP block partitioning process is simplified in terms of reducing the number of operations related to determine the partition type. The proposed method, though simplifies the process, provides an overall bit rate gain of 0.03%. It is recommended to adopt the proposed simplification into the current HTM and the common test conditions.

# References

- S. Shimizu and S. Sugimoto, “3D-CE1.h: Adaptive block partitioning for VSP,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E0207, 5th Meeting: Vienna, AU, 27 Jul.–2 Aug. 2013.

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Test Model 5,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1005, 5th Meeting: Vienna, AU, 27 Jul.–2 Aug. 2013.

# Patent rights declaration(s)

LG Electronics Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).