| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0122 |
| --- | --- |

| Title: | HTM support of depth coding in MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Li Zhang Xin Zhao Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal provides the concrete design of coding depth views in MV-HEVC. The proposal includes both the software for HTM and working draft text changes in MV-HEVC.

# Introduction

It has been discussed that in MV-HEVC, depth coding should be included, with minimum additional cost for the video codec, e.g., using the same profile as in MV-HEVC.

In the 5th JCT-3V meeting, it has been tentatively agreed to code the multiple views with auxiliary pictures.

There are proposals targeting at conveying the information of indicating the depth view characteristics or signaling of the auxiliary pictures with various ways of HLS design. However, this proposal focuses on supporting depth coding in HTM.

# Proposed

In this proposal, a software implementation of depth coding within MV-HEVC has been provided. In the MV-HEVC depth coding design, we propose to code the depth views with the same complexity as HEVC and with minimum communication of the each depth view with other views to facilitate implementations as much as possible.

In this proposal, each depth view is simultaneously coded with HEVC.

# Experimental Results

This section provides some simulation results MV-HEVC based on the proposed 3DV-HTM anchor. The platform 3D-HTM 8.0 is utilized and the proposed method is implemented on top of it. All the simulation tests are performed based on the common test conditions.

In all simulations, for both MV-HEVC and 3D-HEVC, VSO is turned on. The anchor is generated by 3D-HTM v8.0 using CTC configurations.

The performance of the MV-HEVC codec with depth coding is shown in Table 1.

Table 1: Coding performance of MV-HEVC with depth coding

|  | Video 0 | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.1% | 40.2% | 38.4% | 15.0% | 19.7% | 17.5% | 243% | 103% | 124% |
| Kendo | -0.1% | 36.7% | 40.5% | 14.7% | 20.2% | 19.9% | 178% | 81% | 95% |
| Newspapercc | -0.1% | 22.0% | 20.5% | 7.9% | 12.8% | 16.8% | 204% | 81% | 98% |
| GhostTownFly | 0.0% | 35.0% | 32.4% | 8.3% | 15.8% | 22.3% | 220% | 81% | 98% |
| PoznanHall2 | -0.1% | 21.9% | 23.2% | 8.7% | 12.0% | 15.6% | 299% | 96% | 113% |
| PoznanStreet | 0.0% | 17.9% | 18.1% | 5.5% | 9.4% | 11.9% | 212% | 78% | 91% |
| UndoDancer | 0.0% | 27.8% | 27.5% | 7.4% | 12.6% | 21.5% | 263% | 93% | 111% |
| 1024x768 | -0.1% | 33.0% | 33.2% | 12.5% | 17.6% | 18.1% | 208% | 88% | 106% |
| 1920x1088 | 0.0% | 25.7% | 25.3% | 7.5% | 12.4% | 17.8% | 248% | 87% | 103% |
| average | 0.0% | 28.8% | 28.7% | 9.7% | 14.6% | 17.9% | 231% | 88% | 104% |
| Shark | 0.0% | 36.0% | 32.8% | 7.9% | 18.9% | 29.4% | 260% | 84% | 107% |

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

# Proposed text

- G.11	Profiles, tiers, and levels

- G.11.1	Profiles

- G.11.1.1	General

- G.11.1.2	Stereo Main profile

Bitstreams conforming to the Stereo Main profile shall obey the following constraints:

- The sub-bitstream resulting from the sub-bitstream extraction process with any value of tIdTarget and a value of 0 in layerIdListTarget as inputs shall conform to the Main profile.

- The bitstream shall contain one layer with nuh_layer_id equal to i for which ViewScalExtLayerFlag[ i ] is equal to 1.

- When ViewScalExtLayerFlag[ i ] is equal to 1, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_extension( ) syntax structure of the active layer SPS RBSP of any coded pictures with nuh_layer_id equal to i.

- When ViewScalExtLayerFlag[ i ] is equal to 1, ScalabilityId[ LayerIdxInVps [ i ]][ smIdx ] shall be equal to 0 for any smIdx value from 0 to 15, inclusive, that is not equal to 1, for any coded picture with nuh_layer_id equal to i.

- SPSs shall have sps_shvc_reserved_zero_idc equal to 0 only.

- When ViewScalExtLayerFlag[ i ] is equal to 1, the values of pic_width_in_luma_samples and pic_height_in_luma_samples in the active SPS for the layer with nuh_layer_id equal to i shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in the active SPSs of all direct reference layers of that layer.

- The bitstream shall contain a sub-bitstream consisting of two layers having nuh_layer_id equal to 0 and nuhLayerIdA for which ScalabilityId[ LayerIdxInVps [ nuhLayerIdA ]][ smIdx ] shall be equal to 0 for any smIdx from 0 to 15, inclusive that is not equal to 1.

- VPSs shall have avc_base_layer_flag equal to 0 only.

- The bitstream shall not contain more than two layers with scalability_mask[ i ][ 0 ] equal to 1.

- If scalability_mask[ i ][ 0 ] is equal to 1, direct_dependency_flag[ i ][ j ] shall be equal to 0 for any j from 0 through i-1, inclusively.

It is noticed that when auxiliary pictures are used for depth coding in MV-HEVC as proposed in JCT3V-F0031, this proposal can be rephrased as follows. Text that needs to be newly added into the working draft is highlighted in green.

Table Error: Reference source not found-2 – Mapping of AuxId to the type of auxiliary pictures

| AuxId | Name of AuxId | Type of auxiliary pictures |
| --- | --- | --- |
| 0 | - | Not an auxiliary picture |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| … |  |  |

- G.11.1.2	Stereo Main profile

…

- The bitstream shall not contain more than two layers with AuxId[ nuhLayerIdA ] equal to 2.

- For any i-th layer with nuh_layer_id equal to nuhLyaerIdA, if AuxId[ nuhLayerIdA ] is equal to 2, direct_dependency_flag[ i ][ j ] shall be equal to 0 for any j from 0 through i-1, inclusively.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).