| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0123 _proposed text |
| --- | --- |

| Title: | Proposed text for JCT3V-F0123 based on 3D-HEVC Draft Text 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

All the changes are highlighted in green or red strikethrough.

# Syntax tables

H.7.3.8.5	General Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |  |
| --- | --- | --- |
| if( transquant_bypass_enable_flag ) { |  |  |
| cu_transquant_bypass_flag | ae(v) |  |
| } |  |  |
| if( slice_type != I ) |  |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |  |
| nCbS = ( 1 << log2CbSize ) |  |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |  |
| if ( iv_res_pred_flag[ nuh_layer_id ] && TempRefPicInListsFlag ) |  |  |
| iv_res_pred_weight_idx | ae(v) |  |
| if ( !iv_res_pred_weight_idx && icEnableFlag ) |  |  |
| ic_flag | ae(v) |  |
| } |  |  |
| else { |  |  |
| … |  |  |
| } else { |  |  |
| … |  |  |
| } |  |  |
| } |  |  |
| if ( !iv_res_pred_weight_idx && icEnableFlag ) |  |  |
| ic_flag | ae(v) |  |
| if( DepthFlag && PredMode[ x0 ][ y0 ] ! = MODE_INTRA && !skip_flag[ x0 ][ y0 ] ) |  |  |
| … |  |  |

#### 7.3.8.8	Transform tree syntax

| transform_tree( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < MaxTrafoDepth && !( IntraSplitFlag && ( trafoDepth = = 0 ) ) ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if(!iv_res_pred_weight_idx && log2TrafoSize > 2 ) { |  |
| if( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( 1 << ( log2TrafoSize − 1 ) ) |  |
| y1 = y0 + ( 1 << ( log2TrafoSize − 1 ) ) |  |
| transform_tree( x0, y0, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y0, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x0, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x1, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| transform_unit( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) |  |
| } |  |
| } |  |

Semantics

cbf_cb[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cb transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform unit. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When cbf_cb[ x0 ][ y0 ][ trafoDepth ] is not present, the value of cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred as follows:

- If iv_res_pred_weight_idx is equal to 0 and trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

cbf_cr[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cr transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform unit. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When cbf_cr[ x0 ][ y0 ][ trafoDepth ] is not present, the value of cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred as follows:

- If iv_res_pred_weight_idx is equal to 0 and trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

# Decoding process

H.8.3.7	Derivation process for the alternative target reference index for TMVP in merge mode and the first temporal reference picture index

This process is invoked when the current slice is a P or B slice.

The variables AltRefIdxL0 and AltRefIdxL1 are set equal to –1, TempRefPicInListsFlag is set equal to 0 FstRefIdxL0 and FstRefIdxL1 are set equal to –1, and the following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

for( i = 0; i <= num_ref_idx_lX_active_minus1 && FstRefIdxLX = = –1; i++)
	if ( RefPicListX[ i ] is a short-term reference picture)
		FstRefIdxLX = i
zeroIdxLtFlag = RefPicListX[ 0 ] is a short-term reference picture ? 0 : 1
for( i = 1; i <= num_ref_idx_lX_active_minus1 && AltRefIdxLX = = –1; i++)
	if ( ( zeroIdxLtFlag && RefPicListX[ i ] is a short-term reference picture) | |
		( !zeroIdxLtFlag && RefPicListX[ i ] is a long-term reference picture) ) 
		AltRefIdxLX = i 
if( !RapPicFlag ) 
	TempRefPicInListsFlag = TempRefPicInListsFlag | | !zeroIdxLtFlag | | ( AltRefIdxLX ! = –1 )

- H.8.5.3.3.1 General

…

  - For X being each of 0 and 1, when predFlagLX is equal to 1, the following applies:

    - When predFlagLX is equal to 1, the following applies.

      - The variable resPredFlag is derived as specified in the following:

        - resPredFlag = 	( iv_res_pred_weight_idx  !=  0 ) 	
	( PicOrderCnt( RefPicListX[ refIdxLX ] )  !=  PicOrderCntVal ) 	(Error: Reference source not found-1)

- H.8.5.3.3.7	Bilinear sample interpolation and residual prediction process

The process is only invoked if res_pred_flag is equal to 1.

Inputs to this process are:

- a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,prediction list utilization flags, predFlagL0 and predFlagL1,

- the prediction list indication X,

- the prediction list utilization flag predFlagLX,

- the reference index refIdxLX,

- the motion vectors mvLX, mvCLX

Output of this process are:

- the (nPbW)x(nPbH) array predSamplesLXL,

- the (nPbW / 2)x(nPbH / 2) arrays predSamplesLXCb and predSamplesLXCr.

The motion vector mvLX is scaled as specified in the following:

  - tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td		(Error: Reference source not found-2)

  - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(Error: Reference source not found-3)

  - mvLX =	Clip3( −32768, 32767, Sign( distScaleFactor * mvLX ) * 
	( ( Abs( distScaleFactor * mvLX ) + 127 ) >> 8 ) )	(Error: Reference source not found-4)

where td and tb are derived as:

  - td = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) )	(Error: Reference source not found-5)

  - tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ 0 ] ) )	(Error: Reference source not found-6)

- For each block (blkX=0..nPbW /8 -1, blkY=0..nPbH / 8-1), the following applies:

Variable bIvRef is set to ( DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ]  = = 0 ? 1 : 0 ), motion vector dvARP is set to MvDisp[ xP ][ yP ], refViewIdx is set to RefViewIdx[ xP + blkX*8 ][ yP + blkY*8 ], variable bAvail is set equal to false.

- When bIvRef is equal to false and FstRefIdxLX is unequal to -1, the motion vector scaling process specified in subclause H.8.5.3.3.7.3 is invoked with X, RefPicListX[ refIdxLX ] and mvLX as input. refIdxLX is reset to FstRefIdxLX.

- The motion vector refinement process specified in subclause H.8.5.3.3.7.4 is invoked with ( xP, yP ), nPbW equal to 8 and nPbH equal to 8, RefPicListX[ refIdxLX ], mvLX, MvDisp[ xP + blkX*8 ][ yP + blkY*8 ] and bIvRef as input, and the outputs are dvARP and refViewIdx (when bIvRef is equal to false), or the outputs are the bAvail, motion vector mvT and reference picture list Y.

- When FstRefIdxLX is equal to -1 or bAvail is equal to false, iv_res_pred_weight_idx is reset to 0.

The motion vector mvCLX is set equal to mvLX.

The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr  are derived as specified in the following:

- The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX equal to 0 as input.

- The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr. as the inputs.

When iv_res_pred_weight_idx is unequal to 0, tThe arrays currIvSamplesLXL, currIvSamplesLXCb, and currIvSamplesLXCr  are derived as specified in the following:

- Let currIvRefPic be the picture with PicOrderCnt( currIvRefPic ) equal to PicOrderCntVal and ViewIdx equal to refViewIdxRefViewIdx[ xP ][ yP ] when bIvRef is equal to false, and the picture RefPicListY[ FstRefIdxLY ] when bIvRef is equal to true.

- Let the reference picture sample arrays currIvRefPicLXL, currIvRefPicLXCb, and currIvRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture currIvRefPic.

- The arrays currIvSamplesLXL, currIvSamplesLXCb, and currIvSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the width nCS and height nCS, the motion vector mvLX equal to ( bIvRef ? mvT : dvARP) MvDisp[ xP ][ yP ] and the motion vector mvCLX equal to ( bIvRef ? mvT : dvARP) MvDisp[ xP ][ yP ], and the reference arrays with currIvRefPicLXL, currIvRefPicLXCb and currIvRefPicLXCr as the inputs.

When iv_res_pred_weight_idx is unequal to 0, tThe arrays refIvSamplesLXL, refIvSamplesLXCb, and refIvSamplesLXCr  are derived as specified in the following:

- Let refIvRefPic be the picture with PicOrderCnt( refIvRefPic ) equal to PicOrderCnt( bIvRef ? : RefPicListY[ FstRefIdxLY ]: RefPicListX[ FstRefIdxLX 0 ] ) and ViewIdx equal to refViewIdxRefViewIdx[ xP ][ yP ].

- Let the reference picture sample arrays refIvRefPicLXL, refIvRefPicLXCb, and refIvRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture refIvRefPic.

- The arrays refIvSamplesLXL, refIvSamplesLXCb, and refIvSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xC, yC ), ( xB, yB ), the width and the width nCS and height nCS, the motion vector mvLX equal to ( mvLX + ( bIvRef ? mvT : dvARP )MvDisp[ xP ][ yP ] ) and the motion vector mvCLX equal to ( mvCLX + ( bIvRef ? mvT : dvARP ) MvDisp[ xP ][ yP ] ), and the reference arrays with refIvRefPicLXL, refIvRefPicLXCb and refIvRefPicLXCr as the inputs.

The variable shiftVal is set equal to ( iv_res_pred_weight_idx − 1 ).

The modified prediction samples predSamplesLXL[ x + blkX*8 ][ y + blkY*8 ] with x = 0..7( nPbW ) − 1 and y = 0..7( nPbH ) − 1 are derived as specified in the following:

  - predSamplesLXL[ x ][ y ] =	 predSamplesLXL[ x ][ y ] + 
	( ( currIvSamplesLXL[ x ][ y ] − refIvSamplesLXL[ x ][ y ] )  >>  shiftVal )	(Error: Reference source not found-7)

The modified prediction samples predSamplesLXCb[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 )−1 are derived as specified in the following:

  - predSamplesLXCb[ x ][ y ] =	 predSamplesLXCb[ x ][ y ] + 
	( ( currIvSamplesLXCb[ x ][ y ] − refIvSamplesLXCb[ x ][ y ] )  >>  shiftVal )	(Error: Reference source not found-8)

The modified prediction samples predSamplesLXCr[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 ) − 1 are derived as specified in the following:

  - predSamplesLXCr[ x ][ y ] =	 predSamplesLXCr[ x ][ y ] + 
	( ( currIvSamplesLXCr[ x ][ y ] − refIvSamplesLXCr[ x ][ y ] )  >>  shiftVal )	(Error: Reference source not found-9)

H.8.5.3.3.7.3	Motion vector scaling process

Inputs to this process are:

- A reference picture list X (with X being 0 or 1),

- A reference picture refPicLX in RefPicListX,

- A motion vector mvLX

- Output of this process is a scaled motion vector mvLX.

The motion vector mvLX is scaled as specified in the following:

  - tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td		(Error: Reference source not found-XXX)

  - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(Error: Reference source not found-XXX)

  - mvLX =	Clip3( −32768, 32767, Sign( distScaleFactor * mvLX ) * 
	( ( Abs( distScaleFactor * mvLX ) + 127 ) >> 8 ) )	(Error: Reference source not found-XXX)

where td and tb are derived as:

  - td = Clip3( −128, 127, PicOrderCnt(currPic) − PicOrderCnt( refPicLX )	(Error: Reference source not found-XXX)

  - tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ FstRefIdxLX ] ) )	(Error: Reference source not found-XXX)

H.8.5.3.3.7.4	motion vector identification in a reference block

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current block unit relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,prediction list utilization flags, predFlagL0 and predFlagL1,

- a reference picture refPicLX, a motion vector mvLX, and a disparity vector dvARP

- a flag bIvRef to indicate refPicLX is a short-term or long-term picture,

- Output of this process is a modified disparity vector dvARP and reference view order index refViewIdx (when bIvRef is equal to 0), a flag bAvail, motion vector mv and reference picture list Y (when bIvRef is equal to 1).

The reference luma location ( xRef, yRef ) in refPicLX is derived by

  - xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( nPSW  >>  1 ) + ( ( mvLX[ 0 ] + 2 )  >>  2 ) )	(Error: Reference source not found-XXX)
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( nPSH  >>  1 ) + ( ( mvLX[ 1 ] + 2 )  >>  2 ) )	(Error: Reference source not found- XXX)

Let variable refCU and refPU be the coding unit and prediction unit that cover the luma location ( xRef, yRef ), respectively. bAvail is set to false. When the variable PredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following ordered steps apply until bAvai is equal to true, for Y in the range of 0 to 1, inclusive:

- The variable refPredFlagLY is set equal to the prediction utilization flag predFlagLY of the prediction unit refPU.

- When refPredFlagLY is equal to 1, the following applies:

  - When bIvRef is equal to 0 and mvLY of refPU points to an inter-view reference picture, bAvail is set to true, dvARP and and refViewIdx are set equal to the motion vector mvLY and the view order index of the reference picture of the prediction unit refPU, respectively.

  - When bIvRef is equal to 1, mvLY of refPU points to a temporal reference picture and FstRefIdxLY is unequal to -1, bAvail is set to true and H.8.5.3.3.7.3 is invoked with Y, the motion vector mvLY and the reference picture list RefPicListLY of the prediction unit refCU as the inputs, and the output is mv.