| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0123 |
| --- | --- |

| Title: | CE4: Further improvements on advanced residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current 3D-HEVC, the advanced residual prediction (ARP) is employed to code the residue of the current prediction unit in non-base texture views more efficiently. ARP reduces the energy of the temporal residue: residue generated from a temporal reference picture. This proposal is a follow-up of JCT3V-E0124, wherein several aspects were proposed to further improve the coding efficiency of ARP. The techniques in this proposal include: 1) the ARP is extended for inter-view residue, when the current block utilizes inter-view prediction from an inter-view reference picture, in a similar way as current design; 2) a disparity motion vector associated with the reference block of current block, if available, is used during ARP, to replace the derived disparity vector; 3) the signalling of Illumination Compensation (IC) flag and ARP weighting factors are jointly considered wherein the ic_flag is not signalled when the ARP weighting factor is unequal to 0; 4) In addition to the above items, block-level ARP is proposed by splitting one prediction unit into several blocks to perform ARP separately. With these improvements, it is observed that there is a coding gain of 0.7% in terms of BD rate for texture views compared to HTM under the common test condition. Accordingly 0.5% BD rate reduction is achieved for the synthesized views.

# Introduction

In 3D-HEVC, advanced residual prediction (ARP), as proposed in [1], is enabled for dependent texture views. Figure 1 illustrates the prediction structure of ARP in multiview video coding and 3D video coding for one reference picture list. To get the predictor of one prediction unit (PU) in one reference picture list, three blocks are firstly identified, i.e., the temporal reference block in current view (denoted by CurrRef), a corresponding block in the reference view located by the DV from NBDV process (denoted by Base) and the reference block in the reference view (denoted by BaseRef) by reusing the motion information of current block, as shown in Figure 1. Then, the residual predictor is calculated by the differences of two blocks in the reference view, i.e., BaseRef - Base. In addition, a weighting factor w (being 0, 1 or 0.5) is further multiplied to the residual predictor and the weighted residual predictor is added to current temporal reference block to generate the final prediction of current block for one reference picture list.

However, when current block uses an inter-view reference picture (in a different view) for one reference picture list, the residual prediction process is disabled due to the missing of mvLX.

Figure 1. Prediction structure of ARP in 3D-HEVC.

The current design of ARP may be further improved due to the following reasons:

- ARP is always disabled when inter-view prediction is used.

- The disparity vector (DV) used in ARP is obtained from the coded disparity motion vectors of spatial/temporal neighbouring blocks which may be not accurate enough. Therefore, sub-optimal reference blocks in the reference view, i.e., Base and BaseRef in Figure 1 may be used in ARP.

- The signalling of ARP and the signaling of Illumination Compensation (IC) [2], related are not jointly considered although they practically have connections. For example, the current 3D-HEVC design doesn’t enable IC when ARP is applied (the weighting factor for ARP is not zero).

# Proposed Scheme

To solve the above problems, in this proposal, several aspects are proposed to further improve the performance of ARP. Each aspect will be described in details in the following sub-sections. The first three aspects are exactly the same as those proposed in JCT3V-E0124 [3].

## ARP for inter-view residual

Similar to current ARP design for temporal residual, when current prediction unit uses an inter-view reference picture, prediction of inter-view residual is enabled. Firstly the inter-view residual within a different access unit is calculated, then the calculated residual information is used to predict the inter-view residual of the current block.

Practically, still three related blocks are identified: the reference block in the reference view located by the disparity motion vector of the current block (denoted by Base); the reference block of Base in the reference view (denoted by BaseRef) located by the temporal motion vector (mvLX) and reference index, if available, contained by Base; a reference block in current view (denoted by CurrRef) by reusing the temporal motion information from Base, as shown in Figure 2.

With the identified three blocks, the residual predictor of the residual signal of current PU can be calculated as the difference between these two blocks in the different access unit: CurrRef - BaseRef. Furthermore, the inter-view predictor is multiplied by a weighting factor as used in current ARP.

Similarly, bi-linear filter is used to generate the three relative blocks as in current design of ARP for temporal residual prediction. Furthermore, when the temporal motion vector contained by Base points to a reference picture that is in a different access unit of the first available temporal reference picture of current PU, it is firstly scaled to the first available temporal reference picture and the scaled motion vector is used to locate two blocks in a different access unit.

Figure 2. Prediction structure of ARP for inter-view residual.

## Disparity vector refinement for current ARP

When a current prediction unit utilizes temporal prediction, its temporal reference block (CurrRef) is already identified during the decoding of ARP. It the temporal reference block contains a disparity motion vector, it may be more accurate than the disparity vector (DV) derived by NBDV. In this case, the DV is proposed to be replaced by the disparity motion vector during ARP.

For simplicity, only the block covering the center position of CurrRef is checked, if it contains a disparity motion vector and it is not coded with BVSP, the disparity motion vector is used to identify the two reference blocks in the reference view, i.e., Base and BaseRef in Figure 1.

## Joint optimization of IC and ARP weighting factor signaling

In current 3D-HEVC, the usage of IC is indicated by a flag, i.e., ic_flag which may be signalled at the coding unit (CU) level while the ARP weighting factor, when signalled, is also at CU-level. In addition, ARP and IC couldn’t be applied together for one inter-coded CU. Therefore, it is proposed that the signalling of ic_flag is always skipped when the ARP weighting factor is unequal to 0, to avoid the unnecessary signaling overhead of ic_flag.

## Block-level ARP

When ARP is applied to either temporal residual or inter-view residual, the motion information associated with a block covering the center position of the reference block located by current PU’s motion vector is used either to update the DV from NBDV or to update the temporal motion vector, used to generate the final residual predictor. In some cases, the reference block may contain different motion information. Therefore, better coding gain may be expected if considering the multiple motion information within the reference block.

One straightforward way is to perform the ARP on block-level. More specifically, each PU is firstly split into several blocks (8 x 8 block size is used in the simulations), and each block share the same motion information as current PU.

For each 8x8 block, the disparity vector or temporal motion vector information could be updated for each block. When current “ARP coded” PU has a temporal reference picture, the disparity vectors of each 8x8 block within current PU may be different, as shown in Figure 3 (a). When current ARP coded PU has an inter-view reference picture, the temporal motion vectors of each 8x8 block within current PU may be different, as shown in Figure 3 (b).

Furthermore, the residual of chroma components is not coded if the current PU is coded with ARP.

(a)									(b)

Figure 3. Prediction structure of block-level ARP: (a) temporal residual (b) inter-view residual.

## Bug fix item for the generation of reference picture for ARP

In current ARP, if one PU is coded with ARP mode, the motion vector associated with current PU is scaled to the first entry of current reference picture list to improve the cache efficiency. However, the first entry of one reference picture list may not be a temporal reference picture. Therefore, it is proposed to scale the temporal motion vector (either the one associated with current PU or the one associated with the reference block for inter-view residual) to the first available temporal reference picture in current reference picture list. Such a change has no effect on the coding performance under the common test conditions.

# Experimental Results

This section provides simulation results of the proposed ARP improvement in comparison with the 3DV-HTM anchor. The platform 3D-HTM 8.0 [4] is utilized and the proposed method is implemented on top of it. All the simulation tests are performed based on the common test conditions [5].

To have a better understanding of the performance of the proposed four aspects related to ARP, separate results are also provided. Therefore, the following cases are tested and the corresponding performance compared to 3D-HTM anchor are summarized and listed in the following parts:

- Test #1: the full proposal as described in section 2;

- Test #2: combination of the first three aspects, as described from section 2.1, 2.2 and 2.3, without the aspect as described in section 2.4;

- Test #3: support of ARP for inter-view residual, as described in section 2.1;

The coding performance of the whole proposal is given in Table 1. As shown in Table 1, the overall average bitrate reduction is 0.7%, 0.6%, 0.5% for the decoded texture views, coded and synthesized views, respectively.

Table 1: Coding gain of the proposal with respect to anchor for 3-view case (Test #1)

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -2.4% | -2.6% | -1.0% | -0.8% | -0.8% | 96% | 103% | 96% |
| Kendo | -3.0% | -3.5% | -1.3% | -1.1% | -1.0% | 107% | 105% | 109% |
| Newspapercc | -1.7% | -1.4% | -0.6% | -0.5% | -0.4% | 103% | 103% | 103% |
| GhostTownFly | -1.0% | -1.0% | -0.2% | -0.1% | -0.2% | 100% | 104% | 100% |
| PoznanHall2 | -2.1% | -2.0% | -0.8% | -0.8% | -0.6% | 120% | 103% | 116% |
| PoznanStreet | -1.4% | -1.2% | -0.4% | -0.4% | -0.3% | 98% | 105% | 97% |
| UndoDancer | -1.4% | -1.7% | -0.3% | -0.3% | -0.3% | 106% | 102% | 101% |
| 1024x768 | -2.3% | -2.5% | -0.9% | -0.8% | -0.7% | 102% | 104% | 103% |
| 1920x1088 | -1.4% | -1.5% | -0.4% | -0.4% | -0.4% | 106% | 104% | 103% |
| average | -1.8% | -1.9% | -0.7% | -0.6% | -0.5% | 104% | 104% | 103% |

The coding performance of the proposed first three improvements as in JCT3V-E0124 is given in Table 2. As shown in Table 2, the overall average bitrate reduction is 0.4%, 0.3%, 0.3% for the decoded texture views, coded and synthesized views, respectively.

Table 2: Coding gain with respect to anchor for 3-view case (Test #2): without block-level ARP

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -1.2% | -1.4% | -0.6% | -0.5% | -0.4% | 93% | 99% | 99% |
| Kendo | -2.1% | -2.3% | -0.9% | -0.8% | -0.7% | 96% | 100% | 102% |
| Newspapercc | -0.8% | -0.6% | -0.3% | -0.3% | -0.2% | 105% | 100% | 104% |
| GhostTownFly | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 95% | 100% | 94% |
| PoznanHall2 | -1.0% | -0.8% | -0.4% | -0.3% | -0.3% | 117% | 100% | 118% |
| PoznanStreet | -0.7% | -0.6% | -0.3% | -0.2% | -0.2% | 95% | 101% | 98% |
| UndoDancer | -0.6% | -0.4% | -0.1% | -0.1% | -0.2% | 113% | 100% | 110% |
| 1024x768 | -1.4% | -1.4% | -0.6% | -0.5% | -0.4% | 98% | 100% | 102% |
| 1920x1088 | -0.5% | -0.5% | -0.2% | -0.2% | -0.2% | 105% | 100% | 105% |
| average | -0.9% | -0.9% | -0.4% | -0.3% | -0.3% | 102% | 100% | 104% |

The coding performance of ARP for inter-view residual is given in Table 3. As shown in Table 3, the overall average bitrate reduction is 0.2%, 0.2%, 0.2% for the decoded texture views, coded and synthesized views, respectively.

Table 3: Coding gain with respect to anchor for 3-view case (Test #3): inter-view residual

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.8% | -0.9% | -0.4% | -0.3% | -0.2% | 96% | 99% | 98% |
| Kendo | -1.2% | -1.4% | -0.5% | -0.4% | -0.4% | 101% | 100% | 104% |
| Newspapercc | -0.7% | -0.4% | -0.3% | -0.2% | -0.2% | 100% | 100% | 101% |
| GhostTownFly | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% | 101% | 100% | 97% |
| PoznanHall2 | -0.4% | -0.3% | -0.1% | -0.1% | -0.1% | 121% | 100% | 120% |
| PoznanStreet | -0.5% | -0.5% | -0.2% | -0.2% | -0.1% | 97% | 101% | 97% |
| UndoDancer | -0.5% | -0.5% | -0.1% | -0.1% | -0.1% | 111% | 100% | 106% |
| 1024x768 | -0.9% | -0.9% | -0.4% | -0.3% | -0.2% | 99% | 100% | 101% |
| 1920x1088 | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 107% | 100% | 105% |
| average | -0.6% | -0.5% | -0.2% | -0.2% | -0.2% | 104% | 100% | 103% |

The encoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations. The decoding time is calculated using one single machine.

# References

- L. Zhang, Y. Chen, X. Li, M. Karczewicz, “CE4: Advanced residual prediction for multiview coding,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D0177, 4th Meeting: Incheon, KR, 20–26 Apr. 2013.

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Test Model 5,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D1005, 4th Meeting: Incheon, KR, 20–26 Apr. 2013.

- L. Zhang, Y. Chen, X. Zhao, M. Karczewicz, Y. G. Cai, S. W. Ma, “Further improvements on advanced residual prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E0124, 5th Meeting: Vienna, AT, 27 July-2 Aug. 2013.

- 3DV-HTM version 8.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1100, 5th Meeting: Vienna, AT, 27 July-2 Aug. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).