| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0125 _proposed_text |
| --- | --- |

| Title: | Proposed text for JCT3V-F0125 based on 3D-HEVC Draft Text 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-658-5536 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

All the newly added texts are highlighted in green and deleted texts are indicated by red strikethrough.

- H.7.3.2.1.2Video parameter set extension 2 syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( !VpsDepthFlag[ layerId ] ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| view_synthesis_pred_flag[ layerId ] | u(1) |
| } else { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| vps_depth_modes_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| if( vps_depth_modes_flag[ layerId ] ) |  |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) { |  |
| dlt_depth_value[ layerId ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

- H.8.5.1	General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause 8.6.1 is invoked with the luma location ( xCb, yCb ) as input.

The variable nCbSL is set equal to 1  <<  log2CbSize and the variable nCbSC is set equal to 1  <<  ( log2CbSize − 1 ).

The decoding process for coding units coded in inter prediction mode consists of following ordered steps:

  - When DepthFlag is equal to 0, and wWhen iv_mv_pred_flag[ nuh_layer_id ] is equal to 1, or iv_res_pred_flag[ nuh_layer_id ] is equal to 1 or view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1, the derivation process for disparity vectors as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as the inputs.

  - When DepthFlag is equal to 1, and iv_mv_pred_flag[ nuh_layer_id ] is equal to 1, the derivation process for disparity vectors as specified in subclause H.8.5.5.3 is invoked with the luma locations ( xCb , yCb ), the coding block size nCbSL as the inputs

  - The inter prediction process as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the outputs are three arrays predSamplesL, predSamplesCb, and predSamplesCr.

- …

- H.8.5.3.2.1	Derivation process for luma motion vectors for merge mode

…

  - The merge candidate lists mergeCandList and mergeCandIsVspFlag are constructed as specified by the following ordered steps:

  - The variable numMergeCand is set equal to 0.

  - When availableFlagT is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to T, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagIvMC is equal to 1,

      - When the following condition is true,

        - availableFlagT is equal to 0

- or one or more of the following conditions are true, with N being replaced by IvMC:

        - availableFlagN = = 1 && predFlagLXN ! = predFlagLXT, (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && mvLXN ! = mvLXT (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && refIdxLXN ! = refIdxLXT (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvMC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1, the following applies, with N being replaced by ( DepthFlag ? T : IvMC ):

      - When the following condition is true,

        - availableFlagNT = = 0  &&  availableFlagIvMC  = = 0,

        - or one or more of the following conditions are true, with N being replaced by T and IvMC:

        - availableFlagN = = 1  &&  predFlagLXN  !=  predFlagLXA1, (with X being replaced by 0 and 1),

        - availableFlagN = = 1  &&  mvLXN  !=  mvLXA1 (with X being replaced by 0 and 1),

        - availableFlagN = = 1  &&  refIdxLXN  !=  refIdxLXA1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to A1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb + nPbH − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1, the following applies, with N being replaced by (DepthFlag ? T : IvMC ):

      - When the following condition is true ,

        - availableFlagNT  = =  0  &&  availableFlagIvMC = = 0,

or one or more of the following conditions is true, with N being replaced by T and IvMC:

        - availableFlagN  = =  1  &&  predFlagLXN  !=  predFlagLXB1, (with X being replaced by 0 and 1),

        - availableFlagN  = =  1  &&  mvLXN  !=  mvLXB1 (with X being replaced by 0 and 1),

        - availableFlagN  = =  1  &&  refIdxLXN  !=  refIdxLXB1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to B1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb + nPbW − 1 ][ yPb − 1 ] and the variable numMergeCand is increased by 1.

…

- When availableFlagA0 is equal to 1 and numMergeCand is less than 4 +( iv_mv_pred_flag[ nuh_layer_id ] || DepthFlag), the entry mergeCandList[ numMergeCand ] is set equal to A0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb + nPbH ] and the variable numMergeCand is increased by 1.

- When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + ( iv_mv_pred_flag[ nuh_layer_id ] +|| DepthFlag ), the entry mergeCandList[ numMergeCand ] is set equal to B2, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xPb − 1 ][ yPb − 1 ] and the variable numMergeCand is increased by 1.

- When availableFlagCol is equal to 1 and numMergeCand is less than 5 + ( iv_mv_pred_flag[ nuh_layer_id ] +|| DepthFlag ), the entry mergeCandList[ numMergeCand ] is set equal to Col, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

- …

  - When DepthFlag is equal to zero, tThe disparity availability flag ivpMvFlagLX is derived as follows (with X being replace by 0 or 1).

    - If one of the following conditions are true, ivpMvFlagLX is set equal to 1

- [Ed. (GT) There is some redundancy in draft and software since disparities equal for both lists.(#7) ]

      - predFlagLXIvMC  = =  1  &&  mergeCandList[ merge_idx[ xPb][ yPb ] ]  =  =  IvMC

      - predFlagLXIvMCShift  = =  1  &&  and mergeCandList[ merge_idx[ xPb][ yPb ] ]  = =  IvMCShift

[Ed. (GT): PredMode[ xCb ][ yCb ]  = =  MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

    - Otherwise, ivpMvFlagLX is set equal to 0.

- H.8.5.3.2.11	Derivation process for a temporal inter-view motion vector candidate

…

The reference layer luma location ( xRef, yRef ) is derived by

  - xRefFull = xPb + ( ( nPbW – 1 )  >>  1 ) + ( ( mvDisp[ 0 ] + 2 )  >>  2 )	(Error: Reference source not found-1)

  - yRefFull = yPb + ( ( nPbH – 1 )  >>  1 ) + ( ( mvDisp[ 1 ] + 2 )  >>  2 )	(Error: Reference source not found-2)

  - xRef = Clip3( 0, PicWidthInSamplesL – 1, (  xRefFull  >>  3 )  <<  3 )	(Error: Reference source not found-3)

  - yRef = Clip3( 0, PicHeightInSamplesL – 1, (  yRefFull  >>  3 )  <<  3 )	(Error: Reference source not found-4)

The variable ivRefPic is set equal to the picture with ViewIdx equal to refViewIdx in the current access unit.

The variable ivRefPb specifies the luma prediction block covering the location given by ( xRef, yRef ) inside the inter-view reference picture specified by ivRefPic.

…

- H.8.5.3.2.12	Derivation process for a disparity inter-view motion vector candidate

…

For each i from 0 to num_ref_idx_lX_active_minus1, inclusive, the following applies:

  - When PicOrderCnt( RefPicListX[ i ] ) is equal to the PicOrderCntVal, ViewIdx( RefPicListX[ i ] ) is equal to refViewIdx and availableFlagLXInterView is equal to 0 the following applies:

        - availableFlagLXInterView = 1 		(Error: Reference source not found-5)

        - mvLXInterView[ 0 ] = DepthFlag ? mvDisp[ 0 ] : (mvDisp[ 0 ]+2)>>2	(Error: Reference source not found-6)

        - mvLXInterView[ 1 ] = 0		(Error: Reference source not found-7)

      - refIdxLX = i		(Error: Reference source not found-8)

- H.8.5.5.3	Derivation process for disparity vectors from neighboring reconstructed depth samples

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

Let curDepPels be an array of reconstructed depth samples of the current depth view component.

The disparity vector mvDisp are set equal to ( 0, 0 ).

The disparity vector mvDisp is derived by the following steps performed in order:

  - The variable avgDep is derived as follows:

    - If both xCb and yCb are greater than 0, avgDep is set equal to ( ( 5*curDepPels[ xCb-1 ][ yCb-1 ] + 5*curDepPels[ xCb-1 ][ yCb+nCbS-1 ] + 6*curDepPels[ xCb+nCbS-1 ][ yCb-1 ] + 8 ) >> 4 );

    - Otherwise, if xCb is greater than 0 and yCb is equal to 0, avgDep is set equal to curDepPels[ xCb-1 ][ yCb+nCbS-1 ];

    - Otherwise, if xCb is equal to 0 and yCb is greater than 0, avgDep is set equal to curDepPels[ xCb+nCbS-1 ][ yCb-1 ];

    - Otherwise, (when xCb is equal to 0 and yCb is equal to 0), avgDep is set equal to -1.

  - When avgDep is not equal to -1, mvDisp[ 0 ] is derived as specified in the following

    - mvDisp[ 0 ] = DepthToDisparityB[ 0 ][ avgDep ]

- For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xCb.. ( xCb + nCbS − 1 ), y = yCb..( yCb + nCbS− 1 ):

MvRefinedDisp[ x ][ y ] = mvDisp; 
RefViewIdx[ x ][ y ] = 0