| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0125 |
| --- | --- |

| Title: | CE3: Inter-view motion vector prediction for depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-658-5536 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In 3D-HEVC, Inter-View Motion Prediction (IVMP) is utilized for texture coding by adding new merge candidates. In this follow-up contribution of JCT3V-E0133, similar to texture coding, it is proposed to apply IVMP for depth coding. Two major aspects are proposed to enable IVMP in depth coding. Firstly, a disparity vector is derived from the neighboring reconstructed depth pixels and more merge candidates are derived by using the motion information from the reference depth views. Secondly, the center pixel, which is used to identify a reference block, is proposed to be shifted by (1, 1) for coding efficiency improvement. The additional merge candidates for depth coding are generated in the same way as in texture coding in the current 3D-HEVC. Experimental results report that the proposed scheme gives an overall synthesized coding gain of 0.3% when compared to the CTC results.

# Introduction

In 3D-HEVC [1] depth coding, up to six merge candidates are available that is constructed as follows:

- Motion parameter Inheritance (MPI) candidate is derived from the co-located texture block and it added to the merge candidate list, if available.

- Spatial merging candidates derived from neighboring prediction unit (PU) partitions are added to the merge candidate list. This process is similar to that in HEVC, where redundant candidates among the initial available spatial candidates are eliminated from the merging list by comparing motion information of the candidates. In addition, the spatial merging candidates derived from A1 and B1 are compared with MPI to eliminate the redundancy. Figure 1 depicts the spatial neighbours (A1, B1, B0, A0, B2) which are the potential candidates for the merging list.

- Other motion vector merging candidates (including temporal/combined bi-predictive/zero motion vector merging candidates) are added to the merge candidate list if the number of valid candidates is less than the maximum number.

Figure 1. Potential spatial neighbouring candidates for the merge candidate list. Arrows indicate which spatial candidate(s) are to be compared.

In 3D-HEVC texture coding, up to six merge candidates are available for selection when IVMP is enabled. When IVMP is enabled, the merge candidate list is constructed as follows:

- A disparity vector is derived and the corresponding block of current PU based on the disparity vector is identified. The Inter-view Predicted Motion Candidate (IPMC), if derived to be available, is added to the merge candidate list.

- Spatial merging candidates (SMC) A1, B1 and B0 are added to the merge candidate list as described in the current design.

- Disparity motion vector converted from the disparity vector is added to the merge candidate list, if they are different from the A1 and B1.

- The BVSP merging candidate is inserted to the merge candidate list, if BVSP is enabled for current slice.

- Spatial merging candidates (SMC) A0 and B2 are added to the merge candidate list as described in the current design.

- If IPMC was derived to be the additional candidate, it is compared with the IPMC in step 1. If it is not equal to the IPMC in step 1 or if the additional candidate is a DSMV (when the additional IPMC is unavailable), the generated additional candidate is inserted.

- Temporal merging candidate is added to the merge candidate list as the current design.

- Other motion vector merging candidates are added to the merge candidate list if the number of valid candidates is less than the maximum number.

The current design of motion related technologies for the depth coding in 3D-HEVC may have the following problem:

- The motion vector predictive coding of the non-base depth views does not utilize the motion information of the already coded reference (base) depth views for the merge/skip modes. In other words, the correlation between the motion information (including reference index and motion vectors) of the block in one non-base depth view and its corresponding block in its reference depth view is not exploited.

Further coding efficiency improvement may be expected by including the same IVMP candidates in the merge candidate list.

# Proposed scheme

As a follow-up proposal of JCT3V-E0133 [2], this proposal proposes to derive more candidates from the already coded motion vectors of the reference depth views. To derive the merge candidates, a disparity vector is firstly derived from the neighbouring reconstructed depth values. Then, additional merge candidates are generated based on this derived disparity vector and added into the merge candidate list.

## Generation of the disparity vector

A single disparity vector is derived for each CU from a single depth value. The depth value is calculated from the three neighboring samples that are adjacent to the corners of the current block. As shown in Figure 2, three neighboring sample positions namely above-left, above-right and bottom-left, as identified by red blocks in Figure 2, are used.

A single depth value from the three neighboring samples is calculated as

Depth = ( 5*D[ xC-1 ][ yC-1 ] + 5*D[ xC-1 ][ yC + 2N-1 ] + 6*D[ xC + 2N-1 ][ yC-1 ] + 8 )>>4

Here D[x][y] represents the reconstructed depth value at location (x, y) and (xC, yC) represents the top-left corner of the current CU with a size of 2Nx2N.

Note that the weighted average as shown above for the three pixels are utilized maybe to avoid the division by 3.

The weighted average depth value is directly converted to a disparity vector that is denoted as DV.

The derived disparity vector DV is applied to all the PU’s therein.

Figure 2: Derivation of a disparity vector from the three (red) neighboring reconstructed samples.

## Generation of the inter-view motion vector candidates

For depth coding, the disparity vector DV can be converted into an inter-view disparity motion vector and it may be added to the merge candidate list. The disparity vector DV = (mvX, 0) is converted into a disparity motion vector as ((mvX+2) >> 2, 0).

In addition, instead of always using the center position namely C0 with a relative coordination of (xP + ( ( nPSW – 1 )  >>  1 ) + ( ( DV[ 0 ] + 2 )  >>  2), yP + ( ( nPSH – 1 )  >>  1 ) + ( ( DV[1] + 2 )  >>  2 ) ), the C0 position is shifted by (1,1) for the identification of a reference block. More precisely, the location of the reference pixel position (xRef, yRef) in the reference base view is calculated as

xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( (nPSW-1)  >>  1 ) + ( (DV[ 0 ] + 2 )  >>  2 ) )
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( (nPSH-1)  >>  1 ) + ( (DV[ 1 ] + 2 )  >>  2 ) )

Here (xP, yP) represents the top-left corner of the current PU with a size of nPSW x nPSH.

Note that the change of the reference pixel helps for the texture coding as well.

Furthermore, similar to texture coding, additional candidates are also made available for selection in depth coding as follows:

- An additional candidate generated by shifting the disparity vector according to the size of the current PU and utilizing the motion information of an identified block in the reference view.

- If the above identified block is not an available candidate, the candidate is derived by shifting horizontally (by 4) the inter-view disparity motion vector.

## Merge candidate list construction with additional candidates

The merge candidate list for the depth views are generated using the following steps, and the merging candidates related to IPMC are added in the same way as applied in texture coding. The newly introduced candidates are highlighted in green color.

- MPI is derived by the procedure described above. If it is available, it is inserted to the merge list.

- If the IPMC derived from the base view is available, and it is different from the MPI candidate it is inserted into the merge candidate list; otherwise IPMC is not inserted in the list.

- Spatial merging candidates (SMC) A1 and B1 are added to the merge candidate list as described in the current design.

- The disparity vector is converted into a disparity motion vector and it is inserted in the list if it is different from the spatial candidates A1 and B1; otherwise it is not inserted in the list.

- Spatial merging candidates (SMC) B0, A0 and A2 are added to the merge candidate list as described in the current design.

- If IPMC was derived to be the additional candidate, it is compared with the IPMC in step 2. If it is not equal to the IPMC in step 2 or if the additional candidate is a DSMV (when the additional IPMC is unavailable), the generated additional candidate is inserted.

- Other motion vector merging candidates (including temporal/combined bi-predictive/zero motion vector merging candidates) are added to the merge candidate list if the number of valid candidates is less than the maximum number.

It should be noted that the candidate list size is not changed (still six), once there are enough candidates in the candidate list, no more candidates will be further added.

# Experimental results

The proposed method is integrated into current 3D-HTM 8.0 software [3] and the simulation tests are performed based on the common test conditions (CTC) [4].

The coding performance of the proposed scheme in comparison with the CTC results is given in Table 1. As reported in Table 1, the overall average bitrate reduction is 0.3% for the synthesized views.

Table 1: Coding gain with respect to anchor for 3-view case (proposed scheme vs. CTC)

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.6% | -0.5% | -0.2% | -0.2% | -0.3% | 103% | 100% | 98% |
| Kendo | -0.2% | -0.3% | -0.1% | -0.1% | -0.3% | 89% | 102% | 85% |
| Newspapercc | -0.1% | 0.0% | 0.0% | -0.1% | -0.3% | 95% | 100% | 90% |
| GhostTownFly | -0.3% | -0.3% | -0.1% | 0.0% | -0.2% | 91% | 101% | 90% |
| PoznanHall2 | -0.1% | -0.3% | -0.1% | 0.0% | -0.4% | 104% | 100% | 98% |
| PoznanStreet | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 85% | 101% | 82% |
| UndoDancer | -0.4% | -0.3% | -0.1% | -0.1% | -0.4% | 103% | 101% | 95% |
| 1024x768 | -0.3% | -0.3% | -0.1% | -0.1% | -0.3% | 96% | 101% | 91% |
| 1920x1088 | -0.2% | -0.2% | -0.1% | 0.0% | -0.3% | 96% | 101% | 91% |
| average | -0.2% | -0.2% | -0.1% | -0.1% | -0.3% | 96% | 101% | 91% |
| Shark | -0.4% | -0.3% | -0.1% | 0.0% | -0.2% | 107% | 103% | 104% |

The coding performance of the proposed scheme is compared to MPI by replacing the existing MPI candidate with the proposed depth IPMC candidate.

The following two configurations are compared:

- Cfg #1 (Anchor): MPI is disabled, i.e., MPI candidate is not inserted in the candidate list.

- Cfg #2 (Tested): MPI is disabled and proposed depth IPMC candidate is enabled.

Table 2: Proposed method with MPI disabled versus CTC with MPI disabled

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.7% | -0.4% | -0.2% | -1.1% | -1.5% | 100% | 105% | 102% |
| Kendo | -0.1% | -0.4% | -0.1% | -1.0% | -1.7% | 100% | 99% | 100% |
| Newspapercc | -0.1% | -0.1% | 0.0% | -0.8% | -1.2% | 97% | 99% | 97% |
| GhostTownFly | -0.2% | -0.3% | -0.1% | 0.0% | -0.6% | 93% | 97% | 93% |
| PoznanHall2 | -0.2% | -0.1% | -0.1% | -1.9% | -2.9% | 101% | 103% | 101% |
| PoznanStreet | -0.1% | 0.1% | 0.0% | -0.7% | -1.2% | 101% | 106% | 100% |
| UndoDancer | -0.3% | -0.2% | -0.1% | -0.7% | -1.4% | 97% | 101% | 95% |
| 1024x768 | -0.3% | -0.3% | -0.1% | -1.0% | -1.5% | 99% | 101% | 100% |
| 1920x1088 | -0.2% | -0.1% | -0.1% | -0.8% | -1.5% | 98% | 102% | 97% |
| average | -0.2% | -0.2% | -0.1% | -0.9% | -1.5% | 98% | 101% | 98% |
| Shark | -0.2% | -0.3% | -0.1% | -0.3% | -0.7% | 98% | 102% | 95% |

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1001, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

- V. Thirumalai, L. Zhang, Y. Chen, M. Karczewicz, “Inter-view motion vector prediction for depth coding,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E0133, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

- 3DV-HTM version 8.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1100, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).