| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0126_proposed_text |
| --- | --- |

| Title: | Proposed text for JCT3V-F0126 based on 3D-HEVC Draft Text 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen Li Zhang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 86-10-57760696 hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

Based on 3D-HEVC draft text 1, the changes are highlighted in green or red strikethrough.

# Syntax

- H.7.3.9.3 Depth mode parameter syntax

| depth_mode_parameters( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| if( ( log2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( log2CbSize > 3 && log2CbSize < 6 ) ) |  |
| depth_intra_mode_set_indication_flag | ae(v) |
| if ( log2CbSize < 6 ) |  |
| hevc_intra_flag[ x0 ][ y0 ] | ae(v) |
| if ( PartMode[ xC ][ yC ] = = PART_2Nx2N ) |  |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| if ( !hevc_intra_flag[ x0 ][ y0 ] ) { |  |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ¦ ¦ DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WPREDTEX ) |  |
| wedge_predtex_tab_idx[ x0 ][ y0 ] | ae(v) |
| else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_start_left_flag[ x0 ][ y0 ] | ae(v) |
| edge_start_position[ x0 ][ y0 ] | ae(v) |
| num_edge_codes_minus1[ x0 ][ y0 ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| } |  |
| } |  |
| if (!sdc_flag[ x0 ][ y0 ] ) { |  |
| if ( DmmFlag[ x0 ][ y0 ] ) { |  |
| dmm_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( dmm_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < 2; i ++ ) { |  |
| dmm_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( dmm_dc_abs[ x0 ][ y0 ][ i ]) |  |
| dmm_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_dc_flag[ x0 ][ y0 ] | ae(v) |
| if( edge_dc_flag[ x0 ][ y0 ] ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| edge_dc_abs[ x0 ][ y0 ][ i ] | - ae(v) |
| if( edge_dc_abs[ x0 ][ y0 ][ i ] != 0 ) |  |
| edge_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| else if( SdcFlag[ x0 ][ y0 ] ) { |  |
| sdcNumSegments = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL !hevc_intra_flag[ x0 ][ y0 ] ) ? 2 : 1 |  |
| for( i = 0; i < sdcNumSegments; i++ ) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i ] | ae(v) |
| if( sdc_residual_flag[ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |

- H.7.3.9.1 General Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { |  |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| … |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| log2PbSize = log2CbSize − ( PartMode = = PART_NxN ? 1 : 0 ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( vps_depth_modes_flag[ nuh_layer_id ] ) |  |
| depth_mode_parameters( x0 + i , y0+ j , log2CbSize ) |  |
| if( DepthIntraMode[ x0 + i ][ y0 + j ] = = INTRA_DEP_NONE ) |  |
| if( hevc_intra_flag[ x0 ][ y0 ]) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( DepthIntraMode[ x0 + i ][ y0 + j ] = = INTRA_DEP_NONE) { |  |
| if( hevc_intra_flag[ x0 ][ y0 ]) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| } |  |
| if ( !SdcFlag[ x0 ][ y0 ] ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf && !inter_sdc_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |

# Semantics

- H.7.4.9.3 Depth mode parameter semantics

depth_intra_mode_set_indication_flag indicates the set of possible depth intra modes.

The variables depthIntraModeSet is derived as specified in the following:

              - If cLog2CbSize is equal to 6, depthIntraModeSet is set equal to 0.

              - Otherwise, if cLog2CbSize is equal to 3 and PartMode[ xC ][ yC ] is equal to PART_NxN, depthIntraModeSet is set equal to 1.

              - Otherwise ( (cLog2CbSize = = 3 && PartMode[ xC ][ yC ]  = =  PART_2Nx2N) || (cLog2CbSize > 3 && cLog2CbSize < 6 ), the following applies:

              - If depth_intra_mode_set_indication_flag is equal to 1, depthIntraModeSet is set equal to 2.

              - Otherwise ( depth_intra_mode_set_indication_flag is equal to 0 ), depthIntraModeSet is set equal to 3.

hevc_intra_flag[ x0 ][ y0 ] equal to 1 specifies that intra modes with intraPredMode in the range of 0 to 34 is used for the current prediction unit. hevc_intra_flag[ x0 ][ y0 ] equal to 0 specifies that another intra mode is used for the current prediction unit. When not present, hevc_intra_flag[ x0 ][ y0 ] is inferred to be equal to 1.

sdc_flag[ x0 ][ y0 ] equal to 1 specifies that the residual of each partition of the current prediction unit is only represented by sdc_residual_flag[ x0 ][ y0 ][ i ], sdc_residual_sign_flag [ x0 ][ y0 ][ i ] and sdc_residual_abs_minus1[ x0 ][ y0 ][ i ]. sdc_flag[ x0 ][ y0 ] equal to 0 specifies that more residual may be present in the transform tree. When not present, sdc_flag[ x0 ][  y0 ] is inferred to be equal to 0.

depth_intra_mode[ x0 ][ y0 ] specifies the depth intra mode of the current prediction unit. Table Error: Reference source not found-1 Error: Reference source not foundspecifies the value of the variable depthIntraModeMaxLen depending on depthDmmRbcModeSetdepthIntraModeSet and the value of the variable DepthIntraMode and the associated name depending on the on depth_intra_mode and depthDmmRbcModeSetdepthIntraModeSet. When depth_intra_mode[ x0 ][ y0 ] is not present, DepthIntraMode is inferred to be equal to 4.

The variables depthDmmRbcModeSet is derived as specified in the following:

              - If cLog2CbSize is equal to 3 and PartMode[ xC ][ yC ] is equal to PART_NxN, depthDmmRbcModeSet is set equal to 1.

              - Otherwise (cLog2CbSize is not equal to 3 or PartMode[ xC ][ yC ] is not equal to PART_NxN ), depthDmmRbcModeSet is set equal to 0.

Table Error: Reference source not found-1 – Specification of DepthIntraMode and associated name depending on depthDmmRbcModeSet and depth_intra_mode and specification of and depthIntraModeMaxLen depdending on depthDmmRbcModeSet

|  | depthDmmRbcModeSet | 0 | 1 |
| --- | --- | --- | --- |
|  | depthIntraModeMaxLen | 3 | 2 |
| DepthIntraMode | Associated name | depth_intra_mode |  |
| 0 | INTRA_DEP_DMM_WFULL | 0 | 0 |
| 1 | INTRA_DEP_DMM_CPREDTEX | 2 | - |
| 2 | INTRA_DEP_DMM_WPREDTEX | 6 | 2 |
| 3 | INTRA_DEP_CHAIN | 7 | 3 |

The variable SdcFlag[ x0 ][ y0 ] is derived as specified in the following:

  - SdcFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_PLANAR )  | |  	(Error: Reference source not found-1)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_DMM_WFULL )

- SdcFlag[ x0 ][ y0 ] is set to be equal to sdc_flag[ x0 ][ y0 ].

The variable DmmFlag[ x0 ][ y0 ] is derived as specified in the following:

  - DmmFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_WFULL )  | |  	(Error: Reference source not found-2)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_CPREDTEX )  | |  
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_WPREDTEX )

# Decoding process

- H.8.4.2 Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PbSize specifying the size of the current luma prediction block.

Table Error: Reference source not found-2 specifies the value for the intra prediction mode and the associated names.

Table Error: Reference source not found-2 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_DC |
| 2..34 | Intra_Angular |
| 35...40 | Intra_DepthPartition (used only for depth) |
| Otherwise (41, 42) | Intra_Chain (used only for depth) |

IntraPredMode[ xB ][ yB ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1.

              - If DepthIntraMode[ xB ][ yB ] is equal to INTRA_DEP_SDC_PLANAR, IntraPredMode[ xB ][ yB ] is set equal to Intra_Planar.

              - Otherwise, if DepthIntraMode[ xB ][ yB ] is equal to INTRA_DEP_SDC_DMM_WFULL, IntraPredMode[ xB ][ yB ] is set equal to Intra_DepthPartition( 35 ).

              - Otherwise, iIf DepthIntraMode[ xB ][ yB ] is equal to INTRA_DEP_DMM_WFULL, IntraPredMode[ xB ][ yB ] is set equal to Intra_DepthPartition( 35 + dmm_dc_flag[ xB ][ yB ] ) .

              - Otherwise, if DepthIntraMode[ xB ][ yB ] is equal to INTRA_DEP_DMM_WPREDTEX, IntraPredMode[ xB ][ yB ] is set equal to Intra_DepthPartition( 37 + dmm_dc_flag[ xB ][ yB ] ) .

              - Otherwise if DepthIntraMode[ xB ][ yB ] is equal to INTRA_DEP_DMM_CPREDTEX, IntraPredMode[ xB ][ yB ] is set equal to Intra_DepthPartition( 39 + dmm_dc_flag[ xB ][ yB ] ) .

              - Otherwise if DepthIntraMode[ xB ][ yB ] is equal to INTRA_DEP_CHAIN, IntraPredMode[ xB ][ yB ] is set equal to Intra_Chain( 41  + edge_dc_flag[ xB ][ yB ] ).

              - Otherwise ( DepthIntraMode[ xB ][ yB ] is equal to 4 INTRA_DEP_NONE ), IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

…

- H.8.4.4.2.1 General intra sample prediction

The specification in subclause 8.4.4.2.1 with the following paragraphs added to the end of subclause apply:

Set variable bSamplePredFlag to 1.

              - If intraPredMode is equal to Intra_DepthPartition(35,36), the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, the sample array p and the transform block size nT as the inputs and the output are the predicted sample array predSamples.

              - Otherwise, if intraPredMode is equal to Intra_DepthPartition(37,38), the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, the sample array p and the transform block size nT and bSamplePredFlag  as the inputs and the output are the predicted sample array predSamples.

              - Otherwise, if intraPredMode is equal to Intra_DepthPartition(39,40), the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, with the sample array p and the transform block size nT and bSamplePredFlag as the inputs and the output are the predicted sample array predSamples.

              - Otherwise, if intraPredMode is equal to Intra_Chain(41,42), the corresponding intra prediction mode specified in subclause Error: Reference source not found10 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, with the sample array p and the transform block size nT  and bSamplePredFlag as the inputs and the output are the predicted sample array predSamples.

- H.8.4.4.3 Depth value reconstruction process

Inputs to this process are:

              - a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

              - a variable nT specifying the prediction size

              - predicted samples predSamples[ x ][ y ], with x, y =0..nT−1

              - a variable intraPredMode specifying the prediction mode of the current prediction block

Output of this process is:

              - reconstructed depth value samples resSamples[ x ][ y ], with x, y = −1..2*nT−1.

Set variable bSamplePredFlag to 0.

Depending on intraPredMode the array wedgePattern[ x ][ y ] with x, y =0..nT−1 specifying the binary segmentation pattern is derived as follows.

              - If intraPredMode is equal to Intra_DepthPartition( 35 ), the following applies.

- wedgePattern = WedgePatternTable[ Log2( nT) ][ wedge_full_tab_idx[ xB ][ yB ] ]

              - Otherwise, if intraPredMode is equal to Intra_DepthPartition( 37 ), subclause H.8.4.4.2.8 is invoked with the location ( xB, yB ), the intra prediction mode intraPredMode, the transform block size nT and bSamplePredFlag as the inputs and the output is the wedgelet pattern wedgePattern.

              - Otherwise, if intraPredMode is equal to Intra_DepthPartition( 39 ), subclause H.8.4.4.2.9 is invoked with the location ( xB, yB ), the intra prediction mode intraPredMode, the transform block size nT and bSamplePredFlag as the inputs and the output is the wedgelet pattern wedgePattern.

              - Otherwise, if intraPredMode is equal to Intra_Chain( 41 ), subclause H.8.4.4.2.10 is invoked with the location ( xB, yB ), the intra prediction mode intraPredMode, the transform block size nT and bSamplePredFlag as the inputs and the output is the edge pattern wedgePattern.

              - Otherwise ( intraPredMode is smaller than not equal to Intra_DepthPartition( 35 ) ), the following applies.

              - For x, y = 0..nT − 1 wedgePattern[ x ][ y ] is set equal to 0.

Depending on dlt_flag[ nuh_layer_id ] the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

              - If dlt_flag[ nuh_layer_id ] is equal to 0, the following applies:

              - For x, y = 0..nT − 1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

- resSamples[ x ][ y ] = predSamples[ x ][ y ] + 	SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-3)

              - Otherwise ( dlt_flag[ nuh_layer_id ] is equal to 1 ), the following applies:

              - For p in the range of 0 to 1variables dcPred[ 0 ] and dcPred[ 1 ] are derived as specified in the following:

              - If intraPredMode is equal to Intra_DC, the following applies:

- dcPred[ 0 ] = predSamples[ nT − 1 ][ nT − 1 ]		(Error: Reference source not found-4)

              - Otherwise, If intraPredMode is smaller than 35 equal to Intra_Planar, the following applies:

- dcPred[ 0 ] = ( predSamples[ 0 ][ 0 ] + predSamples[ 0 ][ nT − 1 ] + predSamples[ nT − 1 ][ 0 ]
		+ predSamples[ nT − 1 ][ nT − 1 ] + 2 )  >>  2	(Error: Reference source not found-5)

              - Otherwise, if ( intraPredMode is equal to Intra_DepthPartition( 35 ) or intraPredMode is equal to Intra_DepthPartition( 37 ) ), the following applies.

- dcPred[ wedgePattern[ 0 ][ 0 ] ] = predSamples[ 0 ][ 0 ]	(Error: Reference source not found-6)

- dcPred[ wedgePattern[ nT − 1 ][ 0 ] ] = predSamples[ nT − 1 ][ 0 ]	(Error: Reference source not found-7)

- dcPred[ wedgePattern[ 0 ][ nT − 1 ] ] = predSamples[ 0 ][ nT − 1 ]	(Error: Reference source not found-8)

- dcPred[ wedgePattern[ nT − 1 ][ nT − 1 ] ] = predSamples[ nT − 1 ][ nT − 1 ]	(Error: Reference source not found-9)

              - Otherwise, (intraPredMode is equal to Intra_DepthPartition( 39 ) or intraPredMode is equal to Intra_Chain( 41 )), the following applies.

- dcPred[wedgePattern[0][0]] = predSamples[ 0 ][ 0 ]

- dcPred1Found = 0;

for ( x = 0; x <  nT – 1; x++ )
		for ( y = 0; y < nT – 1; y++ )
			if ( wedgePattern[x][y] != wedgePattern[0][0] && dcPred1Found == 0 )  {
				dcPred[wedgePattern[x][y]] = predSamples[x][y]
				dcPred1Found = 1
			}

              - For x, y = 0..nT − 1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

- dltIdxPred = DepthValue2Idx[ dcPred[ wedgePattern[ x ][ y ] ] ] 	(Error: Reference source not found-10)
dltIdxResi = SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-11)
resSamples[ x ][ y ] =	predSamples[ x ][ y ]  + 	Idx2DepthValue[ dltIdxPred + dltIdxResi ]  − 
	dcPred[ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-12)

- H. 8.4.4.2.8 Specification of Intra_DepthPartition (37, 38) prediction mode

Inputs to this process are:

              - a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

              - a variable intraPredMode specifying the intra prediction mode,

              - the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

              - a variable nT specifying the transform size,

              - a variable bSamplePredFlag specifying whether to generate the prediction samples or not

Output of this process is:

              - predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

              - wedgelet pattern wedgePattern[ x ][ y ], with x, y = 0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 37 or 38.

Let textureComp be the texture view component with view order index equal to ViewIdx and DepthFlag equal to 0. The variable texturePredMode is set equal to the PredMode[ xB ][ yB ] of textureComp. The variable textureIntraPredMode is set equal to IntraPredMode[ xB ][ yB ] of the textureComp.

[Ed. (GT) Consider storing texturePredMode and textureIntraPredMode explicitly.]

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived by the following ordered steps.

  - Depending on texturePredMode and textureIntraPredMode, the list candWedgeIndList specifying indices of candidate wedgelets is derived as specified in the following:

              - If texturePredMode is equal to MODE_INTRA and textureIntraPredMode is in the range of 2 to 34, the following applies.

- candWedgeIndList = TextModePredWedgeIndTable[ Log2( nT ) ][ texturePredMode ]	(Error: Reference source not found-13)

              - Otherwise ( texturePredMode is not equal to MODE_INTRA or textureIntraPredMode is not in the range of 2 to 34), the following applies.

- candWedgeIndList = CoarseWedgeIndTable[ Log2( nT ) ]	(Error: Reference source not found-14)

  - The variable wedgeIdx is derived as:

- wedgeIdx = candWedgeIndList[ wedge_predtex_tab_idx[ xB ][ yB ] ]	(Error: Reference source not found-15)

  - The variable wedgePattern [ x ][ y ] with x, y =0..nT−1, specifying a binary partition pattern is derived as.

- wedgePattern = WedgePatternTable[ Log2( nT) ][ wedgeIdx]	(Error: Reference source not found-16)

  - When bSamplePredFlag is equal to 1, the depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(38) ), intraChainFlag set equal to 0, and the DC Offsets DcOffset[ xB ][ yB ][ 0 ], and DcOffset[ xB ][ yB ][ 1 ] as inputs and the output is assigned to predSamples[ x ][ y ].

- H. 8.4.4.2.9 Specification of Intra_DepthPartition (39, 40) prediction mode

Inputs to this process are:

              - a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

              - a variable intraPredMode specifying the intra prediction mode,

              - the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

              - a variable nT specifying the transform size,

              - a variable bSamplePredFlag specifying whether to generate the prediction samples or not

Output of this process is:

              - predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

              - wedgelet pattern wedgeletPattern[ x ][ y ], with x, y = 0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 39 or 40.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified by the following ordered steps:

  - The variable recTextPic is set equal to the array of the reconstructed luma picture samples of the view component with view order index equal to ViewIdx and DepthFlag equal to 0.

  - The variable textThresh specifying a threshold for the segmentation of recTextPic is derived as specified in the following.

              - The variable sumTextPicVals is set equal to 0.

              - For x = 0..nT − 1 the following applies

              - For y = 0..nT − 1 the following applies

sumTextPicVals += recTextPic[ xB + x ][ yB + y ]	(Error: Reference source not found-17)

- The variable textThresh is set equal to ( sumTextPicVals  >>  ( 2 * log2( nT ) ) )

  - The variable wedgeletPattern[ x ][ y ] with x, y =0..nT−1 specifying a binary partition pattern is derived as specified in the following.

              - For x = 0..nT−1 the following applies

              - For y = 0..nT−1 the following applies

wedgeletPattern[ x ][ y ] = ( recTextPic[ xB + x ][ yB + y ] > textThresh )	(Error: Reference source not found-18)

  - When bSamplePredFlag is equal to 1, the depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgeletPattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(40) ), intraChainFlag set equal to 0, and the DC Offsets DcOffset[ xB ][ yB ][ 0 ], and DcOffset[ xB ][ yB ][ 1 ] as inputs and the output is assigned to predSamples[ x ][ y ].

- H. 8.4.4.2.10 Specification of Intra_Chain (41, 42) prediction mode

Inputs to this process are:

              - a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

              - a variable intraPredMode specifying the intra prediction mode,

              - the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

              - a variable nT specifying the transform size,

              - a variable bSamplePredFlag specifying whether to generate the prediction samples or not

Output of this process is:

              - predicted samples predSamples[ x ][ y ], with x, y = 0..nT−1.

              - edge pattern edgePattern[ x ][ y ], with x, y = 0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 43 or 44.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified in the following ordered steps.

- [Ed. (GT) The process of edge reconstruction (items 1 - 3) need to be specified more in detail. In current state the specification is not precise enough. (#3)]

  - The start point of the chain in the current prediction unit is derived from edge_start_left_flag[ xB ][ yB ] and edge_start_position[ xB ][ yB ].

- If edge_start_left_flag[ xB ][ yB ] is equal to 1, the start point is set as s[ 0 ][ yS ] where yS is equal to edge_start_position[ xB ][ yB ].

- Otherwise, the start point is set as s[ xS ][ 0 ] where xS is equal to edge_start_position[ xB ][ yB ].

  - The number of edges and the direction of each edge are derived from num_edge_codes_minus1[ xB ][ yB ] and edge_code[ xB ][ yB ][ k ], for k = 0.. num_edge_codes_minus1[ xB ][ yB ]. The direction of the edge is derived from edge_code as shown in Error: Reference source not found.

  - The region boundary generated by connecting each edge separates the predicted samples predSamples[ x ][ y ], with x, y = 0..nT−1 into two regions: the region rA that covers the top-left pixel (x0, y0) and the region rB that covers the remaining region.

For x, y = 0..nT−1 the binary pattern edgePattern[ x ][ y ] is derived as follows:

- If (x, y) is covered by rA, edgePattern[ x ][ y ] =  0.

- Otherwise, edgePattern[ x ][ y ] = 1.

  - If bSamplePredFlag is equal to 1, the depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with the neighbouring samples p[ x ][ y ], the binary pattern edgePattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(44) ), intraChainFlag set equal to 1, and the DC offsets DcOffset[ xB ][ yB ][ 0 ], and DcOffset[ xB ][ yB ][ 1 ] as inputs and the output is assigned to predSamples[ x ][ y ].

- H.9.3.1.1 Initialization process for context variables

Table Error: Reference source not found-3 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) depth_mode_parameters( ) | depth_intra_mode | Table Error: Reference source not found-4 | 0..1 | 2..3 | 4..5 |
|  | sdc_flag | Error: Reference source not found22 | 0..2 | 3..5 | 6..8 |
|  | hevc_intra_flag | Error: Reference source not found3 | 0..2 | 3..5 | 6..8 |
|  | ... |  |  |  |  |

Table Error: Reference source not found-4 – Values of variable initValue for depth_intra_mode ctxIdx

| Initialization variable | depth_intra_mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 |

Table Error: Reference source not found-52 – Values of variable initValue for sdc_flag ctxIdx

| Initialization variable | sdc_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

Table Error: Reference source not found-63 – Values of variable initValue for hevc_intra_flag ctxIdx

| Initialization variable | hevc_intra_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

- H.9.3.2 Initialization process for context variables

Table Error: Reference source not found-7 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |
| depth_intra_mode | 0 | TU, cMax = depthIntraModeMaxLen | 0 | Table Error: Reference source not found-4 | 0 |
|  | 1 |  | 0 | Table Error: Reference source not found-4 | 2 |
|  | 2 |  | 0 | Table Error: Reference source not found-4 | 4 |
| sdc_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 3 |
|  | 2 |  | 0 | Error: Reference source not found | 6 |
| hevc_intra_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 3 |
|  | 2 |  | 0 | Error: Reference source not found | 6 |
| ... |  |  |  |  |  |

- H.9.3.3.1.1 Derivation process of ctxIdxInc using left and above syntax elements

Table Error: Reference source not found-35 – Specification of ctxIdxInc using left and above syntax elements for depth_intra_mode

| Syntax element | ctxIdxInc |
| --- | --- |
| depth_intra_mode | cLog2CbSize == 3 && PartMode[ x0 ][ y0 ] |

Table Error: Reference source not found-35 – Specification of ctxIdxInc using left and above syntax elements for sdc_flag

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| sdc_flag | sdc_flag[ xNbL ][ yNbL ] | sdc_flag[ xNbA ][ yNbA ] | ( condL ¦¦ !availableL ¦¦ !intra_mode_flag[ xNbL ][ yNbL ] ) + ( condA ¦¦ !availableA ¦¦ !intra_mode_flag[ xNbA ][ yNbA ] ) |

Table Error: Reference source not found-35 – Specification of ctxIdxInc using left and above syntax elements for hevc_intra_flag

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| hevc_intra_flag | hevc_intra_flag[ xNbL ][ yNbL ] | hevc_intra_flag[ xNbA ][ yNbA ] | ( condL ¦¦ !availableL ¦¦ !intra_mode_flag[ xNbL ][ yNbL ] ) + ( condA ¦¦ !availableA ¦¦ !intra_mode_flag[ xNbA ][ yNbA ] ) |