| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0126 |
| --- | --- |

| Title: | CE5 related: Generic SDC for all Intra modes in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen Li Zhang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 86-10-57760696 hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes an additional depth residual coding method for HEVC intra modes and region boundary chain coding (RBC) mode. Similar with simplified depth coding (SDC) in 3D-HEVC, only a DC residual is coded for each partition within a prediction unit, and transform and quantization are skipped. It is reported that -0.40% (and -0.55% coding gain for Shark sequence) average coding gain on synthesized view is achieved for the first 7 sequences in CTC case.

# Introduction

In 3D-HEVC, SDC is proposed to reduce the bit rate of depth residual. In SDC, only one DC residual is coded for each partition within a prediction unit (PU) coded in intra prediction mode. Optionally, depth lookup table (DLT) can be used to code the DC residual. When DLT is used, DC residual is generated as the difference between DLT index of original DC signal and DLT index of predicted DC signal. Because the range of indices to the DLT table is usually much smaller than the range of depth sample, DLT can further reduce bit rate of residual.

However, SDC is applied only to Planar mode and DMM mode 1, and it does not work in HEVC intra modes (excluding Planar mode) or RBC mode.

DMM and RBC modes are more like PU partitioning/prediction modes, however SDC mode (including DLT) is mainly for the purpose of coding the residual after the prediction based on the different characteristics of the depth.

Therefore, it makes sense to extend the good functionality of the SDC mode also to the normal HEVC Intra modes.

# Proposed method

To further reduce the bit rate of depth residual, SDC is extended to all other depth intra prediction modes including HEVC intra modes (excluding Planar mode), DMM mode 3, DMM mode 4 and RBC mode. This not only makes the efficient prediction in HEVC to be possibly combined with the efficient residual coding introduced by SDC, but also unifies the existing syntax design related to various Intra modes.

Therefore, each of the prediction modes can work with the traditional HEVC (3D-HEVC) residual as well as the SDC residual coding.

## Detailed description

In SDC, one delta DC value needs to be generated for a whole PU or a partition.

To generate the delta DC, a predicted DC signal is calculated for each partition of the prediction unit (PU) as follows.

- HEVC intra modes

All pixels in the PU belong to one partition. Predicted DC signal is generated as the average of the four corner pixels of the pixel specific predictor of the PU.

- DMM3, DMM4 and RBC modes

DMM modes and RBC modes contains two partitions. For each partition, the predicted DC signal is set to the predictor of the partition directly since all pixels within a partition share one same predictor.

When DLT is not used, delta DC is generated as the difference between original DC signal and predicted DC signal; when DLT is used, delta DC is generated as the difference between DLT index of original DC signal and DLT index of predicted DC signal.

With predicted DC signal and delta DC, residual for all pixels with in a partition can be generated using same method with 3D-HEVC. After generating the residual, depth pixel can be reconstructed as the sum of its prediction and residual of the partition that it belongs to.

The proposed method is only applied to CU with 2Nx2N partition pattern, and is not applied when DMM modes or RBC mode already encode one delta DC.

Because delta DC based depth residual coding is applied to all depth intra modes, one flag is required for all depth intra modes to indicate whether delta DC residual is coded or pixel specific residual is coded.

## Signalling

Signaling of whether delta DC based depth residual coding is used is unified for all intra modes. Signaling of delta DC is same with that of SDC in 3D-HEVC.

The major syntax change is in the depth_mode_parameters( ) syntax table, other small syntax changes, as well as detailed semantics and decoding processes modifications are provided in the attached document with working draft text for this proposal.

Green part is new added part

Strikethrough part is deleted part

### Syntax

### H.7.3.9.3 Depth mode parameter syntax

| depth_mode_parameters( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| if( ( log2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( log2CbSize > 3 && log2CbSize < 6 ) ) |  |
| depth_intra_mode_set_indication_flag | ae(v) |
| if ( log2CbSize < 6 ) |  |
| hevc_intra_flag[ x0 ][ y0 ] | ae(v) |
| if ( PartMode[ xC ][ yC ] = = PART_2Nx2N ) |  |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| if ( !hevc_intra_flag[ x0 ][ y0 ] ) { |  |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ¦ ¦ DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WPREDTEX ) |  |
| wedge_predtex_tab_idx[ x0 ][ y0 ] | ae(v) |
| else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_start_left_flag[ x0 ][ y0 ] | ae(v) |
| edge_start_position[ x0 ][ y0 ] | ae(v) |
| num_edge_codes_minus1[ x0 ][ y0 ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| } |  |
| } |  |
| if (!sdc_flag[ x0 ][ y0 ] ) { |  |
| if ( DmmFlag[ x0 ][ y0 ] ) { |  |
| dmm_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( dmm_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < 2; i ++ ) { |  |
| dmm_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( dmm_dc_abs[ x0 ][ y0 ][ i ]) |  |
| dmm_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_dc_flag[ x0 ][ y0 ] | ae(v) |
| if( edge_dc_flag[ x0 ][ y0 ] ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| edge_dc_abs[ x0 ][ y0 ][ i ] | - ae(v) |
| if( edge_dc_abs[ x0 ][ y0 ][ i ] != 0 ) |  |
| edge_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| else if( SdcFlag[ x0 ][ y0 ] ) { |  |
| sdcNumSegments = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL !hevc_intra_flag[ x0 ][ y0 ] ) ? 2 : 1 |  |
| for( i = 0; i < sdcNumSegments; i++ ) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i ] | ae(v) |
| if( sdc_residual_flag[ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |

H.7.3.9.1 General Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { |  |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| … |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| log2PbSize = log2CbSize − ( PartMode = = PART_NxN ? 1 : 0 ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( vps_depth_modes_flag[ nuh_layer_id ] ) |  |
| depth_mode_parameters( x0 + i , y0+ j , log2CbSize ) |  |
| if( DepthIntraMode[ x0 + i ][ y0 + j ] = = INTRA_DEP_NONE ) |  |
| if( hevc_intra_flag[ x0 ][ y0 ]) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( DepthIntraMode[ x0 + i ][ y0 + j ] = = INTRA_DEP_NONE) { |  |
| if( hevc_intra_flag[ x0 ][ y0 ]) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| } |  |
| if ( !SdcFlag[ x0 ][ y0 ] ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf && !inter_sdc_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |

### Semantics

H.7.4.9.3 Depth mode parameter semantics

depth_intra_mode_set_indication_flag indicates the set of possible depth intra modes.

The variables depthIntraModeSet is derived as specified in the following:

              - If cLog2CbSize is equal to 6, depthIntraModeSet is set equal to 0.

              - Otherwise, if cLog2CbSize is equal to 3 and PartMode[ xC ][ yC ] is equal to PART_NxN, depthIntraModeSet is set equal to 1.

              - Otherwise ( (cLog2CbSize = = 3 && PartMode[ xC ][ yC ]  = =  PART_2Nx2N) || (cLog2CbSize > 3 && cLog2CbSize < 6 ), the following applies:

              - If depth_intra_mode_set_indication_flag is equal to 1, depthIntraModeSet is set equal to 2.

              - Otherwise ( depth_intra_mode_set_indication_flag is equal to 0 ), depthIntraModeSet is set equal to 3.

hevc_intra_flag[ x0 ][ y0 ] equal to 1 specifies that intra modes with intraPredMode in the range of 0 to 34 is used for the current prediction unit. hevc_intra_flag[ x0 ][ y0 ] equal to 0 specifies that another intra mode is used for the current prediction unit. When not present, hevc_intra_flag[ x0 ][ y0 ] is inferred to be equal to 1.

sdc_flag[ x0 ][ y0 ] equal to 1 specifies that the residual of each partition of the current prediction unit is only represented by sdc_residual_flag[ x0 ][ y0 ][ i ], sdc_residual_sign_flag [ x0 ][ y0 ][ i ] and sdc_residual_abs_minus1[ x0 ][ y0 ][ i ]. sdc_flag[ x0 ][ y0 ] equal to 0 specifies that more residual may be present in the transform tree. When not present, sdc_flag[ x0 ][  y0 ] is inferred to be equal to 0.

depth_intra_mode[ x0 ][ y0 ] specifies the depth intra mode of the current prediction unit. Table Error: Reference source not found-1 Error: Reference source not foundspecifies the value of the variable depthIntraModeMaxLen depending on depthDmmRbcModeSetdepthIntraModeSet and the value of the variable DepthIntraMode and the associated name depending on the on depth_intra_mode and depthDmmRbcModeSetdepthIntraModeSet. When depth_intra_mode[ x0 ][ y0 ] is not present, DepthIntraMode is inferred to be equal to 4.

The variables depthDmmRbcModeSet is derived as specified in the following:

              - If cLog2CbSize is equal to 3 and PartMode[ xC ][ yC ] is equal to PART_NxN, depthDmmRbcModeSet is set equal to 1.

              - Otherwise (cLog2CbSize is not equal to 3 or PartMode[ xC ][ yC ] is not equal to PART_NxN ), depthDmmRbcModeSet is set equal to 0.

Table Error: Reference source not found-1 – Specification of DepthIntraMode and associated name depending on depthDmmRbcModeSet and depth_intra_mode and specification of and depthIntraModeMaxLen depdending on depthDmmRbcModeSet

|  | depthDmmRbcModeSet | 0 | 1 |
| --- | --- | --- | --- |
|  | depthIntraModeMaxLen | 3 | 2 |
| DepthIntraMode | Associated name | depth_intra_mode |  |
| 0 | INTRA_DEP_DMM_WFULL | 0 | 0 |
| 1 | INTRA_DEP_DMM_CPREDTEX | 2 | - |
| 2 | INTRA_DEP_DMM_WPREDTEX | 6 | 2 |
| 3 | INTRA_DEP_CHAIN | 7 | 3 |

The variable SdcFlag[ x0 ][ y0 ] is derived as specified in the following:

  - SdcFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_PLANAR )  | |  	(Error: Reference source not found-1)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_DMM_WFULL )

- SdcFlag[ x0 ][ y0 ] is set to be equal to sdc_flag[ x0 ][ y0 ].

The variable DmmFlag[ x0 ][ y0 ] is derived as specified in the following:

  - DmmFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_WFULL )  | |  	(Error: Reference source not found-2)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_CPREDTEX )  | |  
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_WPREDTEX )

# Simulation results

Proposed method is integrated into HTM-8.0 and compared with it under common test condition [3] and all intra case.

## Comparison of proposed method with HTM-8.0

As shown in Table 1, proposed method can achieve -0.40% average coding gain on synthesized view for the first 7 sequences, and can achieve -0.55% coding gain on synthesized view for Shark sequence. Results for all intra case are also reported in Table 2. -0.35% and -0.24% coding gain on synthesized view are achieved for the first 7 sequences and Shark sequences respectively.

Table 1: Comparison of proposed method with HTM-8.0 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.11% | 0.11% | -0.01% | -0.10% | -0.25% | 86.3% | 84.4% | 84.9% |
| Kendo | 0.00% | -0.04% | -0.11% | -0.04% | -0.44% | -0.54% | 90.6% | 91.3% | 92.2% |
| Newspaper_CC | 0.00% | 0.07% | -0.02% | 0.01% | -0.21% | -0.34% | 93.4% | 89.5% | 91.1% |
| GT_Fly | 0.00% | 0.31% | 0.04% | 0.03% | -0.17% | -0.42% | 87.8% | 90.1% | 85.7% |
| Poznan_Hall2 | 0.00% | 0.14% | -0.22% | -0.01% | -0.20% | -0.48% | 122.9% | 116.5% | 121.0% |
| Poznan_Street | 0.00% | 0.41% | -0.03% | 0.05% | -0.06% | -0.21% | 92.6% | 93.7% | 91.2% |
| Undo_Dancer | 0.00% | -0.04% | -0.23% | -0.03% | -0.15% | -0.56% | 117.9% | 111.5% | 113.2% |
| 1024x768 | 0.00% | -0.02% | -0.01% | -0.01% | -0.25% | -0.38% | 90.1% | 88.4% | 89.4% |
| 1920x1088 | 0.00% | 0.20% | -0.11% | 0.01% | -0.15% | -0.42% | 105.3% | 103.0% | 102.8% |
| average | 0.00% | 0.11% | -0.06% | 0.00% | -0.19% | -0.40% | 98.8% | 96.7% | 97.0% |
| Shark | 0.00% | -0.08% | 0.29% | 0.03% | -0.12% | -0.55% | 106.5% | 110.0% | 109.1% |

Table 2: Comparison of proposed method with HTM-8.0 (All intra case)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | -0.12% | -0.27% | 103.4% | 83.2% | 90.1% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | -0.25% | -0.35% | 110.9% | 87.5% | 94.1% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | -0.14% | -0.34% | 121.4% | 99.9% | 108.5% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | -0.09% | -0.22% | 99.7% | 81.3% | 90.6% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | -0.30% | -0.58% | 128.0% | 96.2% | 104.8% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | -0.09% | -0.20% | 112.4% | 92.4% | 99.8% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | -0.18% | -0.49% | 128.1% | 96.9% | 107.9% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | -0.17% | -0.32% | 111.9% | 90.2% | 97.6% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | -0.17% | -0.37% | 117.1% | 91.7% | 100.8% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | -0.17% | -0.35% | 114.9% | 91.0% | 99.4% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | -0.24% | 103.8% | 87.0% | 91.9% |

## Combination of proposed method and JCT3V-F0125 “CE3: Inter-view motion vector prediction for depth coding”

We also show results of combination of proposed method and another proposal “CE3: Inter-view motion vector prediction for depth coding” which also aims at improving coding efficiency of depth. In JCT3V-F0125, -0.29% and -0.16% average coding on synthesis view is achieved for the first 7 sequences and shark respectively.

The combined coding performance of these two proposals is shown in Table 3. It can be seen that coding gain of these two proposals are additive, and the combined coding gain is -0.67% and -0.7% on synthesis view for the first 7 sequences and shark respectively.

Table 3: Comparison with HTM-8.0 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.46% | -0.33% | -0.16% | -0.23% | -0.51% | 88.4% | 90.6% | 86.5% |
| Kendo | 0.00% | -0.21% | -0.19% | -0.09% | -0.50% | -0.86% | 97.0% | 96.3% | 97.3% |
| Newspaper_CC | 0.00% | -0.10% | -0.05% | -0.03% | -0.26% | -0.62% | 100.2% | 98.4% | 98.0% |
| GT_Fly | 0.00% | 0.00% | -0.17% | -0.03% | -0.14% | -0.61% | 97.0% | 102.4% | 94.5% |
| Poznan_Hall2 | 0.00% | -0.26% | -0.40% | -0.13% | -0.27% | -0.81% | 105.5% | 99.4% | 99.3% |
| Poznan_Street | 0.00% | 0.12% | 0.15% | 0.03% | -0.07% | -0.29% | 92.1% | 92.3% | 89.5% |
| Undo_Dancer | 0.00% | -0.46% | -0.32% | -0.10% | -0.19% | -0.98% | 113.9% | 111.8% | 108.7% |
| 1024x768 | 0.00% | -0.26% | -0.19% | -0.09% | -0.33% | -0.66% | 95.2% | 95.1% | 94.0% |
| 1920x1088 | 0.00% | -0.15% | -0.19% | -0.06% | -0.17% | -0.67% | 102.1% | 101.5% | 98.0% |
| average | 0.00% | -0.20% | -0.19% | -0.07% | -0.24% | -0.67% | 99.1% | 98.7% | 96.2% |
| Shark | 0.00% | -0.50% | -0.10% | -0.05% | -0.09% | -0.70% | 110.3% | 111.7% | 111.1% |

# References

- https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0/

- Gerhard Tech, Krzysztof Wegner, Ying Chen, Sehoon Yea, “3D-HEVC Draft Text 1”, JCT3V-E1001, Vienna, Austria, 29 July – 02 Aug. 2012.

- Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-E1100, Vienna, Austria, 29 July – 02 Aug. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).