| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0127 |
| --- | --- |

| Title: | CE3 related: Simplifications to sub-PU level inter-view motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen Vijayaraghavan Thirumalai 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-658-5536 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes simplifications to the sub-PU level inter-view motion prediction with two parts. The first part removes the pruning process for two spatial merge candidates when compared to the inter-view predicted merge candidate. The second part simplifies the sub-PU level motion prediction process for cases that a sub-PU does not reach available motion from the reference block. Experimental results show that, average 0.0% and 0.0% BD-rate is observed on synthesized views with the first and second proposed part, respectively.

# Introduction

In the following sub-sections, the merge list construction process and sub-PU level inter-view motion prediction method as proposed in JCT3V-E0184 [1] are briefly introduced, and two complexity aspects related to the sub-PU level inter-view motion prediction are also discussed.

## Merge candidate list construction process

The merge candidate list construction process of current 3D-HEVC is briefly described as below,

- A reference block in the reference view is identified using a disparity vector, and the motion information of the identified reference block, if available, is added to the merge candidate list. Such a candidate is named inter-view merge candidate.

- Spatial merge candidates left (A1), above (B1) and above-right (B0) are added to the merge candidate list.

- Disparity motion vector converted from the disparity vector is added to the merge candidate list.

- If BVSP is enabled, the BVSP merge candidate is added to the merge candidate list.

- Spatial merging candidates (left-bottom) A0 and (above-left) B2 are added to the merge candidate list.

- An additional inter-view merge candidate (shifted inter-view merge candidate) or shifted disparity motion vector candidate, if available, is added to the merge candidate list.

- Other merge candidates (including temporal/combined bi-predictive/zero motion vector merge candidates) are added to the merge candidate list if the number of valid candidates is less than the maximum number.

During the merge candidate list construction process, a duplicate merge candidate may be identified, which carries the same motion information of an existing merge candidate in the merge candidate list, and in this case, a pruning process may be applied. The pruning process compares a merge candidate with each merge candidate in the merge candidate list, and if identical motion is found, the merge candidate is excluded from the merge candidate list.

In current 3D-HEVC, 5 more pruning processes are applied in addition to HEVC 5 pruning processes. The additional pruning process is applied as illustrated below:

- Pruning to the inter-view merge candidate is applied for merge candidates A1 and above B1.

- Pruning to the spatial merge candidate A1 and B1 is applied for spatial merge candidate B0.

- Pruning to the inter-view merge candidate is applied to the shifted inter-view merge candidate.

## Sub-PU level inter-view motion prediction

In last meeting, a sub-PU level inter-view motion prediction method is proposed in JCT3V-E0184 [1], which achieves coding gain by doing motion prediction with smaller block size instead of the full PU size.

As proposed, the inter-view merge candidate derivation process is performed on sub-PU level. In other words, each sub-PU may have its own motion information derived from a reference block in a reference view.

## Pruning of spatial merge candidates

In 3D-HEVC [2], to avoid adding duplicate merge candidates, the spatial merge candidates derived from the left (A1) and above (B1) neighboring blocks are compared with the inter-view merge candidate before being added to the merge candidate list, i.e., the pruning process. However, when sub-PU level inter-view motion prediction is applied, the two spatial merge candidates are compared to the the inter-view merge candidate associated with bottom-right sub-PU within current PU. Meanwhile, the usage of inter-view merge candidate is increased and it is questionable whether the complexity introduced by the above pruning process is still justifying the corresponding gain.

## Motion hole-filling process

In the sub-PU level inter-view motion prediction process, one set of temporary motion parameter is maintained and updated. For each sub-PU, if the reference block has available motion information, the temporary motion parameter is updated with the motion of the reference block. Otherwise, i.e., the reference block does not have available motion information, the temporary motion parameter is copied to current sub-PU, and such a process is called the motion hole-filling process in this contribution.

According to the current design of motion hole-filling process, it is noted that there is dependency on the motion prediction process among sub-PUs within one PU, and therefore the current motion hole-filling process is not friendly to parallel processing.

# Proposal

To simplify the sub-PU level inter-view motion prediction, two parts are proposed in this contribution as listed below:

- Pruning of spatial merge candidates derived from A1 and B1 to the inter-view merge candidate is removed.

- For each sub-PU in the current PU, if the identified reference block does not contain available motion information, the followings are applied:

- If the original PU-level inter-view merge candidate is available, the corresponding motion information is inherited by the inter-view merge candidate associated with this sub-PU;

- Otherwise, zero motion (reference index is set as 0, motion vectors for both List0 and List1 are set as (0, 0)) is set for this sub-PU.

With the first proposed part, the complexity of the additional pruning process involved in the sub-PU level inter-view motion prediction process is reduced. With the second proposed part, the dependency on the motion prediction process among sub-PUs within one PU is removed, and the sub-PU level motion prediction can be implemented in a parallel fashion.

# Experimental results

The proposed method is integrated based on the sub-PU inter-view motion prediction method as proposed in [3] and simulations are performed based on the common test conditions (CTC) [4]. Two tests, namely Test #1 and Test #2, are simulated in this section, as described below,

Test #1:

- Anchor: sub-PU level inter-view motion prediction

- Tested: sub-PU level inter-view motion prediction with the proposed removal of pruning process

Test #2:

- Anchor: sub-PU level inter-view motion prediction

- Tested: sub-PU level inter-view motion prediction with the proposed motion hole-filling process

In above two tests, the sub-PU level inter-view motion prediction process is exactly the same as proposed in JCT3V-F0110 [3], and sub-PU size of 8x8 is always applied.

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

The coding performance of Test #1 is given in Table 1. As reported in Table 1, the overall average BD rate is 0.0% for the synthesized views.

Table 1: Coding results of Test #1 for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 102% | 101% | 109% |
| Kendo | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 107% | 114% | 110% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 108% | 113% | 108% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 93% | 100% | 94% |
| PoznanHall2 | 0.4% | 0.5% | 0.3% | 0.3% | 0.0% | 99% | 104% | 102% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 127% | 130% | 126% |
| UndoDancer | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 105% | 105% | 103% |
| 1024x768 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 106% | 109% | 109% |
| 1920x1088 | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 106% | 110% | 106% |
| average | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 106% | 110% | 107% |
| Shark | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 112% | 111% | 110% |

The coding performance of Test #2 is given in Table 2. As reported in Table 2, the overall average BD rate is 0.0% for the synthesized views.

Table 2: Coding results of Test #2 for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98% | 97% | 96% |
| Kendo | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 98% | 101% | 100% |
| Newspapercc | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 103% | 102% | 102% |
| GhostTownFly | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 92% | 95% | 95% |
| PoznanHall2 | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 87% | 90% | 91% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 125% | 122% | 123% |
| UndoDancer | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 119% | 116% | 113% |
| 1024x768 | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100% | 100% | 99% |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 105% | 106% | 105% |
| average | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 103% | 103% | 103% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 117% | 113% | 115% |

# Complexity analysis

The complexity changes by using proposed methods have impact on the following two aspects:

- Number of pruning

With the proposed methods, the pruning to the inter-view merge candidate is removed for merge candidates A1 and above B1, and a reduction of 2 pruning is achieved. Therefore, the total number of pruning operations in merge list construction process is reduced from 10 to 8.

- Coding delay of motion hole-filling process

The motion hole-filling process is not parallel friendly and may cause coding delay. Assume the amount of decoding time used for accessing the motion information of one reference block for each sub-PU is T. In the worst case, when only the bottom right sub-PU in a 64x64 PU has reached available motion from reference view, the delay of motion prediction is maximized as 63*T. However, with the proposed method, the worst case coding delay is largely reduced to T, which indicates the accessing to the original PU-level inter-view merge candidate.

# References

- J. An, K. Zhang, J.-L. Lin, S. Lei, “3D-CE3.h related: Sub-PU level inter-view motion prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E0184, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1001, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

- J. An, K. Zhang, J.-L. Lin, S. Lei, “3D-CE3: Sub-PU level inter-view motion prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0110, 6th Meeting: Geneva, CH, Oct. 25– Nov. 1, 2013.

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1100, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).