| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0129 |
| --- | --- |

| Title: | CE3 related: combined bi-predictive merging candidates for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current 3D-HEVC, the combined bi-predictive merging candidate derivation process is different from that in HEVC. The differences lie in two aspects: 1) When constructing a combined bi-predictive merging candidate, whether the first or the second candidate used for the combination are BVSP candidates are checked; 2) the table used to listed the ordered combinations of the indices to the two candidates is enlarged from 12 (P42) to 20 (P52). These changes result in additional logics for 3D-HEVC design while they may not contribute too much to the coding efficiency. In this proposal, it is proposed to align the bi-predictive merging candidate derivation process in 3D-HEVC the existing HEVC design. Simulation results show almost no impact on coding performance by doing this alignment.

# Introduction

In current 3D-HEVC, up to six merge candidates may be added to the merge candidate list [1]. The merge candidates could be: texture merging candidate/inter-view merging candidate, spatial/BVSP/disparity inter-view motion vector candidate/temporal merging candidate.

In addition, if the current slice is a B slice, the derivation process of combined bi-predictive merging candidates is then invoked after the temporal merging candidate derivation process. The derivation process of combined bi-predictive merging candidates is described as follows (with the differences compared to HEVC highlighted):

For each pair of candidates that are already in the candidate list and have necessary motion information, combined bi-predictive motion vector candidates (with index denoted by combIdx) are derived by a combination of the motion vector of the first candidate (with merge candidate index equal to l0CandIdx) referring to a picture in the list 0 (if available) and the motion vector of a second candidate (with merge candidate index equal to l1CandIdx) referring to a picture in the list 1 (if available and either reference picture or motion vector is different from the first candidate). When either the first candidate or the second candidate is BVSP coded, they couldn’t be used to derive a combined bi-predictive merge candidate. The definitions of l0CandIdx and l1CandIdx corresponding to combIdx are illustrated in Error: Reference source not found which is an extension of current HEVC design.

For combIdx being 0 …  19, the generation process of combined bi-predictive motion vector candidates is terminated when one the following conditions is true:

- combIdx is equal to ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) wherein numOrigMergeCand denotes the number of candidates in the merge list before invoking this process.

- Number of total candidates (including newly generated combined bi-predictive merging candidates) in the merge list is equal to MaxNumMergeCand.

Table 1– Specification of l0CandIdx and l1CandIdx in 3D-HEVC

| combIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l0CandIdx | 0 | 1 | 0 | 2 | 1 | 2 | 0 | 3 | 1 | 3 | 2 | 3 | 0 | 4 | 1 | 4 | 2 | 4 | 3 | 4 |
| l1CandIdx | 1 | 0 | 2 | 0 | 2 | 1 | 3 | 0 | 3 | 1 | 3 | 2 | 4 | 0 | 4 | 1 | 4 | 2 | 4 | 3 |

Note that in Table 1, the grey highlighted entries are newly added ones in 3D-HEVC.

- Proposal

It is proposed to the design of the derivation process of combined bi-predictive merging candidates in current 3D-HEVC is replaced by that used in HEVC. Therefore, there is no need to check the BVSP flags in the combined bi-predictive merging candidate derivation process.

Furthermore, when invoking the derivation process of combined bi-predictive merging candidates in HEVC, the combination table can be kept unchanged. In this way, the specification of l0CandIdx and l1CandIdx in HEVC could be reused without any changes.

With the proposed method, there is no need to change the modules mentioned above which reduces the complexity for hardware implementation and aligns with some of existing HEVC modules.

# Experimental results

The proposed method was implemented into 3D-HTM v8.0 [2]. Simulation results of the proposal are shown in Table 1. Simulations were conducted following common test conditions [3].

As shown in Table 1, the proposed method shows almost the same performance as current 3D-HEVC. Note the encoding and decoding time may be not accurate because of hybrid CPUs used for the simulations.

Table 2: Coding gain of proposed method with respect to 3D-HTM anchor for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 98% | 104% | 101% |
| Kendo | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 105% | 108% | 108% |
| Newspapercc | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 98% | 102% | 99% |
| GhostTownFly | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 102% | 105% | 101% |
| PoznanHall2 | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 114% | 115% | 113% |
| PoznanStreet | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 106% | 111% | 108% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 113% | 114% | 108% |
| 1024x768 | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100% | 105% | 103% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 109% | 111% | 107% |
| average | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 105% | 108% | 106% |
| Shark | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 117% | 124% | 120% |

- Proposed working draft

All the changes are highlighted in green or red strikethrough.

- H.8.5.3.2.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xPb ][ yPb ] is equal to 1, where ( xPb, yPb ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- ..

Outputs of this process are:

- ..

The location ( xOrigP, yOrigP ) and the variables nOrigPbW and nOrigPbH are derived to store the values of ( xPb, yPb ), nPbW, and nPbH as follows:

…

- 6.	The variable numOrigMergeCand is set equal to numMergeCand.

- 7.	When slice_type is equal to B and numMergeCand is less than 5, the derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found  is invoked with mergeCandList, mergeCandIsVspFlag, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand, and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk, and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added into mergeCandList. The number of candidates being added, numCombMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive, and mergeCandIsVspFlag[ numOrigMergeCand + k ] is set equal to 0.

…

- H.8.5.3.2.3	Derivation process for combined bi-predictive merging candidates

Inputs to this process are:

- a merging candidate list mergeCandList,

- a list mergeCandIsVspFlag,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0combCandk and refIdxL1combCandk of every new candidate combCandk added into mergeCandList during the invokation of this process,

- the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk of every new candidate combCandk added into mergeCandList during the invokation of this process,

- the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk added into mergeCandList during the invokation of this process.

When numOrigMergeCand is greater than 1 and less than MaxNumMergeCand, the variable numInputMergeCand is set equal to numCurrMergeCand, the variable combIdx is set equal to 0, the variable combStop is set equal to FALSE, and the following steps are repeated until combStop is equal to TRUE:

  - The variables l0CandIdx and l1CandIdx are derived using combIdx as specified in Table 8-6.

  - The following assignments are made, with l0Cand being the candidate at position l0CandIdx and l1Cand being the candidate at position l1CandIdx in the merging candidate list mergeCandList:

    - l0Cand = mergeCandList[ l0CandIdx ]

    - l1Cand = mergeCandList[ l1CandIdx ]

  - When all of the following conditions are true:

    - mergeCandIsVspFlag[ l0CandIdx ] = = 0,

    - mergeCandIsVspFlag[ l1CandIdx ] = = 0,

    - predFlagL0l0Cand  = =  1

    - predFlagL1l1Cand  = =  1

    - ( DiffPicOrderCnt( RefPicList0[ refIdxL0l0Cand ], RefPicList1[ refIdxL1l1Cand ] )  !=  0 )  | |  
( mvL0l0Cand  !=  mvL1l1Cand )

the candidate combCandk with k equal to ( numCurrMergeCand − numInputMergeCand ) is added at the end of mergeCandList, i.e. mergeCandList[ numCurrMergeCand ] is set equal to combCandk, and the reference indices, the prediction list utilization flags, and the motion vectors of combCandk are derived as follows and numCurrMergeCand is incremented by 1:

      - refIdxL0combCandk = refIdxL0l0Cand		(Error: Reference source not found-1)

      - refIdxL1combCandk = refIdxL1l1Cand		(Error: Reference source not found-2)

      - predFlagL0combCandk = 1		(Error: Reference source not found-3)

      - predFlagL1combCandk = 1		(Error: Reference source not found-4)

      - mvL0combCandk[ 0 ] = mvL0l0Cand[ 0 ]		(Error: Reference source not found-5)

      - mvL0combCandk[ 1 ] = mvL0l0Cand[ 1 ]		(Error: Reference source not found-6)

      - mvL1combCandk[ 0 ] = mvL1l1Cand[ 0 ]		(Error: Reference source not found-7)

      - mvL1combCandk[ 1 ] = mvL1l1Cand[ 1 ]		(Error: Reference source not found-8)

      - numCurrMergeCand = numCurrMergeCand + 1	(Error: Reference source not found-9)

  - The variable combIdx is incremented by 1.

  - When combIdx is equal to ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) or numCurrMergeCand is equal to MaxNumMergeCand, combStop is set equal to TRUE.

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 1,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1001, 5th Meeting: Vienna, AT, 27 July-2 Aug. 2013.

- 3DV-HTM version 8.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1100, 5th Meeting: Vienna, AT, 27 July-2 Aug. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).