| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0130 |
| --- | --- |

| Title: | CE1 related: BVSP for asymmetric motion partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current implementation of Backward View Synthesis Prediction (BVSP) in 3D-HEVC, it may have two problems for the asymmetric motion partitioning case. Firstly, when either the width or height of current prediction unit (PU) is equal to 12, 4x4 based motion compensation may be invoked. Secondly, when current PU size is equal to 4x16 or 16x4, the motion compensation size is set equal to the PU size which is different for all the other cases. In this proposal, it is proposed to use 4x8 (or 8x4) based motion compensation for asymmetric PUs with size smaller than 16x16. The contribution reports minor coding differences, 0.02% coding gain for synthesized views.

# Introduction

In the previous JCT-3V meeting, adaptive block partitioning for BVSP was adopted [1]. For the BVSP coded prediction units (PUs), it is firstly split into several 8x8 blocks. For each 8x8 block, either 4x8 or 8x4 partition for motion compensation is selected by analyzing depth values at 4 corners of one corresponding 8x8 depth block. The block partition size is decided by the following rule:

if vdepthTL<vdepthBR?0:1 !=vdepthTR<vdepthBL?0:1 use 4x8partition ( W=4, H=8 )else use 8x4partition ( W=8, H=4 )

Figure 1: Four corner pixels of one 8x8 depth block

, where TL, TR, BL, and BR represent top-left, top-right, bottom-left, and bottom-right corner pixels, respectively, and ^ represents the exclusive OR (XOR) operator. For each partition (either 4x8 or 8x4), one representative depth value is selected and stored in 4x4 unit. The stored representative depth values are then used to perform the motion compensation in 4x4 unit in current software implementation.

When asymmetric motion partitioning is enabled in the latest 3D-HTM reference software, the current implementation of the adaptive block partitioning method may have the following problems:

- When the current PU size is 12x16 or 16x12, the motion compensation size may be equal to 4x4. Here we take a 12x16 PU for example. If a 12x16 PU is coded in BVSP mode, it is split to two 8x8 blocks and two remaining 4x8 blocks. For each of the block, one 8x8 depth block is analyzed to determine the motion compensation size equal to 4x8 or 8x4. In this way, for a 4x8 block, the determined partition size may be equal to 8x4. Therefore, two different representative depth values may be selected for one 4x8 block which result in two different disparity motion vectors. In other words, such a 4x8 block will perform the motion compensation based on 4x4 units which is not allowed by HEVC.

- When the current PU size is 4x16 or 16x4 and the PU is uni-directional predicted, BVSP is achieved by deriving one disparity vector for the whole PU and used it for motion compensation. Using larger block size may make the BVSP less efficient.

- Proposal

Two aspects are proposed in this contribution:

- When the current PU size is 16x12 (or 12x16) and the PU is uni-directional predicted, it is still firstly split to two 8x8 blocks and two remaining 4x8 or 8x4 blocks. However, for the remaining 4x8 or 8x4 blocks, the motion compensation size is directly set equal to the block size. Therefore, 4x4 based motion compensation could be avoided, as proposed in JCT3V-E0209 [1].

- When the current PU size is 16x4 (or 4x16) and the PU is uni-directional predicted, the motion compensation size is set to 8x4 (or 4x8). In this way, the motion compensation size for BVSP coded PUs could be aligned and potential coding gain may be expected.

# Experimental results

The proposed method has been implemented into 3D-HTM v8.0 [3]. Simulations were conducted following common test conditions [4].

Three sets of simulations are provided, including separate results for proposed two aspects and results of the whole proposal.

Table 1 shows the performance of the first proposed method which is applied to 16x12 or 12x16 PUs. As shown in Table 1, the proposed method has almost no impact on the overall performance.

Table 1: Coding gain of 1st aspect with respect to 3D-HTM anchor for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.09% | -0.09% | 0.00% | -0.01% | 0.05% | 96% | 104% | 101% |
| Kendo | -0.03% | 0.02% | 0.00% | 0.00% | 0.01% | 96% | 107% | 104% |
| Newspapercc | 0.10% | 0.09% | 0.04% | 0.04% | 0.05% | 108% | 114% | 112% |
| GhostTownFly | 0.09% | -0.07% | 0.00% | 0.00% | 0.05% | 105% | 105% | 104% |
| PoznanHall2 | 0.06% | -0.03% | 0.01% | 0.01% | -0.03% | 112% | 115% | 112% |
| PoznanStreet | 0.10% | 0.05% | 0.02% | 0.01% | -0.02% | 101% | 106% | 103% |
| UndoDancer | 0.08% | -0.06% | 0.01% | 0.01% | -0.02% | 116% | 113% | 111% |
| 1024x768 | 0.05% | 0.01% | 0.01% | 0.01% | 0.04% | 100% | 108% | 106% |
| 1920x1088 | 0.08% | -0.03% | 0.01% | 0.01% | -0.01% | 109% | 110% | 107% |
| average | 0.07% | -0.01% | 0.01% | 0.01% | 0.01% | 105% | 109% | 107% |
| Shark | 0.07% | 0.06% | 0.01% | 0.01% | 0.00% | 99% | 106% | 103% |

shows the performance of the second proposed method which is applied to 16x4 or 4x16 PUs. As shown in  shows the performance of the second proposed method which is applied to 16x4 or 4x16 PUs. As shown in  shows the performance of the second proposed method which is applied to 16x4 or 4x16 PUs. As shown in , minor coding gain, i.e., 0.03% bitrate reduction for synthesized views could be achieved.  , minor coding gain, i.e., 0.03% bitrate reduction for synthesized views could be achieved.  , minor coding gain, i.e., 0.03% bitrate reduction for synthesized views could be achieved.   shows the performance of the second proposed method which is applied to 16x4 or 4x16 PUs. As shown in  shows the performance of the second proposed method which is applied to 16x4 or 4x16 PUs. As shown in  shows the performance of the second proposed method which is applied to 16x4 or 4x16 PUs. As shown in  shows the performance of the second proposed method which is applied to 16x4 or 4x16 PUs. As shown in , minor coding gain, i.e., 0.03% bitrate reduction for synthesized views could be achieved.  , minor coding gain, i.e., 0.03% bitrate reduction for synthesized views could be achieved.  , minor coding gain, i.e., 0.03% bitrate reduction for synthesized views could be achieved.  , minor coding gain, i.e., 0.03% bitrate reduction for synthesized views could be achieved.

Table 2: Coding gain of 2nd aspect with respect to 3D-HTM anchor for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.07% | -0.19% | -0.05% | -0.05% | -0.02% | 88% | 93% | 91% |
| Kendo | 0.10% | -0.02% | 0.02% | 0.01% | -0.04% | 100% | 104% | 106% |
| Newspapercc | -0.02% | 0.03% | 0.00% | 0.00% | -0.03% | 102% | 104% | 102% |
| GhostTownFly | -0.02% | -0.06% | -0.01% | -0.02% | -0.04% | 91% | 104% | 95% |
| PoznanHall2 | 0.03% | -0.09% | -0.02% | 0.00% | -0.09% | 105% | 106% | 107% |
| PoznanStreet | 0.14% | -0.03% | 0.02% | 0.01% | -0.02% | 96% | 103% | 101% |
| UndoDancer | 0.09% | -0.13% | 0.00% | 0.00% | -0.01% | 109% | 110% | 108% |
| 1024x768 | 0.01% | -0.06% | -0.01% | -0.01% | -0.03% | 97% | 100% | 99% |
| 1920x1088 | 0.06% | -0.08% | 0.00% | 0.00% | -0.04% | 100% | 106% | 103% |
| average | 0.04% | -0.07% | -0.01% | -0.01% | -0.03% | 99% | 103% | 101% |
| Shark | -0.06% | 0.10% | 0.00% | 0.00% | 0.01% | 100% | 107% | 104% |

shows the performance of the whole proposal. As shown in   shows the performance of the whole proposal. As shown in   shows the performance of the whole proposal. As shown in , minor coding gain, i.e., 0.02% bitrate reduction for synthesized views could be achieved.  , minor coding gain, i.e., 0.02% bitrate reduction for synthesized views could be achieved.  , minor coding gain, i.e., 0.02% bitrate reduction for synthesized views could be achieved.    shows the performance of the whole proposal. As shown in   shows the performance of the whole proposal. As shown in   shows the performance of the whole proposal. As shown in   shows the performance of the whole proposal. As shown in , minor coding gain, i.e., 0.02% bitrate reduction for synthesized views could be achieved.  , minor coding gain, i.e., 0.02% bitrate reduction for synthesized views could be achieved.  , minor coding gain, i.e., 0.02% bitrate reduction for synthesized views could be achieved.  , minor coding gain, i.e., 0.02% bitrate reduction for synthesized views could be achieved.

Table 3: Coding gain of the proposal with respect to 3D-HTM anchor for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.01% | -0.05% | -0.01% | 0.01% | 0.03% | 96% | 100% | 99% |
| Kendo | 0.02% | -0.03% | 0.00% | 0.00% | -0.04% | 94% | 99% | 98% |
| Newspapercc | 0.05% | 0.06% | 0.01% | 0.01% | 0.02% | 100% | 103% | 101% |
| GhostTownFly | 0.11% | -0.16% | -0.02% | -0.02% | -0.02% | 100% | 102% | 100% |
| PoznanHall2 | 0.06% | -0.18% | -0.03% | -0.02% | -0.10% | 115% | 116% | 116% |
| PoznanStreet | 0.04% | -0.03% | 0.00% | -0.01% | -0.02% | 97% | 100% | 100% |
| UndoDancer | 0.12% | -0.17% | 0.00% | 0.00% | 0.00% | 106% | 109% | 104% |
| 1024x768 | 0.03% | -0.01% | 0.00% | 0.01% | 0.01% | 97% | 101% | 99% |
| 1920x1088 | 0.08% | -0.13% | -0.01% | -0.01% | -0.03% | 104% | 107% | 105% |
| average | 0.06% | -0.08% | -0.01% | 0.00% | -0.02% | 101% | 104% | 103% |
| Shark | -0.03% | 0.11% | 0.00% | 0.00% | 0.03% | 103% | 108% | 105% |

Note the encoding and decoding time may be not accurate because of hybrid CPUs used for the simulations.

# References

- S. Shimizu, S. Sugimoto, “3D-CE1.h: Adaptive block partitioning for VSP,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E0207, 5th Meeting: Vienna, AT, 27 July-2 Aug. 2013.

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 1,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1001, 5th Meeting: Vienna, AT, 27 July-2 Aug. 2013.

- 3DV-HTM version 8.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1100, 5th Meeting: Vienna, AT, 27 July-2 Aug. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).