| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0130 _proposed text |
| --- | --- |

| Title: | Proposed text for JCT3V-F0130 based on 3D-HEVC Draft Text 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated, Peking University |  |  |

_____________________________

# Abstract

All the changes are highlighted in green or red strikethrough.

# Decoding process

- H.8.5.3.3.8	View synthesis prediction process

Inputs to this process are:

- …

Outputs of this process are:

–	…

The location ( xP, yP ) given in full-sample units of the upper-left luma samples of the current prediction block relative to the upper-left luma sample location of the current picture is derived by:

  - xP = xCb + xBl		(Error: Reference source not found-1)

  - yP = yCb + yBl		(Error: Reference source not found-2)

The reference picture consisting of an ordered two-dimensional array refPicL of luma samples and two ordered two-dimensional arrays refPicCb and refPicCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX as input.

The variable refViewIdx is set equal to the ViewIdx( RefPicListX[ refIdxLX ] ) and the variable depthViewIdx is set equal to RefViewIdx[ xCb + xBl ][ yCb + yBl ]. The variable mvDisp is set equal to MvDisp[ xCb + xBl ][ yCb + yBl ]

The derivation process for a disparity sample array as specified in section  is invoked with the luma location ( xP, yP ), the disparity vector mvDisp, the variable refViewIdx, the variable depthViewIdx, the variable nPSW equal to ( nPbW  = =  12 ? 16 : nPbW ), the variable nPSH equal to ( nPbH  = =  12 ? 16 : nPbH ), and the variable splitFlag equal to 1( nPbW > 4  &&  nPbH > 4 ) as the inputs, and the output is the array disparitySamples of size (nPSW)x(nPSH).

Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in quarter-sample units.

For each luma sample location ( xL = 0..nPbW − 1, yL = 0..nPbH − 1 ) inside the prediction luma sample array predSamplesL, the corresponding prediction luma sample value predSamplesL[ xL ][ yL ] is derived as follows:

- …

- Derivation process for a disparity sample array

- Inputs to this process are:

- a luma location ( xP, yP ) relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view order index refViewIdx specifying a reference view,

- a view order index depthViewIdx specifying the view the depth should be derived from

- variables nPSW and nPSH specifying a width and a height, respectively

- a variable splitFlag.

Outputs of this process are:

- a (nPSW)x(nPSH) array disparitySamples of disparities values.

Let refDepPic the picture in the current access unit with ViewIdx( refDepPic ) equal to ViewIdx and DepthFlag( refDepPic ) equal to 1.

Let refDepPels be an array of reconstructed depth samples refDepPic. The luma location (xTL, yTL) of top-left luma sample of a block in refDepPels is derived by

  - xTL = xP + ( ( mvDisp[ 0 ] + 2 )  >>  2 )		(Error: Reference source not found-3)

  - yTL = yP + ( ( mvDisp[ 1 ] + 2 )  >>  2 )		(Error: Reference source not found-4)

- The variables nSubBlkW and nSubBlkH are derived as specified in the following:

  - nSubBlkW = splitFlag ? 8 : nPSW		(Error: Reference source not found-5)

  - nSubBlkH = splitFlag ? 8 : nPSH		(Error: Reference source not found-6)

The vairables bhorAsy is set to ( nPSH % nSubBlkH)==0? 1: 0 ) and bVerAsy is set to ( nPSW % nSubBlkW)==0? 1: 0 ).

The array disparitySamples of size (nPSW)x(nPSH) is derived as specified in the following:

- For sBy in the range of 0 to ( ( nPSH / nSubBlkH) – bVerAsy1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nPSW / nSubBlkW) – bhorAsy1 ), inclusive, the following applies:

    - The variables xB, yB, xP0, yP0, xP1, yP1, are derived as specified in the following:

xB = sBx * nSubBlkW
			yB = sBy * nSubBlkH
			xP0 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xB )
			yP0 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + yB )
			xP1 = Clip3( 0, pic_width_in_luma_samples – 1, xTL + xB + nSubBlkW – 1 )
			yP1 = Clip3( 0, pic_height_in_luma_samples – 1, yTL + yB + nSubBlkH – 1 )

    - The variable nSubSubBlkW is set equal to nSubBlkW and the variable nSubSubBlkH is set equal to nSubBlkH.

    - When splitFlag is equal to 1, nSubSubBlkW and nSubSubBlkW are modified as follows:

      - The variable horSplitFlag is derived as specified in the following:

- If both (nPSW – xB) is equal to nSubBlkW and (nPSH – yB) is equal to nSubBlkH)), the following apply:

        - horSplitFlag = ( refDepPels[ xP0 ][ yP0 ] > refDepPels[ xP1 ][ yP1 ] )
	  = =  ( refDepPels[ xP1 ][ yP0 ] > refDepPels[ xP0 ][ yP1] ) )	(Error: Reference source not found-7)

        - nSubSubBlkW = horSplitFlag ? nSubSubBlkW : ( nSubSubBlkW  >>  1 )		(Error: Reference source not found-8)

        - nSubSubBlkH = horSplitFlag ? ( nSubSubBlkH  >>  1 ) : nSubSubBlkW		(Error: Reference source not found-9)

- Otherwise, horSplitFlag is equa to 0, nSubSubBlkW is set to (nPSW – xB) and nSubSubBlkH is set to (nPSH – yB).

    - The derivation process for a disparity sample block as specified in subclause Error: Reference source not found is invoked with the luma location ( xB, yB ), variables nSubBlkW and nSubBlkH, the array of reconstructed depth samples refDepPels, the luma location ( xTL, yTL ), the variables nSubSubBlkW and nSubSubBlkW, the view order index refViewIdx, and the array disparitySamples as the inputs, and the output is the modified array disparitySamples.