| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0132 |
| --- | --- |

| Title: | CE5 related: Unification of delta DC coding for depth intra modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Ying Chen Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-658-5536 xinzhao@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current 3D-HEVC, several enhanced depth intra modes, including Depth Modeling Modes (DMM), Region Boundary Chain coding (RBC) mode and Simplified Depth Coding (SDC) modes, are utilized for depth coding. The enhanced depth intra modes partition a depth PU into one or two segments, and each segment is coded together with an optional delta DC value (or residual). To simplify the delta DC coding, in this contribution, a unification scheme is proposed to code the delta DC values. It is reported that -0.1% BD-rate gain is achieved using proposed method under both common test conditions and Intra-only test conditions for synthesized views.

# Introduction

In current 3D-HEVC [1], partition-based methods including DMM, RBC and SDC are applied for depth map intra coding. With partition-based depth map intra coding methods, a depth block is partitioned by one or two segments, and each segment is represented by a single DC value. The DC value for each segment can be further compensated by a delta DC value which is explicitly signaled in the bitstream.

## Syntax and semantics design for delta DC

To show the differences of the syntax design regarding to delta DC, the syntax for different depth modes are highlighted with different color as shown below:

- H.7.3.8.5.1	Depth mode parameter syntax

| depth_mode_parameters( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( DmmFlag[ x0 ][ y0 ] ) { |  |
| dmm_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( dmm_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < 2; i ++ ) { |  |
| dmm_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( dmm_dc_abs[ x0 ][ y0 ][ i ]) |  |
| dmm_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_dc_flag[ x0 ][ y0 ] | ae(v) |
| if( edge_dc_flag[ x0 ][ y0 ] ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| edge_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if( edge_dc_abs[ x0 ][ y0 ][ i ] != 0 ) |  |
| edge_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| else if( SdcFlag[ x0 ][ y0 ] ) { |  |
| sdcNumSegments = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ) ? 2 : 1 |  |
| for( i = 0; i < sdcNumSegments; i++ ) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i ] | ae(v) |
| if( sdc_residual_flag[ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | - ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | - ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

- H.7.4.9.5.1	Depth mode parameter semantics

…

dmm_dc_flag[ x0 ][ y0 ] equal to 1 specifies that dmm_dc_abs[ x0 ][ y0 ][ i ] and dmm_dc_sign_flag[ x0 ][ y0 ][ i ] are present, dmm_dc_flag[ x0 ][ y0 ] equal to 0 specifies that  dmm_dc_abs[ x0 ][ y0 ][ i ] and  dmm_dc_sign_flag[ x0 ][ y0 ][ i ] are not present.

dmm_dc_abs[ x0][ y0 ][ i ], dmm_dc_sign_flag[ x0 ][ y0 ][ i ] are used to derive DcOffset[ x0 ][ y0 ][ i ] as follows:

  - DcOffset[ x0 ][ y0 ][ i ]  = 
( 1 − 2 *dmm_dc_sign_flag[ x0 ][ y0 ][ i ] ) * dmm_dc_abs[ x0 ][ y0 ][ i ]	(Error: Reference source not found-1)

edge_dc_flag[ x0 ][ y0 ] equal to 1 specifies that edge_dc_abs[ x0 ][ y0 ][ i ] and edge_dc_sign_flag[ x0 ][ y0 ][ i ] are present, edge_dc_flag[ x0 ][ y0 ] equal to 0 specifies that  edge_dc_abs[ x0 ][ y0 ][ i ] and  edge_dc_sign_flag[ x0 ][ y0 ][ i ] are not present.

edge_dc_abs[ x0][ y0 ][ i ], edge_dc_sign_flag[ x0 ][ y0 ][ i ] are used to derive DcOffset[ x0 ][ y0 ][ i ] as follows:

  - DcOffset[ x0 ][ y0 ][ i ]  = 
( 1 − 2 *edge_dc_sign_flag[ x0 ][ y0 ][ i ] ) * edge_dc_abs[ x0 ][ y0 ][ i ]	(Error: Reference source not found-2)

sdc_residual_flag[ x0 ][ y0 ][ i ] equal to 0 specifies that the residual is zero for segment i. sdc_residual_flag equal to 1 specifies that the residual is non-zero and the sdc_residual_sign_flag and sdc_residual_abs[ i ] syntax elements are present for segment i.

sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] and sdc_residual_sign_flag[ x0 ][ y0 ][ i ] are used to derive SdcResidual[ x0 ][ y0 ][ i ] for segment i as follows:

  - SdcResidual[ x0 ][ y0 ][ i ] = 
( 1 − 2 * sdc_residual_sign_flag[ x0 ][ y0 ][ i ] ) * ( sdc_residual_mag_minus1[ x0 ][ y0 ][ i ] + 1) 	(Error: Reference source not found-3)

# Proposal

In this contribution, we propose unified syntax design as well as parsing and decoding of delta DC coding for different depth coding modes.

The syntax and semantics simplifications are shown below:

| depth_mode_parameters( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| if( ( log2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( log2CbSize > 3 && log2CbSize < 6 ) ) |  |
| depth_intra_mode_set_indication_flag | ae(v) |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ¦ ¦ DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WPREDTEX ) |  |
| wedge_predtex_tab_idx[ x0 ][ y0 ] | ae(v) |
| } else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_start_left_flag[ x0 ][ y0 ] | ae(v) |
| edge_start_position[ x0 ][ y0 ] | ae(v) |
| num_edge_codes_minus1[ x0 ][ y0 ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| } |  |
| if( DmmFlag[ x0 ][ y0 ] ¦ ¦ DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ¦ ¦ SdcFlag[ x0 ][ y0 ] ) { |  |
| dcNumSeg = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_PLANAR ) ? 1 : 2 |  |
| dmmdepth_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( dmmdepth_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < 2dcNumSeg; i ++ ) { |  |
| dmmdepth_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( dmm_dc_abs[ x0 ][ y0 ][ i ]) |  |
| dmmdepth_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_dc_flag[ x0 ][ y0 ] | ae(v) |
| if( edge_dc_flag[ x0 ][ y0 ] ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| edge_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if( edge_dc_abs[ x0 ][ y0 ][ i ] != 0 ) |  |
| edge_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| else if( SdcFlag[ x0 ][ y0 ] ) { |  |
| sdcNumSegments = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ) ? 2 : 1 |  |
| for( i = 0; i < sdcNumSegments; i++ ) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i ] | ae(v) |
| if( sdc_residual_flag[ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | - ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | - ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

depth_dc_flag[ x0 ][ y0 ] equal to 1 specifies that depth_dc_abs[ x0][ y0 ][ i ] and  depth_dc_sign_flag[ x0 ][ y0 ][ i ] are present, depth_dc_flag[ x0 ][ y0 ] equal to 0 specifies that  depth_dc_abs[ x0 ][ y0 ][ i ] and  depth_dc_sign_flag[ x0 ][ y0 ][ i ] are not present and derived as 0.

depth_dc_abs[ x0][ y0 ][ i ] is used to derive depth_dc_abs[ x0][ y0 ][ i ] as follows:

  - depth_dc_abs[ x0 ][ y0 ][ i ] + = ( dcNumSeg = = 1 ? 1 : 0 )

depth_dc_sign_flag[ x0 ][ y0 ][ i ] are used to derive DcOffset[ x0 ][ y0 ][ i ] as follows:

  - DcOffset[ x0 ][ y0 ][ i ] = 
( 1 − 2 *depth_dc_sign_flag[ x0 ][ y0 ][ i ] ) * depth_dc_abs[ x0 ][ y0 ][ i ]	(Error: Reference source not found-4)

dmm_dc_flag[ x0 ][ y0 ] equal to 1 specifies that dmm_dc_abs[ x0 ][ y0 ][ i ] and dmm_dc_sign_flag[ x0 ][ y0 ][ i ] are present, dmm_dc_flag[ x0 ][ y0 ] equal to 0 specifies that  dmm_dc_abs[ x0 ][ y0 ][ i ] and  dmm_dc_sign_flag[ x0 ][ y0 ][ i ] are not present.

dmm_dc_abs[ x0][ y0 ][ i ], dmm_dc_sign_flag[ x0 ][ y0 ][ i ] are used to derive DcOffset[ x0 ][ y0 ][ i ] as follows:

  - DcOffset[ x0 ][ y0 ][ i ]  = 
( 1 − 2 *dmm_dc_sign_flag[ x0 ][ y0 ][ i ] ) * dmm_dc_abs[ x0 ][ y0 ][ i ]	(Error: Reference source not found-5)

edge_dc_flag[ x0 ][ y0 ] equal to 1 specifies that edge_dc_abs[ x0 ][ y0 ][ i ] and edge_dc_sign_flag[ x0 ][ y0 ][ i ] are present, edge_dc_flag[ x0 ][ y0 ] equal to 0 specifies that  edge_dc_abs[ x0 ][ y0 ][ i ] and  edge_dc_sign_flag[ x0 ][ y0 ][ i ] are not present.

edge_dc_abs[ x0][ y0 ][ i ], edge_dc_sign_flag[ x0 ][ y0 ][ i ] are used to derive DcOffset[ x0 ][ y0 ][ i ] as follows:

  - DcOffset[ x0 ][ y0 ][ i ]  = 
( 1 − 2 *edge_dc_sign_flag[ x0 ][ y0 ][ i ] ) * edge_dc_abs[ x0 ][ y0 ][ i ]	(Error: Reference source not found-6)

sdc_residual_flag[ x0 ][ y0 ][ i ] equal to 0 specifies that the residual is zero for segment i. sdc_residual_flag equal to 1 specifies that the residual is non-zero and the sdc_residual_sign_flag and sdc_residual_abs[ i ] syntax elements are present for segment i.

sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] and sdc_residual_sign_flag[ x0 ][ y0 ][ i ] are used to derive SdcResidual[ x0 ][ y0 ][ i ] for segment i as follows:

  - SdcResidual[ x0 ][ y0 ][ i ] = 
( 1 − 2 * sdc_residual_sign_flag[ x0 ][ y0 ][ i ] ) * ( sdc_residual_mag_minus1[ x0 ][ y0 ][ i ] + 1) 	(Error: Reference source not found-7)

- More detail on specification changes, including the deletions in the decoding processes can be found in the attached working draft text for this proposal.

# Experimental results

The proposed method is integrated into current 3D-HTM 8.0 software [2] and the simulation tests are performed under the common test conditions (CTC) [3] and Intra-only test condition as specified in Core Experiment 5 document [4].

In current HTM-8.0, the filtering of neighboring reference sample is always applied for chain coding, but never applied for DMM and SDC DMM1. To further unify the depth intra modes on the filtering of neighboring reference samples, two tests, depending on whether reference sample filtering is applied for DMM, SDC DMM1 and chain coding, are performed in this section.

In Test 1, the reference sample filtering is always applied on and SDC DMM1, DMM and chain coding in the proposed scheme. The coding performance of the proposed scheme under CTC and Intra-only test conditions are tabulated in Table 1 and Table 2, respectively. As reported in Table 1 and Table 2, when the reference sample filtering is always applied on and SDC DMM1, DMM and chain coding, average bitrate reduction of -0.1% on the synthesized views is achieved for both CTC and Intra-only test conditions.

Table 1: Coding performance of proposed scheme in Test 1 under CTC

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | 0.1% | 0.0% | 0.0% | -0.1% | 121% | 119% | 117% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% | 95% | 96% | 94% |
| Newspapercc | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 104% | 103% | 103% |
| GhostTownFly | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% | 87% | 88% | 84% |
| PoznanHall2 | 0.1% | -0.1% | 0.0% | 0.0% | -0.2% | 113% | 110% | 107% |
| PoznanStreet | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 91% | 91% | 89% |
| UndoDancer | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 121% | 111% | 112% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 107% | 106% | 105% |
| 1920x1088 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 103% | 100% | 98% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 105% | 102% | 101% |
| Shark | -0.2% | 0.1% | 0.0% | 0.1% | -0.1% | 97% | 89% | 90% |

Table 2: Coding performance of proposed scheme in Test 1 under Intra-only test condition

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 89% | 86% | 87% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 94% | 90% | 89% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 92% | 88% | 88% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100% | 99% | 101% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 105% | 99% | 101% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 103% | 101% | 105% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 104% | 104% | 102% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 92% | 88% | 88% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 103% | 101% | 102% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 98% | 95% | 96% |
| Shark | 0.0% | 0.0% | 0.0% | 0.1% | -0.2% | 92% | 90% | 91% |

In Test 2, the reference sample filtering is never applied on SDC DMM1, DMM and chain coding in the proposed scheme. The coding performance of proposed scheme under CTC and Intra-only test conditions are tabulated in Table 3 and Table 4, respectively. As reported in Table 3 and Table 4, when the reference sample filtering is never applied on SDC DMM1, DMM and chain coding, average bitrate reduction of -0.1% on the synthesized views is achieved for both CTC and Intra-only test conditions.

Table 3: Coding performance of proposed scheme in Test 2 under CTC

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | 0.0% | 0.0% | 0.0% | -0.1% | 102% | 107% | 105% |
| Kendo | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 86% | 91% | 89% |
| Newspapercc | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 92% | 89% | 89% |
| GhostTownFly | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 97% | 95% | 97% |
| PoznanHall2 | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | 111% | 106% | 106% |
| PoznanStreet | 0.2% | -0.2% | 0.0% | 0.0% | -0.1% | 100% | 99% | 99% |
| UndoDancer | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% | 110% | 107% | 107% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 93% | 96% | 94% |
| 1920x1088 | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 105% | 102% | 102% |
| average | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 100% | 99% | 99% |
| Shark | -0.2% | 0.0% | 0.0% | 0.1% | -0.2% | 96% | 95% | 97% |

Table 4: Coding performance of proposed scheme in Test 2 under Intra-only test condition

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 100% | 101% | 102% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 105% | 107% | 104% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 110% | 108% | 109% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 92% | 97% | 97% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 105% | 105% | 103% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101% | 103% | 104% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 108% | 110% | 107% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 105% | 105% | 105% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 101% | 104% | 103% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 103% | 105% | 104% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 89% | 93% | 93% |

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

# Complexity comparisons

To demonstrate the complexity reduction achieved by the proposed unification on delta DC coding, several related aspects are discussed for complexity comparisons, including syntax design, context numbers, binarization and decoding process, as listed in below Table 3.

Table 5: Complexity comparisons between current 3D-HEVC and proposed scheme

|  | Anchor | Proposed |
| --- | --- | --- |
| Syntax design | dmm_dc_flag dmm_dc_abs dmm_dc_sign_flag edge_dc_flag edge_dc_abs edge_dc_sign_flag sdc_residual_flag sdc_residual_sign_flag sdc_residual_abs_minus1 | depth_dc_flag depth_dc_abs depth_dc_sign_flag |
| Num of contexts | 6 | 3 |
| Binarization | - Exp-Golomb binarization - Specific binarization for sdc_residual_abs_minus1 | Exp-Golomb binarization |
| Decoding process | - DLT applied for DMM and SDC - Specific quantization for RBC | DLT applied for DMM, SDC and RBC |

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1001, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

- 3DV-HTM version 8.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1100, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

- F. Jäger, “Description of Core Experiment 5 (CE5) on Depth Intra Modes,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1105, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).