| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0134 _proposed_text |
| --- | --- |

| Title: | Proposed text for JCT3V-F0134 based on 3D-HEVC Draft Text 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-658-5536 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

All the newly added texts are highlighted in green and deleted texts are indicated by red strikethrough.

- H.7.3.8.5.1	Depth mode parameter syntax

| depth_mode_parameters( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| if( ( log2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ¦ ¦ ( log2CbSize > 3 && log2CbSize < 6 ) ) |  |
| depth_intra_mode_set_indication_flag | ae(v) |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ¦ ¦ DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ){ |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| wedge_sub_col[ x0 ][ y0 ] | u(v) |
| wedge_sub_row[ x0 ][ y0 ] | u(v) |
| } |  |
| else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WPREDTEX ){ |  |
| wedge_predtex_tab_idx[ x0 ][ y0 ] | ae(v)ae(4) |
| wedge_sub_col[ x0 ][ y0 ] | u(v) |
| wedge_sub_row[ x0 ][ y0 ] | u(v) |
| } else if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_start_left_flag[ x0 ][ y0 ] | ae(v) |
| edge_start_position[ x0 ][ y0 ] | ae(v) |
| num_edge_codes_minus1[ x0 ][ y0 ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| } |  |
| … |  |
| } |  |

- H.7.4.9.5.1	Depth mode parameter semantics

…

wedge_full_tab_idx[ x0 ][ y0 ] specifies the index of the wedgelet pattern in the corresponding 4x4pattern list when DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL.

wedge_sub_col[ x0 ][ y0 ] and wedge_sub_row[ x0 ][ y0 ] specify the position of the 4x4 block for which the wedgelet pattern is signalled.

wedge_predtex_tab_idx[ x0 ][ y0 ] specifies the index of the wedgelet pattern of the 4x4 block within the current prediction unit when DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WPREDTEX .

- …

- H.8.4.4.2.7	Specification of intra prediction mode INTRA_DMM_WFULL

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size,

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The Wedgelet pattern extension process as specified in subclause H.8.4.4.2.12.3 is invoked with Log2( nTbS ), wedge_full_tab_idx[ xTb ][ yTb ], wedge_sub_col[ xTb ][ yTb ] and wedge_sub_row[ xTb ][ yTb ] as inputs, and the output is assigned to wedgePattern.The variable wedgePattern[ x ][ y ] with x, y =0..nTbS − 1, specifying a binary partition pattern is derived as.

      - wedgePattern = WedgePatternTable[ Log2( nTbS ) ][ wedge_full_tab_idx[ xTb ][ yTb ] ]	(Error: Reference source not found-1)

  - The depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ xTb ][ yTb ], the transform size nTbS, the dcOffsetAvailFlag set equal to dmm_dc_flag[ xTb ][ yTb ], intraChainFlag set equal to 0, and the DC Offsets DcOffset[ xTb ][ yTb ][ 0 ], and DcOffset[ xTb ][ yTb ][ 1 ] as inputs and the output is assigned to predSamples[ x ][ y ].

  - For x, y = 0..nTbs − 1, inclusive the following applies:

    - WedgeIdx[ xTb + x ][ yTb + y ] is set equal to wedge_full_tab_idx[ xTb ][ yTb ].

- H.8.4.4.2.8	Specification of intra prediction mode INTRA_DMM_WPREDTEX

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size,

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are as follows:.

  - Depending on textureIntraPredMode, the variable wedgeIdx  is derived as specified in the following:

      - candWedgeIndList = TextModePredWedgeIndTable[ Log2( nTbS )2 ][ TextureIntraPredMode[ xTb ][ yTb ] ]	(Error: Reference source not found-2)

      - wedgeIdx = candWedgeIndList[ wedge_predtex_tab_idx[ xTb ][ yTb ] ]	(Error: Reference source not found-3)

  - The Wedgelet pattern extension process as specified in subclause H.8.4.4.2.12.3 is invoked with Log2( nTbS ), wedgeIdx, wedge_sub_col[ xTb ][ yTb ] and wedge_sub_row[ xTb ][ yTb ] as inputs, and the output is assigned to wedgePattern. The variable wedgePattern [ x ][ y ] with x, y =0..nTbS − 1, specifying a binary partition pattern is derived as.

      - wedgePattern = WedgePatternTable[ Log2( nTbS ) ][ wedgeIdx ]	(Error: Reference source not found-4)

  - The depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to dmm_dc_flag[ xTb ][ yTb ], intraChainFlag set equal to 0, and the DC Offsets DcOffset[ xTb ][ yTb ][ 0 ], and DcOffset[ xTb ][ yTb ][ 1 ] as inputs and the output is assigned to predSamples[ x ][ y ].

- H.8.4.4.2.12.1	Wedgelet pattern generation process

…

  - The binary partition pattern wedgePattern[ x ][ y ], with x, y = 0..patternSize − 1, is derived as specified in the following.

    - If resShift is equal to 1, the following applies.

      - Depending on wedgeOri, the variables xOff and yOff are set as specified in Error: Reference source not found.

Table Error: Reference source not found−1 Specification of xOff, yOff

| wedgeOri | ( xS + xE ) <curSize | xOff | yOff |
| --- | --- | --- | --- |
| 0 |  | 0 | 0 |
| 1 |  | 1 | 0 |
| 2 |  | 1 | 1 |
| 3 |  | 0 | 1 |
| 4 | 0 | 1 | 0 |
| 4 | 1 | 0 | 0 |
| 5 | 0 | 0 | 1 |
| 5 | 1 | 0 | 0 |

      - For x, y = 0 ..patternSize − 1 the following applies:

        - wedgePattern[ x ][ y ] = curPattern[ ( x  <<  1 ) + xOff ][ ( y  <<  1 ) + yOff ]	(Error: Reference source not found-5)

    - Otherwise ( resShift is not equal to 1 ), wedgePattern is set equal to curPattern.

- H.8.4.4.2.12.3	Wedgelet pattern extension process

Inputs to this process are:

- a variable log2BlkSize specifying the binary partition pattern size

- a variable subIdx specifying a 4x4 Wedgelet index,

- a variable xSub specifying the horizontal position of a left-top 4x4 block,

- a variable ySub specifying the vertical position of a left-top 4x4 block,

Output of this process is:

- binary array wedgePattern[ x ][ y ] of size ( 1<<log2BlkSize )x( 1<<log2BlkSize )

The variable xS and yS specifying the partition line start position of a 4x4 Wedgelet pattern are derived as follows:

  - xS = WedgeStartPos[ 2 ][ subIdx ][ 0 ];

  - yS = WedgeStartPos[ 2 ][ subIdx ][ 1 ];

The variable xE and yE specifying the partition line end position of a 4x4 Wedgelet pattern are derived as follows:

  - xE = WedgeEndPos[ 2 ][ subIdx ][ 0 ];

  - yE = WedgeEndPos[ 2 ][ subIdx ][ 1 ];

If  log2BlkSize is equal to 2, the following applies

- WedgePatternTable[ 2 ][ subIdx ] is assigned to wedgePattern;

Otherwise, the following applies:

- If xS != xE or yS !=yE, the followings applies:

  - a = ( yE – yS );

  - b = ( yE + ( ySub << 3 ) ) * ( xE – xS ) – a * ( xE + ( xSub << 3 ) );

  - For x, y = 0.. blkSize – 1

  - wedgePattern[ x ][ y ] = = ( ( y * ( xE – xS ) – a * x ) << 1 ) < b ? 1 : 0

- Otherwise, the following applies:

  - For x, y = 0..blkSize – 1

    - If  xSub = = 0  &&  ySub = = 0 , the followings applies:

  - wedgePattern[ x ][ y ] = (!x) && (!y) ? 1 : 0

    - Otherwise, if  ySub = = 0 , the followings applies:

  - wedgePattern[ x ][ y ] = x < ( xSub << 2 ) ? 1 : 0

    - Otherwise, the followings applies:

wedgePattern[ x ][ y ] = y < ( ySub << 2 ) ? 1 : 0

- H.8.4.4.3	Depth value reconstruction process

Inputs to this process are:

- a luma location ( xTb, yTb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable nTbS specifying the transform block size,

- predicted samples predSamples[ x ][ y ], with x, y =0..nTbS − 1

- the intra prediction mode predModeIntra,

Output of this process is:

- reconstructed depth value samples resSamples[ x ][ y ], with x, y = 0.. nTbS − 1.

Depending on predModeIntra the array wedgePattern[ x ][ y ] with x, y =0..nTbS − 1 specifying the binary segmentation pattern is derived as follows.

- If predModeIntra is equal to INTRA_DMM_WFULL, the Wedgelet pattern extension process as specified in subclause H.8.4.4.2.12.3 is invoked with Log2( nTbS ), wedge_full_tab_idx[ xTb ][ yTb ], wedge_sub_col[ xTb ][ yTb ] and wedge_sub_row[ xTb ][ yTb ] as inputs, and the output is assigned to wedgePattern. the following applies.

    - wedgePattern = WedgePatternTable[ Log2( nTbS ) ][ wedge_full_tab_idx[ xTb ][ yTb ] ]

- Otherwise ( predModeIntra is not equal to INTRA_DMM_WFULL ), the following applies.

  - For x, y = 0..nTbS − 1 wedgePattern[ x ][ y ] is set equal to 0.

Depending on dlt_flag[ nuh_layer_id ] the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

…