| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0134 |
| --- | --- |

| Title: | CE5 related: Wedgelet pattern extension from 4x4 block |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-658-5536 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the Depth Modeling Modes (DMM) of current 3D-HEVC, different sets of wedgelet patterns need to be generated for each square prediction unit (PU) size. In this contribution, a wedgelet pattern extension mechanism is proposed so that only the pattern list for the smallest size (4x4) needs to be maintained while any wedgelet pattern for a larger PU size can be extended from a 4x4 block within the PU. Experimental results report that the proposed method has almost no impact on the coding performance under common test condition.

# Introduction

In current 3D-HEVC [1], DMM is applied for efficient depth map intra coding. With DMM, a depth block is partitioned by one or two segments, and each segment is represented by a single DC value. Two types of partition patterns are enabled in DMM, including wedgelet and contour patterns. The wedgelet pattern has a partition boundary that is close to a straight line, while the Contour pattern can be of arbitrary shape. Moreover, in SDC_DMM1 mode, the same wedgelet patterns applied for DMM 1 are utilized, but the residual coding scheme is simplified with only one residual value signalled for each partition.

For each block size ranging from 4x4 to 32x32, a wedgelet pattern list, including all available wedgelet pattern, is generated during encoder and decoder initialization. To generate an NxN wedgelet partition pattern which is specified by an NxN binary block, a pair of start point (xS, yS) and end point (xE, yE) position is given, and a temporary KxK partition pattern is first generated with all samples initialized as 0, where K equals 2*N for half-sample accuracy, and K equals N for other cases. Then the samples which form the partition boundary line connecting (xS, yS) and (xE, yE) are set as 1, and the temporary partition pattern is divided as two parts, as illustrated in below Figure 1.

The accuracy of start and end point position depends on the block size, as tabulated below,

Table 1: Settings of Start/End point accuracy depending on wedgelet pattern size

| Wedgelet pattern size | Start/End point accuracy |
| --- | --- |
| 4x4 | half-pel |
| 8x8 | half-pel |
| 16x16 | full-pel |
| 32x32 | double-pel |

During the wedgelet pattern list generation, all possible combinations of start and end point positions are exploited. Each generated wedgelet pattern is compared to all the existing wedgelet patterns in current wedgelet list, duplicated wedgelet patterns are excluded and only unique wedgelet pattern is appended to the end of the current wedgelet pattern list. The numbers of total available wedgelet patterns included in the wedgelet pattern list for different block sizes are listed below,

Table 2: Numbers of available wedgelet patterns for different block sizes

| Block size | Num of wedgelet patterns |
| --- | --- |
| 4x4 | 86 |
| 8x8 | 782 |
| 16x16 | 1394 |
| 32x32 | 1503 |

To signal an NxN wedgelet pattern, in DMM mode 1 (DMM1) or simplified depth coding DMM mode 1 (SDC_DMM1), an NxN wedgelet index is explicitly transmitted, and an NxN wedgelet subset index is explicitly signaled in DMM mode 3.

# Problems

The current design of wedgelet pattern generation in 3D-HEVC has the following problems:

- For large block sizes, considerable number of wedgelet patterns is available in the wedgelet pattern list, i.e., totally 1394 and 1503 wedgelet patterns for 16x16 and 32x32 block sizes, respectively. The large number of wedgelet patterns adds the cost for storage, which is not desirable for either encoder or decoder implementations.

- According to the current wedgelet pattern generation process, the computation cost for generating a wedgelet pattern is somewhat large, especially when the block is of a larger size, e.g., 32x32.

- During the wedgelet pattern list generation process, to avoid adding duplicate wedgelet patterns, each generated wedgelet pattern is compared to all the existing patterns (up to 1052) in the wedgelet list, and NxN bit-wise comparisons are needed for each comparison of two wedgelet patterns. The wedgelet comparisons involve relatively large computational cost, especially for large block sizes.

# Proposal

The basic concept of the proposed method is to generate an NxN wedgelet pattern by extending a given 4x4 wedgelet pattern located at a given position. More specifically, given a 4x4 wedgelet pattern subIdx and a position ( i,  j ), the partition boundary line of the given 4x4 wedgelet pattern located at position ( i<<2, j<<2 ) is extended to the whole NxN region, and an NxN wedgelet pattern is derived accordingly. An example of deriving a 16x16 wedgelet pattern by extending a 4x4 wedgelet pattern located at (2, 2) is illustrated in below Figure 2.

(2, 2)

Figure 2: Example of deriving a16x16 wedgelet pattern by extending a 4x4 wedgelet pattern located at (2, 2)

## Indication of wedgelet patterns

When DMM 1 is applied to one PU, the original 4x4 wedgelet pattern index is directly signalled when the PU size is equal to 4x4. For all other PU sizes, the 4x4 wedgelet pattern index is signalled together with a coordinate of a 4x4 unit.

In DMM 3, a 4x4 wedgelet subset index together with a coordinate of a 4x4 unit is signaled to identify a wedgelet pattern in DMM 3. The subset index of the 4x4 wedgelet subset is selected based on the co-located texture intra mode, similar as in the current DMM 3 mode.

## Proposed wedgelet generation process

For wedgelet patterns of size 4x4, the original 4x4 wedgelet pattern indexed by subIdx is directly used. For wedgelet patterns larger than 4x4, given a 4x4 wedgelet index subIdx and its position (i, j), the start and end position of the 4x4 wedgelet index is first specified as (xS, yS) and (xE, yE). Then a line function y= a*x+b, which connects the point position ( xS + (i<<2), yS + (j<<2) ) and point position ( xE + (i<<2), yE + (j<<2)), is derived. With the derived line function, the partition of each sample (x, y) is derived as follows

y – a * x < b ? 1 : 0,

where a and b represents the slope and intercept of the partition boundary line, respectively, and a, b are calculated as follows,

a = ( yS - yE ) / ( xS - xE ); b = ( xS * yE – yS * xE ) / ( xS - xE ).

Note that, with proposed method, the wedgelet pattern is generated during decoding, and the original large wedgelet lists used to store 8x8 ~32x32 wedgelet patterns are not needed any more.

## Simplified wedgelet pattern generation for 4x4

In current 3D-HEVC, each 4x4 wedgelet pattern bPattern is generated as a downsampled 8x8 wedgelet pattern bTempPattern, which is generated by a given pair of start/end point positions, as follows,

bPattern[m][n] = bTempPattern[2*m+X][2*n+Y]

where m, n = 0, …, 3, X and Y are shifting values (0 or 1) dependent on wedgelet patterns specified in 3D-HEVC.

In this contribution, it is proposed to do downsampling for all wedgelet patterns as follows,

bPattern[m][n] = bTempPattern[2*m][2*n]

This aspect has also been proposed in a separate proposal JCT3V-F0133.

# Complexity analysis

The decoder complexity changes by using proposed methods have impact on the following two aspects:

- Memory cost of the storage of wedgelet patterns

Assume one bit is used to store the partition value of one sample in the wedgelet pattern, given the number of total available wedgelet patterns shown in Table 2, the total amount of storage used for the storage of wedgelet patterns in current 3D-HEVC is 243.42 Kbytes (= (4*4*86 + 8*8*782 + 16*16*1394 + 32*32*1503)/8 bytes).

With the proposed method, since only storage of 4x4 wedgelet patterns is needed, the total amount of storage is reduced to 0.172 Kbytes, which is 99.93% memory reduction.

- Computational cost for excluding duplicate wedgelet pattern

In the current wedgelet initialization process, for each generated NxN (N ranges from 4 to 32) wedgelet pattern, it will be compared to all the existing wedgelet patterns in the wedgelet list, and each time maximum NxN bits need to be compared. The computation load is relatively large, especially for large block sizes. However, this part of computational cost is only needed for 4x4 wedgelet list generation with the proposed method.

# Experimental results

The proposed method is integrated into current 3D-HTM 8.0 software [2] and the simulation tests are performed under the common test conditions (CTC) [3] and Intra-only test condition as specified in Core Experiment 5 document [4].

The coding performance of the proposed scheme under CTC and Intra-only test conditions are tabulated in Table 3 and Table 4, respectively. As reported in Table 3 and Table 4, average BD bitrate on the synthesized views is 0.0% and 0.1% for CTC and Intra-only test condition, respectively.

Table 3: Coding performance of proposed scheme under CTC

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 107% | 107% | 104% |
| Kendo | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 97% | 95% | 97% |
| Newspapercc | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 99% | 93% | 96% |
| GhostTownFly | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 94% | 91% | 92% |
| PoznanHall2 | 0.0% | -0.2% | 0.0% | -0.1% | -0.2% | 102% | 99% | 98% |
| PoznanStreet | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 86% | 86% | 85% |
| UndoDancer | 0.3% | 0.0% | 0.0% | 0.1% | 0.2% | 118% | 110% | 109% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 101% | 99% | 99% |
| 1920x1088 | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 100% | 97% | 96% |
| average | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100% | 97% | 97% |

Table 4: Coding performance of proposed scheme under Intra-only test condition

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 97% | 96% | 95% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 102% | 100% | 98% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 107% | 103% | 101% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 103% | 91% | 94% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 101% | 110% | 106% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 121% | 108% | 108% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 101% | 90% | 88% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 102% | 100% | 98% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 106% | 100% | 99% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 105% | 100% | 99% |

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1001, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

- 3DV-HTM version 8.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1100, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

- F. Jäger, “Description of Core Experiment 5 (CE5) on Depth Intra Modes,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1105, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).