| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0135 |
| --- | --- |

| Title: | CE3 related: HEVC compatible de-blocking for sub-PUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen Geert Van Der Auwera Li Zhang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-658-5134 geertv@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In JCT3V-E0184, sub-PU (prediction unit) based inter-view motion prediction has been proposed. This method provides significant coding efficiency benefit. However, sub-PU based motion may lead to blocking artifacts because sub-blocks within a PU are allowed to be predicted using different motion vectors while the boundary of such sub-blocks are typically not considered edges for de-blocking. This contribution proposes to allow de-blocking sub-PU boundary by introducing transform unit boundaries before the de-blocking process. The basic idea is to make sub-PU boundary be transform unit boundary. From the side-by-side comparison, it reportedly shows that blocking artifact caused by sub-PU prediction is suppressed. It is also reported that -0.26% and -0.18% (and -0.20% and -0.07% coding gain for Shark sequence) average coding gain are achieved on texture and synthesized view respectively for the first 7 sequences.

# Introduction

In 5th JCT-3V meeting, sub-PU [1] based inter-view motion prediction is proposed, in which sub-PUs within a PU are allowed to be predicted using different motion vectors. Sub-PU based inter-view motion prediction can improve coding efficiency of non-base views significantly by utilizing more motion information of base views. However, it does not de-block sub-PU boundary, therefore may lead to blocking artifacts at sub-PU boundary.

Similarly, backward view synthesis prediction (BVSP) in 3D-HEVC [2] employs sub-PU inter-view prediction and it allows sub-blocks within a PU to be predicted using different disparity vectors. However, though different sub-blocks may use different disparity vectors, one same disparity vector is assigned to all sub-blocks within the PU after motion compensation and no de-blocking is performed at sub-PU boundary.

An example is illustrated in Fig.1 to show the problem. As can be seen, when sub-PU prediction is used and sub-PU boundary is not de-blocked, the blocking artifact gets much worse.

Fig.1 Blocking artifact caused by sub-PU prediction (58th picture in first non-base view of Balloons sequence, qp = 35). Left: HTM-8.0, Right: HTM-8.0 + sub-PU prediction (but not de-block sub-PU boundary).

# Proposed method

To address the above issue while not requiring changing of the de-blocking process as in HEVC version 1, it is proposed to convert each sub-PU to be a transform unit right before de-blocking process thus boundary of sub-PUs within a PU are considered as de-blocking edge.

Note in HEVC version 1, the edges are identified when either CU/PU boundaries or TU boundaries exist.

For BVSP, only 8x8 sub-block is converted to TU though disparity vector is derived at smaller block size, e.g. 8x4 or 4x8 block, in BVSP.

For each sub-PU, when its TU size is larger than sub-PU size, it will be converted to a TU. cbf (of luma, cb and cr) of the converted TU is set to be equal to cbf (of luma, cb and cr) of the TU (in the original TU tree) that covers the converted TU.

Fig.2 illustrates an example of the conversion of sub-PU to transform unit. In Fig.2, CU size is 32x32 and it is partitioned with partition size Nx2N, and PU on the left side does not use sub-PU prediction while PU on the right side uses sub-PU prediction. The sub-PU size is 8x8. As can be seen, TU structure of the left PU is not changed because there is no sub-PU there; while TU structure of the right PU is modified. TU on the top half of the right PU is further split to four 8x8 TUs. TU tree on the bottom half of the right PU is not changed because TU size in it is already equal to or smaller than 8x8.

Fig.2. Conversion of sub-PU to transform unit. Left: TU tree structure of the CU before converting sub-PU to TU, Right: TU tree structure of the CU after converting sub-PU to TU.

Proposed method is only applied to CU with partition size 2Nx2N, 2NxN, Nx2N and NxN, and asymmetric partition is not processed.

# Simulation results

Proposed method is integrated into HTM-8.0 + subPU [3] and compared with it under common test condition [4]. Sub-PU size is set to be equal to 8x8 in the simulation.

Side-by-side subjective comparison is shown for several clips of Balloons and Kendo in Fig.3 and Fig.4.

Fig.3 Subjective comparison of Balloons on first non-base view with qp = 35 (58th picture). Left: proposed method, Right: not de-block sub-PU boundary.

Fig.4 Subjective comparison of Kendo on first non-base view with qp = 35 (11th picture). Left: proposed method, Right: not de-block sub-PU boundary.

Objective comparison is shown in Table 1. As can be seen, proposed method can achieve -0.26% and -0.18% average coding gain on texture and synthesized view respectively for the first 7 sequences, and can achieve -0.20% and -0.07% coding gain on texture and synthesized view respectively for Shark sequence.

Table 1: Comparison of proposed method with HTM-8.0 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.89% | -0.62% | -0.38% | -0.37% | -0.30% | 103.7% | 120.6% | 102.4% |
| Kendo | 0.00% | -0.48% | -0.83% | -0.37% | -0.39% | -0.20% | 125.9% | 133.4% | 119.5% |
| Newspaper_CC | 0.00% | -0.45% | -0.32% | -0.19% | -0.16% | -0.07% | 103.7% | 122.6% | 104.8% |
| GT_Fly | 0.00% | -0.12% | -0.21% | -0.13% | -0.16% | -0.13% | 106.8% | 113.2% | 105.4% |
| Poznan_Hall2 | 0.00% | -0.42% | -0.77% | -0.30% | -0.30% | -0.29% | 106.0% | 120.7% | 102.5% |
| Poznan_Street | 0.00% | -0.50% | -0.36% | -0.23% | -0.23% | -0.15% | 121.2% | 122.7% | 118.2% |
| Undo_Dancer | 0.00% | -0.60% | -0.38% | -0.24% | -0.23% | -0.11% | 112.7% | 121.0% | 104.0% |
| 1024x768 | 0.00% | -0.61% | -0.59% | -0.31% | -0.31% | -0.19% | 111.1% | 125.5% | 108.9% |
| 1920x1088 | 0.00% | -0.41% | -0.43% | -0.22% | -0.23% | -0.17% | 111.7% | 119.4% | 107.5% |
| average | 0.00% | -0.49% | -0.50% | -0.26% | -0.26% | -0.18% | 111.4% | 122.0% | 108.1% |
| Shark | 0.00% | -0.25% | -0.27% | -0.20% | -0.20% | -0.07% | 107.8% | 119.2% | 106.5% |

# References

- Jicheng An, Kai Zhang, Jian-Liang Lin, Shawmin Lei, “3D-CE3.h related: Sub-PU level inter-view motion prediction”, JCT3V-E0184, Vienna, Austria, 29 July – 02 Aug. 2012.

- Gerhard Tech, Krzysztof Wegner, Ying Chen, Sehoon Yea, “3D-HEVC Draft Text 1”, JCT3V-E1001, Vienna, Austria, 29 July – 02 Aug. 2012.

- J. An, K. Zhang, J.-L. Lin, S. Lei, “3D-CE3: Sub-PU level inter-view motion prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0110, 6th Meeting: Geneva, CH, Oct. 25– Nov. 1, 2013.

- Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-E1100, Vienna, Austria, 29 July – 02 Aug. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).