| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0136 |
| --- | --- |

| Title: | Comments on camera parameters in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Vijayaraghavan Thirumalai 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal provides discussions on the signaling of camera parameters especially for depth views in 3D-HEVC.

# Introduction

## Status of the current 3D-HEVC

In the current 3D-HEVC, camera parameters for both texture view and depth view of the same view could be signalled. However, it is unclear whether texture layer and depth layer of the same view share the same set of camera parameters or not and if they are not the same, whether duplications are allowed.

Relevant notes in the 3D-HEVC WD are copied as follows:

[Ed. (CY): it is unclear whether texture layer and depth layer of the same view share the same set of camera parameters or not and if they are not the same, whether duplications are allowed. However, the current design enables the full functionality and optimizations can be considered later.]

[Ed. (GT): In current design camera parameters of depth layers overwrite camera parameters of texture layers, when they have the same ViewIdx. This makes no sense. Moreover camera parameters are not used for depth layers. Parameter presence should be signalled for each ViewIdx. ]

## Assumptions in 3D-HEVC

3D-HEVC assumes that the cameras are arranged in one single horizontal line and are with possible horizontal shift. A lot of coding tools are with such assumption and it is assumed that the texture and depth are fully calibrated to perform the depth oriented inter-view prediction.

Although there might be other cases such as unpaired texture and depth views, such a scenario can be well supported by simply extending MV-HEVC with more views and with depth being coded with HEVC. Therefore, they are not considered at this stage as major use cases for 3D-HEVC, especially due to the fact that the coding benefit of 3D-HEVC compared with MV-HEVC for texture only coding is not huge.

## Typical use cases that require camera parameters from depth

In the current 3D-HEVC, camera parameters for depth views are not used, since there is no need to convert a depth value into a disparity vector for the objects in the depth image.

In future, some scenarios may require such camera parameters. In such circumstances, it should be sufficient to use the same camera parameters of the associated texture view, as discussed below in section 1.4.

## Reuse of the camera parameters for depth

Assume that the resolution of the texture image is N1xN2 and the corresponding depth image D has the resolution M1xM2, where M1<=N1 and M2 <=N2.

Let C= N1/M1 = N2/M2.

Denote dt and dd, respectively the calculated disparity for the texture and depth images.

Case 1: Same resolution

In this case the depth and texture images are captured at same resolution.

The disparity values for the texture image at each position x from 0 to N1-1, inclusive and y from 0 to N2-1, inclusive can be calculated as:

dt(x,y) = D(x,y)*s + o.

Similarly, the disparity values for the depth image at each position x from 0 to M1-1, inclusive and y from 0 to M2-1, inclusive can be calculated as

dd(x,y) = D(x,y)*s + o.

As the resolution of the images are the same, it is straightforward to conclude that dd(x,y) = dt(x,y). In other words, the disparity values calculated at each sample position (x,y) in the texture image can be used as the disparity value for the depth image at the same respective sample position.

Case 2: Different resolutions

In this case the resolution of the depth image is half the resolution of the texture image.

First assume that there is a virtual depth image D with the same size as texture.

The disparity values for the texture image at each position x from 0 to N1-1, inclusive and y from 0 to N2-1, inclusive can be calculated as

dt(x,y) = D(x, y)*s + o,

The disparity for the virtual depth image is calculated as

d'd(x,y) = D(x, y)*s + o.

So the pixel corresponding to (x, y) in a different view has a coordination of (x+ d'd(x,y), y).

Assume now that the low-resolution depth image D’ is a down-sampled version of the virtual depth image D. The pixel (x, y) in image D has a new coordination of (x/C, y/C) in image D’ and the corresponding pixel has a coordination of ((x+ d'd(x,y))/C, y/C).

In this case disparity value becomes as

dd(x/C,y/C) = d'd(x,y)/C,

Therefore, for each sample position in the depth image, the disparity can be calculated as follows:

dd(x/C,y/C) = D(x, y)*s/C + o/C.

In other words, the camera parameters to be used for converting a depth value to a disparity can be derived by scaling the camera parameters that are used for the associated texture view, if the resolutions of the texture view and depth views are different.

# Conclusion

There is no need to separately signal the camera parameters for depth views and texture views in 3D-HEVC. When needed, the depth views can use camera parameters of the texture views.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.